# AVIH0008 Operate remote pilot aircraft systems in extended visual line of sight (EVLOS)

Release: 1

AVIH0008 Operate remote pilot aircraft systems in extended visual line of sight (EVLOS)

# Modification History

Release 1. This is the first release of this unit of competency in the AVI Aviation Training Package.

# Application

This unit involves the skills and knowledge required to operate remote pilot aircraft systems (RPAS) in extended visual line of sight (EVLOS) in compliance with relevant regulatory requirements of the Civil Aviation Safety Authority (CASA) and national operating standards. It includes operating and managing RPAS during normal flight and managing RPAS during abnormal and emergency procedures. This unit addresses aviation technical skill requirements (physical, mental and task-management abilities) related to equipment and system operations of flight or ground operations personnel and contributes to safe and effective performance in complex aviation operational environments. Operations are conducted as part of commercial and military aircraft activities across a variety of operational contexts within the Australian aviation industry. Work is performed independently or under limited supervision within a single-pilot or multi-crew RPAS environment. Licensing, legislative, regulatory or certification requirements are applicable to this unit.

# Pre-requisite Unit

Not applicable.

# Competency Field

W – Equipment and Systems Operations

# Unit Sector

Not applicable.

# Elements and Performance Criteria

| ELEMENTS |  | PERFORMANCE CRITERIA |  |
| --- | --- | --- | --- |
| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| 1 | Operate RPAS during normal flight | 1.1 | Command controls systems are checked and tested for the range of operation |
|  |  | 1.2 | Crew resources are managed prior to operations including observer briefings |
|  |  | 1.3 | RPAS, sub-systems and devices are operated to achieve operational requirements |
|  |  | 1.4 | RPAS, sub-systems and devices are monitored using a systematic scan technique |
|  |  | 1.5 | RPAS and flight environment information is analysed to identify actual and potential threats or errors |
|  |  | 1.6 | RPAS automation is utilised to manage control station workload |
|  |  | 1.7 | Checklist procedures are implemented during normal RPAS operations |
| 2 | Operate RPAS during abnormal and emergency procedures | 2.1 | Abnormal or emergency situations are identified, diagnosed and communicated |
|  |  | 2.2 | RPAS control is maintained at all times |
|  |  | 2.3 | RPAS abnormal or emergency checklist procedures are implemented to maintain safety of flight |
|  |  | 2.4 | Course of action is decided, implemented, evaluated and revised to achieve safest outcomes |
|  |  | 2.5 | Location and operation of emergency systems applicable to RPAS are explained |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

Range is restricted to essential operating conditions and any other variables essential to the work environment.

# Unit Mapping Information

No equivalent unit.

# Links

AVI Training Package Companion Volume Implementation Guide available on VET Net: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=4725260a-0af3-4daf-912b-ef1c2f3e5816

Assessment Requirements for AVIH0008 Operate remote pilot aircraft systems in extended visual line of sight (EVLOS)

# Modification History

Release 1. This is the first release of this unit of competency in the AVI Aviation Training Package.

# Performance Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy all of the requirements of the elements, performance criteria and range of conditions on at least one occasion and include: adapting to differences in equipment and operating environment in accordance with standard operating procedures (SOPs) applying precautions and required action to minimise, control or eliminate identified hazards applying relevant legislation and workplace procedures communicating effectively with others completing relevant documentation identifying and correctly using relevant equipment implementing contingency plans implementing work health and safety (WHS) procedures and relevant regulations interpreting and following operational instructions and prioritising work interpreting remote pilot aircraft systems (RPAS) displays modifying activities depending on workplace contingencies, situations and environments monitoring and anticipating operational problems and hazards and taking appropriate action monitoring work activities in terms of planned schedule operating electronic communications equipment to required protocol performing systematic scan technique for monitoring RPAS, sub-systems (equipment) and devices reading, interpreting and following relevant regulations, instructions, procedures, information and signs reporting and/or rectifying problems, faults or malfunctions promptly in accordance with workplace procedures selecting and using required personal protective equipment (PPE) conforming to industry and WHS standards undertaking fault finding in RPAS using automated systems to manage workload working collaboratively with others working systematically with required attention to detail without injury to self or others, or damage to goods or equipment.

# Knowledge Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy all of the requirements of the elements, performance criteria and range of conditions and include knowledge of: aeronautical decision-making processes relevant to RPAS operations effective communication radio protocols between flight crew observer briefings command and control links including: link quality radio range effectiveness link security setting up antennas and/or antenna trackers error management, including error types, causes and consequences fatigue risk management human factors relevant to RPAS operations in extended visual line of sight (EVLOS) human performance and its limitations, including the senses, memory and situational awareness normal, minimum and maximum fuel pressures or battery/power levels and power draw risk identification, analysis and control individual remote pilot aircraft sub-systems as applicable to RPAS type and relevant rating/endorsement requirements, including: battery/fuel/power system: use of a schematic diagram of battery/fuel/power system to explain layout and normal operating procedures likely faults that may affect battery/fuel/power system emergency operating procedures for battery/fuel/power system operation of /battery/fuel/power selector panel or display use of cross-feed or power distribution fuel-dumping procedures full battery/fuel capacity and fuel grade hydraulic system, including: use of a schematic diagram of hydraulic system to explain layout and normal operating procedures likely faults that may affect hydraulic system emergency operating procedures for hydraulic system units or services operated by hydraulics type of hydraulic fluid, operating pressure and capacity of reservoir flight environment information must include one or more of the following: head up display (HUD) suitable for flight RPAS control systems suitable for flight indications, including height, speed, direction and location electrical system, including: use of a schematic diagram of electrical system to explain type/s of electrical system likely faults that may affect electrical system emergency operating procedures for electrical system voltage and amperage of battery or power cell number and output of generators methods of circuit protection location of fuses and circuit breakers precautions to be taken when operating electrical devices instruments and displays operated by electrics oil/lubrication system, including: use of a schematic diagram of oil system to explain functions of oil system likely faults that may affect oil system emergency operating procedures for oil system number of tanks, capacity and oil grade normal, minimum and maximum oil pressure and temperature operation of oil cooling system autopilot, including: principles of operation of autopilot system likely faults that may affect autopilot system emergency operating procedures for autopilot system identification of power sources, voltage or pressure procedure to determine gyros are operating normally procedure to engage autopilot normal and emergency procedure to disengage autopilot limits of gyro units detection and avoidance systems, including: surveillance and collision avoidance functions of detection and avoidance systems system limitations, selectivity and inhibits basic components of detection and avoidance systems identification and demonstration of controls or explanation of function of RPAS control station detection and avoidance systems visual displays and symbology functions of audio alerts and annunciations appropriate crew response to multiple detection and avoidance systems events recall of radiotelephone procedures following a detection and avoidance system alert requirements for a written report of a detection and avoidance systems alert and to whom it must be submitted automated system, including: limitations of automated systems operating procedures for systems, such as flight management system, auto throttle/engine/thrust control, flight director system, automated aircraft navigation systems, automated engine condition and monitoring system workload management procedures for utilising automated systems warning systems/indicators to identify automated systems failure RPAS checklists, including: explanation of the normal system operating procedures of RPAS, sub-systems and devices used to operate specific RPAS, including use of published scans and checklists, immediate action items, warnings and limitations stress, workload and time pressure management.

# Assessment Conditions

Assessors must hold credentials specified within the Standards for Registered Training Organisations current at the time of assessment. Assessment must satisfy the Principles of Assessment and Rules of Evidence and all regulatory requirements included within the Standards for Registered Training Organisations current at the time of assessment. Assessment must occur in workplace operational situations. Where this is not appropriate, assessment must occur in simulated workplace operational situations that reflect workplace conditions. Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate. Resources for assessment must include access to: a range of relevant exercises, case studies and/or simulations acceptable means of simulation assessment applicable documentation, including workplace procedures, regulations, codes of practice and operation manuals relevant materials, tools, equipment and PPE currently used in industry.

# Links

AVI Training Package Companion Volume Implementation Guide available on VET Net: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=4725260a-0af3-4daf-912b-ef1c2f3e5816