# Assessment Requirements for AVIH0018 Implement flight planning procedures

Release: 1

Assessment Requirements for AVIH0018 Implement flight planning procedures

# Modification History

Release 1. This is the first release of this unit of competency in the AVI Aviation Training Package.

# Performance Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy all of the requirements of the elements and performance criteria on at least one occasion and include: adapting to differences in equipment and operating environments in accordance with standard operating procedures (SOPs) applying knowledge of: climb and descent tables and charts constant mach flight planning tables and charts long range cruise charts and tables applying knowledge of standard extended twin-engine operations (ETOPS) considerations applying published visual flight rules (VFR) and instrument flight rules (IFR) procedures applying precautions and required actions to minimise, control or eliminate identified hazards applying relevant legislation and workplace procedures calculating altimeter corrections from authorised sources, true altitude above mean sea level and height above terrain for a range of pressure altitudes and temperatures, and using standard atmosphere tables calculating flight time and fuel with varying fuel quantities to determine cost of carriage calculating minimum fuel and flight time, including taxi fuel, optimum altitude, use of charts, reserve fuel and critical point (CP) fuel calculating minimum fuel to destination including distant alternates and using destination as the alternate communicating effectively with others completing relevant documentation decoding aviation weather reports, including observing, trending and analysing simultaneous reports and conditions determining aircraft limits and maximum payloads determining conversion angles, convergency applications, plotting relative true north and grid north using Mercator, lambert conformal and polar stereographic charts determining operational requirements estimating winds and temperatures at flight levels corresponding to upper-air charts and at intermediate flight levels examining surface and upper-air charts in mid-latitude and tropical regions examining synoptic and aeronautical prognostic charts to determine movement and time evolution examining typical charts and forecasts for flight planning to decode aerodrome and trend type landing forecasts identifying and briefing aeronautical climatology of a specific route identifying and correctly using relevant equipment identifying and selecting suitable navigation aids/systems implementing contingency plans implementing work health and safety (WHS)/occupational health and safety (OHS) procedures and relevant regulations inspecting and identifying actual and forecast charts for pressure patterns on surface and upper-air charts, and identifying pressure gradients interpreting and following operational instructions and prioritising work interpreting flight charts interpreting meteorological forecasts interpreting tropopause and maximum winds charts interpreting weather observations plotted on standard format synoptic charts making flight notifications making proficient operational decisions modifying activities depending on workplace contingencies, situations and environments monitoring and anticipating operational problems and hazards and taking appropriate actions monitoring work activities in terms of planned schedule obtaining and using current operational documents operating electronic communications equipment to required protocol performing calculations for a given true track, true airspeed (TAS) and distance to CP in zero wind, tailwind and beam wind performing calculations for a given true track, TAS and endurance, distance to point of no return (PNR) in zero wind, tailwind and beam wind establishing maximum PNR, return to departure airport and alternate airport, all engines and one engine inoperative reading, interpreting and following relevant regulations, instructions, procedures, information and signs reporting and/or rectifying problems, faults or malfunctions promptly in accordance workplace procedures reviewing aircraft performance and cruise control procedures and variations using aircraft flight manual graphs selecting and using required personal protective equipment (PPE) conforming to industry and WHS/OHS standards working collaboratively with others working systematically with required attention to detail without injury to self or others, or damage to goods or equipment.

# Knowledge Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy all of the requirements of the elements and performance criteria and include knowledge of: aerodrome and en route holding procedures aircraft documents pouch, including: aircraft radio licence certificate of airworthiness certificate of registration emergency en route charts fuel supplier carnet sabotage checklist aircraft fuel planning, including holding, alternate, fixed reserve and usage rates aircraft library, including: aeronautical information publications aircraft journey logbook aircraft operating manual section of operating manual applicable to flight crew airspace requirements and procedures under IFR conditions airspace types and requirements: controlled/uncontrolled airspace designated remote areas (DRA) military airspace restricted/prohibited/danger areas special procedures and fly neighbourly advice airspace CP and PNR customs and immigration forms, including: crew manifest general declaration forms passenger manifest ETOPS: 120 minute operation 180 minute operation 60 minute operation adequate airport airworthiness all engines operating performance altitude capability auxiliary power unit cruise altitude and coverage at 10,000 feet drift down en route alternate airport requirements ETOPS configuration extended range entry point extended range operations fail-safe flight planning fuel fuel and oil supply holding inflight shutdown minimum equipment list missed approach non-standard atmospheric conditions one engine inoperative performance operational practices and procedures precision and non-precision approaches suitable airport weather factors affecting en route performance, range and endurance gross error checking requirements, including: aircraft registration alternate/s flight designator fuel over destination maximum permissible take-off mass minimum fuel name of dispatcher route taxi fuel wind component and temperature ground and space-based navigation systems and infrastructure, including: continuous descent final approach (CDFA) techniques global navigation satellite system (GNSS) operating procedure errors ground-based systems space-based systems how to determine currency of operational documents icing conditions and hazards IFR/VFR cruising levels, selection and hazards IFR/VFR route planning requirements in an Australian Defence Force (ADF) context, relevant Defence Orders and Instructions flight documentation requirements flight procedures, including: 2-D/3-D instrument approach operations air traffic service requirements alternate aerodrome weather minima altimeter accuracy and variations due to temperature chart symbology and information flight instrument operations, errors and limitations flight plan validity lost communications procedures pilot activated lighting (PAL) pilot responsibilities radio communication phraseology reporting requirements runway visual approach slope lighting system operation and limitations search and rescue times (SARTIME) and pilot obligations lowest safe altitude (LSALT), including: calculate route LSALT not specified in aeronautical information publication (AIP) descent below LSALT or minimum safety altitude requirements by day/night/night-visual flight rules (NVFR) establish aircraft above LSALT requirements minimum obstacle clearance provided by minimum circling altitude missed approach minimum obstacle clearance track establishment after take-off meteorological considerations for a flight, including: actual atmospheric heating adiabatic processes aeronautical forecasts aeronautical meteorological reports air masses and fronts aircraft icing analysis of surface and upper-air charts atmosphere, composition and structure atmospheric humidity atmospheric pressures atmospheric stability atmospheric temperature and humidity causes of reduced visibility climatology components of aerodrome operating minima (AOM) (visibility and runway visual range (RVR)) constant pressure charts fog types formation of clouds and precipitation and processes involved formation of types of precipitation frontal depressions and characteristics horizontal forces acting on the air International Standard Atmosphere (ISA) meteorological communications motion in cloud formation and precipitation mountain waves (rotors) observations from satellites other types of pressure systems pressure-wind relationships prognostic charts role of international meteorological services smoke and fire-induced weather station model surface observations synoptic charts in the tropics temperature at the earth’s surface thunderstorms tropical weather types of atmospheric turbulence units of measurement upper-air observations variation to pressure height vertical distribution of temperature (lapse rate) visibility and RVR/slant visual range (SVR) volcanic ash weather reporting services winds in the free atmosphere winds near the earth surface navigation requirements, including: aircraft performance categories and operational implications AOM chart projections (all types) charts used by a typical operator CP (equal time point) decision altitude (DA) general determination of aircraft position GNSS requirements ground-based radar and direction finding gyro heading and grid direction instrument landing system (ILS) International Civil Aviation Organization (ICAO) chart requirements introduction to radio navigation measure of aircraft altitude measurement of distance measurement of true airspeed by airspeed indicator measurement of true airspeed by other means navigation procedures normal segment gradient PNR position and distance position fixing requirements pressure error correction (PEC) relative bearings, including ADF aircraft speed limitations and restrictions time and time zones track and ground speed tracking tolerances true, magnetic and compass directions use of slide rules and scientific calculators very high frequency (VHF) omnidirectional range (VOR) and distance measuring equipment (DME) type radio navigation visual circling by day or night waypoints, symbology and pilot requirements operational planning requirements, including: alternate aerodrome requirements flight planning holding fuel requirements instrument approach ban NVFR operational requirements planned/alternate destination weather conditions below minima receiver autonomous integrity monitoring (RAIM) prediction implications performance-based navigation (PBN), including: APV Baro-VNAV instrument approach charts area navigation (RNAV) required navigation performance (RNP) capability augmented and non-augmented approaches Baro-VNAV vertical guidance principles core components GNSS receiver mode conditions and actions for Required Navigation Performance Authorization Required (RNP APCH) GNSS receiver requirements for RNP APCH operations IAP charts for minima information and operational restrictions navigation system performance requirements PBN principles performance monitoring and alerting RNP approach differentiation RNP instrument approach requirements RNP leg transitions RNP leg types RNP navigation authorisation requirements RNP navigation system errors RNP specifications and system requirements space-based augmentation systems (SBAS) validity and accuracy of altimeter subscale setting to obtain elevation or altitude (QNH) or RNP APCH types vertical guidance information and operational considerations practical uses of CP privileges and limitations conferred by an instrument rating reduced vertical separation minima (RVSM) operations, including: aircraft altimeter accuracy requirements altimetry system failures operational requirements procedures and standard communication phraseology range of flight levels within Australian airspace vertical height tolerances relevant WHS/OHS and environmental procedures and regulations relevant sections of Civil Aviation Safety Regulations (CASRs) and Civil Aviation Orders for IFR and planning requirements requirements for an alternate aerodrome use of a navigational computer.

# Assessment Conditions

Assessors must hold credentials specified within the Standards for Registered Training Organisations current at the time of assessment. Assessment must satisfy the Principles of Assessment and Rules of Evidence and all regulatory requirements included within the Standards for Registered Training Organisations current at the time of assessment. Assessment must occur in workplace operational situations. Where this is not appropriate, assessment must occur in simulated workplace operational situations that reflect workplace conditions. Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate. Resources for assessment must include access to: a range of relevant exercises, case studies and/or simulations acceptable means of simulation assessment applicable documentation, including workplace procedures, regulations, codes of practice and operation manuals relevant materials, tools, equipment and PPE currently used in industry.

# Links

Companion Volume Implementation Guides are found in VETNet’ - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=4725260a-0af3-4daf-912b-ef1c2f3e5816