# Assessment Requirements for AVIY0030 Operate fixed wing remote pilot aircraft systems

Release: 1

Assessment Requirements for AVIY0030 Operate fixed wing remote pilot aircraft systems

# Modification History

Release 1. This is the first release of this unit of competency in the AVI Aviation Training Package.

# Performance Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy all of the requirements of the elements and performance criteria on at least one occasion and include: adapting to differences in equipment and operating environment in accordance with standard operating procedures (SOPs) adhering to restricted, controlled and other appropriately designated airspace requirements applying precautions and required action to minimise, control or eliminate identified hazards applying relevant aircraft aeronautical knowledge applying relevant legislation and workplace procedures applying the techniques of straight and level, climbing and descending flight to achieve a consistent traffic pattern and approach to landing communicating effectively with others when controlling an aircraft in normal flight, including using an aeronautical radio compensating for the secondary effects of controls completing relevant documentation confirming area of operation serviceability and availability controlling an aircraft during slow speed flight controlling an aircraft during turning manoeuvres determining appropriate runway and circuit procedures identifying and correctly using relevant equipment identifying geographical features from charts, including: landing areas within local area geographical limits geographical limits of flight training areas restricted, controlled and uncontrolled airspace areas state/territory local airspace limits implementing contingency plans implementing work health and safety (WHS) procedures and relevant regulations interpreting and following operational instructions and prioritising work maintaining compliance with regulatory requirements maintaining separation between aircraft modifying activities depending on workplace contingencies, situations and environments monitoring and anticipating operational problems and hazards and taking appropriate action monitoring work activities in terms of planned schedule operating electronic communications equipment to required protocol operating safely in the area of operations and landing areas performing circuits and approaches reading, interpreting and following relevant regulations, instructions, procedures, information and signs remaining within a designated area while complying with airspace and air traffic requirements reporting and/or rectifying identified problems promptly in accordance with regulatory requirements and workplace procedures selecting and using required personal protective equipment (PPE) conforming to industry and WHS standards setting local or area barometric pressure adjusted for sea level (QNH) at appropriate stages of flight using instruments to monitor aircraft performance working collaboratively with others when controlling an aircraft in normal flight, including using an aeronautical radio working systematically with required attention to detail without injury to self or others, or damage to goods or equipment.

# Knowledge Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy all of the requirements of the elements and performance criteria and include knowledge of: after launch checks aircraft performance aircraft systems aircraft weight and balance airspace cleared procedure to be carried out before all turns Civil Aviation Safety Regulation (CASR) Part 101 Manual of Standards (MOS) Aeronautical Knowledge relevant to remote pilot aircraft operations cause of and compensation for aileron drag circuit patterns and procedures contents of the operations manual and pilot’s operating handbook (POH) dangers associated with mechanical and wake turbulence dangers of turbulence and wake turbulence when flying at low speed day visual flight rules (VFR) criteria effect of angle of bank and load factor on stall speeds effect of turning and acceleration on magnetic compass accuracy effects and use of carburettor heat or de-icing systems effects of carburettor or intake icing effects of excessive cooling on engine performance effects of flap engine considerations during prolonged climbing and descending environmental conditions that represent visual meteorological conditions (VMC) functions and effects of all aircraft controls go-around procedures from base leg and final approach hazards during maximum rate descent hazards when performing performance manoeuvres local area operating procedures operation of stall warning devices fitted to aircraft pre-landing checks primary effects of controls principles of aerodynamics procedures for setting power in normally aspirated, turbocharged or supercharged engines relationship between angle of bank, load factor and stall speed relationship between induced drag and operating at slow speed relationship of attitude and power to balanced flight relevant sections of aeronautical information package (AIP) relevant sections of CASRs and Civil Aviation Orders relevant WHS and environmental procedures and regulations requirements and procedures for maximum rate descent tendency to under bank in descending turn and over bank in a climbing turn theory and application of best rate and angle of climb turning using a magnetic compass use of autopilot/flight director functions use of flap use of instruments to monitor aircraft performance.

# Assessment Conditions

Assessors must hold credentials specified within the Standards for Registered Training Organisations current at the time of assessment. Assessment must satisfy the Principles of Assessment and Rules of Evidence and all regulatory requirements included within the Standards for Registered Training Organisations current at the time of assessment. Assessment must occur in workplace operational situations. Where this is not appropriate, assessment must occur in simulated workplace operational situations that reflect workplace conditions. Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate. Resources for assessment must include access to: a range of relevant exercises, case studies and/or simulations acceptable means of simulation assessment applicable documentation, including workplace procedures, regulations, codes of practice and operation manuals relevant materials, tools, equipment and PPE currently used in industry.

# Links

AVI Training Package Companion Volume Implementation Guide available on VET Net: - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=4725260a-0af3-4daf-912b-ef1c2f3e5816