# Assessment Requirements for CPCCBL2001 Handle and prepare bricklaying and blocklaying materials

Release: 2

Assessment Requirements for CPCCBL2001 Handle and prepare bricklaying and blocklaying materials

# Modification History

| Release 2 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCBL2001A Handle and prepare bricklaying and blocklaying materials. Updated to meet the Standards for Training Packages 2012. |

# Performance Evidence

To demonstrate competency in this unit, a candidate must meet the performance criteria of this unit by preparing one bricklaying and one blocklaying project on ground level.

In doing this, the candidate must:

use manual techniques to appropriately position a pallet of bricks and blocks to reflect the laying sequence of the project

blend bricks and blocks from different pallets to achieve a balanced masonry finish

produce consistent mortar mixes to a strength factor of M3 using the bucket gauging technique

relocate a pallet of bricks using mechanical equipment

stack bricks and blocks and load mortar boards on a raised platform.

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

terminology relating to bricklaying and blocklaying

relevant building and construction standards and codes

environmental requirements for workplace processes and waste disposal

characteristics, applications and limitations of specified materials for bricklaying and blocklaying:

clay bricks

solid and hollow blocks

pavers

refractory bricks and mortars

various natural and manufactured stone

aggregates, cement, lime, mortar additives

processes and techniques for:

laying bricks and blocks

mixing mortar

manual handling

mechanical handling

functional and operational features of plant, equipment and hand and power tools:

cement mixers

brick cutting saw

brick buggies

elevators

forklifts

materials hoists

pallet trolleys

scaffolds

workplace safety:

job safety and environmental analysis (JSEA)

hazardous manual tasks

exposure to silica

exposure to loud noise

electrical hazards and the use of earth leakage boxes.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

This unit must be assessed in the workplace or a close simulation reflecting workplace conditions and standards, materials, equipment, activities, responsibilities, procedures, safety requirements and environmental considerations.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad