# Assessment Requirements for CPCCBL3001 Lay paving

Release: 2

Assessment Requirements for CPCCBL3001 Lay paving

# Modification History

| Release 2 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCBL3001A Lay paving. Updated to meet the Standards for Training Packages 2012. |

# Performance Evidence

To demonstrate competency in this unit, a candidate must meet the performance criteria of this unit by laying two different paving patterns onto two separate areas.

In doing this, the candidate must:

prepare and lay pavers to a specified design to an area of a minimum of 3.0 m x 2.4 m on a sand bed and fill joints with sand:

set out paved area and excavate for a sand bed

compact the substrate and sand bed using a plate compactor

prepare and lay butter and joint clay paving bricks to an area of a minimum of 1.2 m x 1.4 m on a cement bed

apply the appropriate paving processes for the two paved areas.

# Knowledge Evidence

To be competent in this unit, a person must demonstrate knowledge of:

characteristics, applications and limitations of specified paving products and materials:

bedding materials

clay bricks and pavers

concrete blocks and pavers

mortar and sand

paving materials

slate

stone segments

waterproofing materials

mortar ingredients and consistency

paver sealing products

processes and techniques for:

setting out paver levels and heights

manual and mechanical excavation

compacting subsoils, substrates and bedding

laying paving in various patterns

finishing paver surface evenly and free of water ponding

controlling surface water runoff and subsoil drainage

construction terminology relating to paving

functional and operational features of plant, equipment and hand and power tools:

plate compactors

cement mixers

wet cut saws

battery operated tools

workplace safety:

job safety and environmental analysis (JSEA)

noise reduction

dust suppression

hazardous manual tasks

exposure to silica

exposure to loud noise

standards and codes relating to laying pavers

features of working drawings and specifications

environmental requirements for workplace processes and waste disposal.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

This unit must be assessed in the workplace or a close simulation reflecting workplace conditions and standards, materials, equipment, activities, responsibilities, procedures, safety requirements and environmental considerations.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad