# CPCCBL3001 Lay paving

Release: 2

CPCCBL3001 Lay paving

# Modification History

| Release 2 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCBL3001A Lay paving. Updated to meet the Standards for Training Packages 2012. |

# Application

This unit specifies the skills and knowledge required to pave external areas. It includes laying pavers to varying grades, substrates and patterns.

This unit applies to those who carry out paving on new or existing residential or commercial sites. It is suitable for people operating with some autonomy. A person working at this level would be expected to take responsibility for organising and completing tasks assigned to them without needing close supervision.

Completion of the general construction induction training program specified by the model Code of Practice for Construction Work is required for any person who is to carry out construction work. Achievement of CPCWHS1001 Prepare to work safely in the construction industry meets this requirement.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

CPCCWHS2001 Apply WHS requirements, policies and procedures in the construction industry

# Unit Sector

Bricklaying and Blocklaying

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Prepare for work. | 1.1 | Select, fit correctly and use personal protective equipment (PPE) appropriate for job. |
|  |  | 1.2 | Read and interpret paving requirements from current plans, specifications, standards and codes. |
|  |  | 1.3 | Follow requirements in workplace safety and environmental documentation and workplace procedures. |
|  |  | 1.4 | Calculate material quantities required for the task. |
|  |  | 1.5 | Determine job priorities and sequence job tasks in consultation with others on site. |
|  |  | 1.6 | Identify underground services and potential hazards and determine and implement control measures. |
|  |  | 1.7 | Select and check condition of plant, equipment and tools and report any damage or faults to relevant person. |
|  |  | 1.8 | Check conformity of materials against the specifications and place materials ready for the work. |
| 2 | Set out for paving. | 2.1 | Identify sub-soil, substrate types and compaction requirements. |
|  |  | 2.2 | Identify paver thickness and bed thickness and determine excavation depth. |
|  |  | 2.3 | Set out paved area, levels, grades and finished height of pavers. |
| 3 | Prepare for paving | 3.1 | Excavate area to required depth, treat area for sub-soil drainage and dispose of spoil. |
|  |  | 3.2 | Position and straighten screed rails. |
|  |  | 3.3 | Spread, screed and compact bedding material to specified height. |
|  |  | 3.4 | Grade surface evenly to avoid ponding. |
| 4 | Lay pavers | 4.1 | Determine specified pattern, starting point, and check measurements for parallel and square. |
|  |  | 4.2 | Lay paving to specified pattern, cut segments and align joints. |
|  |  | 4.3 | Compact pavers as specified, grout joints and clean pavers. |
|  |  | 4.4 | Finish perimeters of pavers as specified. |
| 5 | Clean up. | 5.1 | Clear the work area, and dispose of, reuse or recycle materials following workplace and environmental requirements. |
|  |  | 5.2 | Clean tools and equipment, check for serviceability and report damage or faults. |
|  |  | 5.3 | Store and secure tools and equipment following workplace procedures. |

# Foundation Skills

Candidates require:

numeracy skills to:

use measurements and formulas to calculate material quantities

apply measurements to set heights and paver patterns

technology skills to:

use digital tools and devices to source job information and communicate effectively with others.

# Unit Mapping Information

Supersedes and is equivalent to CPCCBL3001A Lay paving

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad

Assessment Requirements for CPCCBL3001 Lay paving

# Modification History

| Release 2 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCBL3001A Lay paving. Updated to meet the Standards for Training Packages 2012. |

# Performance Evidence

To demonstrate competency in this unit, a candidate must meet the performance criteria of this unit by laying two different paving patterns onto two separate areas.

In doing this, the candidate must:

prepare and lay pavers to a specified design to an area of a minimum of 3.0 m x 2.4 m on a sand bed and fill joints with sand:

set out paved area and excavate for a sand bed

compact the substrate and sand bed using a plate compactor

prepare and lay butter and joint clay paving bricks to an area of a minimum of 1.2 m x 1.4 m on a cement bed

apply the appropriate paving processes for the two paved areas.

# Knowledge Evidence

To be competent in this unit, a person must demonstrate knowledge of:

characteristics, applications and limitations of specified paving products and materials:

bedding materials

clay bricks and pavers

concrete blocks and pavers

mortar and sand

paving materials

slate

stone segments

waterproofing materials

mortar ingredients and consistency

paver sealing products

processes and techniques for:

setting out paver levels and heights

manual and mechanical excavation

compacting subsoils, substrates and bedding

laying paving in various patterns

finishing paver surface evenly and free of water ponding

controlling surface water runoff and subsoil drainage

construction terminology relating to paving

functional and operational features of plant, equipment and hand and power tools:

plate compactors

cement mixers

wet cut saws

battery operated tools

workplace safety:

job safety and environmental analysis (JSEA)

noise reduction

dust suppression

hazardous manual tasks

exposure to silica

exposure to loud noise

standards and codes relating to laying pavers

features of working drawings and specifications

environmental requirements for workplace processes and waste disposal.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

This unit must be assessed in the workplace or a close simulation reflecting workplace conditions and standards, materials, equipment, activities, responsibilities, procedures, safety requirements and environmental considerations.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad