# Assessment Requirements for CPCCBL3004 Construct masonry steps and stairs

Release: 2

Assessment Requirements for CPCCBL3004 Construct masonry steps and stairs

# Modification History

| Release 2 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCBL3004A Construct masonry steps and stairs. Updated to meet the Standards for Training Packages 2012. |

# Performance Evidence

To demonstrate competency in this unit, a candidate must meet the performance criteria of this unit by constructing two flights of stairs both with a parallel wing wall to one side.

In doing this, the candidate must construct:

one flight with a minimum of three pre-cast concrete treads

one flight with a minimum of three solid masonry treads

all treads with a 3 mm slope for water run-off

wing wall to match the slope of the stairs with symmetrically cut mitres and uniform joints

both flights to comply with current and relevant Australian Standards, specifications and within the codes for brickwork tolerances.

# Knowledge Evidence

To be competent in this unit, a person must demonstrate knowledge of:

construction terminology relating to the construction of masonry steps and stairs:

total rise

total going

landing

nosing

wing wall

current Australian Standards and codes for staircase construction:

min/max permitted riser size

min/max permitted tread size

slope calculation 2R + 1G rule

min height of a wing wall

max number of steps in one flight

step size tolerance _/+5 mm

max permitted deviation from level (10 mm in 10 m, or max of 15 mm)

max permitted bow of a masonry wall (+/_ 5 mm)

max lipping of materials (Max 2 mm)

max/min permitted perpend joint size (Min size 5 mm, +/- 10 mm from specified size)

max min permitted bed joint size (10 mm +/_ 3 mm)

vertical alignment of perpend joints (15 mm per 2 m height of wall, measured centre to centre)

key features of National Construction Code (NCC) relevant to constructing masonry stairs and steps

characteristics, applications and limitations of approved materials for step and stair construction

brick bond patterns

functional and operational features of plant, equipment and hand and power tools

workplace safety:

job safety and environmental analysis (JSEA)

hazardous manual handling

exposure to silica

exposure to loud noise

features of working drawings and specification

processes and techniques for:

masonry steps and stairs construction

applying geometry to stair calculations and set out

environmental requirements for workplace processes and waste disposal.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

This unit must be assessed in the workplace or a close simulation reflecting workplace conditions and standards, materials, equipment, activities, responsibilities, procedures, safety requirements and environmental considerations.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad