# Assessment Requirements for CPCCBL3006 Lay multi-thickness walls and piers

Release: 2

Assessment Requirements for CPCCBL3006 Lay multi-thickness walls and piers

# Modification History

| Release 2 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCBL3006A Lay multi-thickness walls and piers. Updated to meet the Standards for Training Packages 2012. |

# Performance Evidence

To demonstrate competency in this unit, a candidate must meet the performance criteria of this unit by laying three 230 mm thick brick walls, to a minimum height of 634 mm.

In doing this, the candidate must:

lay one English bond wall to minimum length of 2100 mm with one 900 return external corner to the same height and a minimum length of 700 mm

lay one Flemish bond wall to a minimum length of 1700 mm which incorporates a 590 mm (minimum) squint corner return; wall to be finished with a brick on edge capping finish with symmetrically mitred cuts over squint corner

lay one wall in chosen bond (not stretcher) to a minimum length of 2100 mm with a ‘T’ junction with a minimum length of 600 mm

engaged one pier to each wall at the same height of wall

set out and lay one isolated pier in the same bond for each wall, to the set-out wall height

comply to the brickwork tolerances stated in the current Australian Standards and codes.

# Knowledge Evidence

To be competent in this unit, a person must demonstrate knowledge of:

characteristics, applications and limitations of approved materials used for multi-thickness walls and pier construction

processes and techniques for:

multi-thickness brick and pier construction

installing specified reinforcing

laying bricks to specified bond to provide structural stability

damp-proofing, installing drainage pipes and weep holes

construction terminology relating to multi-thickness wall and pier construction

brick bond patterns:

Flemish

Flemish garden wall

English bond

English garden wall bond

Monk bond

Dutch

1/3 bond

Stretcher

functional and operational features of plant, equipment and hand and power tools

workplace safety:

job safety and environmental  analysis (JSEA)

working at heights

hazardous manual tasks

dust suppression

exposure to silica

exposure to loud noise

features of working drawings and specification

relevant and current building, brick and block laying standards and codes

key features of National Construction Code (NCC) relating to multi-thickness walls and piers

environmental requirements for workplace processes and waste disposal.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

This unit must be assessed in the workplace or a close simulation reflecting workplace conditions and standards, materials, equipment, activities, responsibilities, procedures, safety requirements and environmental considerations.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad