# CPCCBL3007 Install glass blockwork

Release: 2

CPCCBL3007 Install glass blockwork

# Modification History

| Release 2 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCBL3007A Install glass blockwork. Updated to meet the Standards for Training Packages 2012. |

# Application

This unit specifies the skills and knowledge required to install glass blocks into non-structural panels.

This unit applies to people who install fire rated or standard glass blocks using various fixing methods to new or existing structures. It is suitable for people operating with some autonomy. A person working at this level would be expected to take responsibility for organising and completing tasks assigned to them without needing close supervision.

Completion of the general construction induction training program specified by the model Code of Practice for Construction Work is required for any person who is to carry out construction work. Achievement of CPCWHS1001 Prepare to work safely in the construction industry meets this requirement.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

CPCCWHS2001 Apply WHS requirements, policies and procedures in the construction industry

# Unit Sector

Bricklaying and Blocklaying

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Prepare for work. | 1.1 | Select, fit correctly and use personal protective equipment (PPE) appropriate for job. |
|  |  | 1.2 | Read, interpret and apply glass block installation requirements from current plans, specifications, standards and codes. |
|  |  | 1.3 | Follow requirements in workplace safety and environmental documentation and workplace procedures. |
|  |  | 1.4 | Use measurements and apply formulas to calculate material quantities. |
|  |  | 1.5 | Determine job priorities and sequence job tasks in consultation with others on site. |
|  |  | 1.6 | Identify potential hazards and determine and implement control measures. |
|  |  | 1.7 | Select and check condition of plant, equipment and tools and report damage or faults to relevant person. |
|  |  | 1.8 | Check conformity of glass bricks, adhesives and accessories and place materials in preparation for the work. |
| 2 | Set out for installation. | 2.1 | Check opening size, lintel and flashing to suit glass block installation. |
|  |  | 2.2 | Check base and head for level and abutting surfaces for plumb. |
|  |  | 2.3 | Mix mortar to the correct consistency or use approved silicone product following manufacturer’s instructions. |
| 3 | Install glass blocks. | 3.1 | Install expansion strips and reinforcements. |
|  |  | 3.2 | Lay glass blocks level and plumb to the specified design. |
|  |  | 3.3 | Remove excess mortar or silicone at completion and clean glass blocks. |
|  |  | 3.4 | Use appropriate tool to achieve specified joint finish. |
| 4 | Clean up. | 4.1 | Clear the work area, and dispose of, reuse or recycle materials following workplace and environmental requirements. |
|  |  | 4.2 | Clean tools and equipment, check for serviceability and report damage or faults. |
|  |  | 4.3 | Store and secure tools and equipment following workplace procedures. |

# Foundation Skills

Candidates require:

technology skills to:

use digital tools and devices to source job information and communicate effectively with others.

# Unit Mapping Information

Supersedes and is equivalent to CPCCBL3007A Install glass blockwork

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad