# CPCCBL3014 Install fire-rated masonry construction

Release: 2

CPCCBL3014 Install fire-rated masonry construction

# Modification History

| Release 2 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCBL3014A Install fire-rated masonry construction. Updated to meet the Standards for Training Packages 2012. |

# Application

This unit specifies the skills and knowledge required to install fire-rated masonry construction. It includes laying fire resistant masonry to buildings, walls and service openings

This unit applies to bricklayers and block layers who work on new and existing structures. It is suitable for people operating with some autonomy. A person working at this level would be expected to take responsibility for organising and completing tasks assigned to them without needing close supervision.

Completion of the general construction induction training program specified by the model Code of Practice for Construction Work is required for any person who is to carry out construction work. Achievement of CPCWHS1001 Prepare to work safely in the construction industry meets this requirement.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

CPCCWHS2001 Apply WHS requirements, policies and procedures in the construction industry

# Unit Sector

Bricklaying and Blocklaying

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Prepare for work. | 1.1 | Select, fit correctly and use personal protective equipment (PPE) appropriate for job. |
|  |  | 1.2 | Read, interpret and apply fire-rated masonry construction requirements from current plans, specifications, standards and codes. |
|  |  | 1.3 | Follow requirements in workplace safety and environmental documentation and workplace procedures. |
|  |  | 1.4 | Identify fire resistance level (FRL), location and composition of masonry wall. |
|  |  | 1.5 | Use measurements and apply formulas to calculate material quantities. |
|  |  | 1.6 | Determine job priorities and sequence job tasks in consultation with others on site. |
|  |  | 1.7 | Identify potential hazards and determine and implement control measures. |
|  |  | 1.8 | Select and check condition of plant, equipment and tools and report damage or malfunction to relevant person. |
|  |  | 1.9 | Check conformity of materials against the specifications and place materials in preparation for the work. |
| 2 | Construct masonry wall. | 2.1 | Set out and mark position, gauge and bond of masonry wall. |
|  |  | 2.2 | Mix mortar and check consistency with job requirements. |
|  |  | 2.3 | Construct fire-rated masonry wall maintaining bond, gauge and level to the set-out height. |
|  |  | 2.4 | Remove excess mortar, rake or rule joints to specified profile and depth. |
|  |  | 2.5 | Clean and brush down face of brickwork prior to mortar drying. |
| 3 | Clean up. | 3.1 | Clear the work area, and dispose of, reuse or recycle materials following workplace and environmental requirements. |
|  |  | 3.2 | Clean tools and equipment, check for serviceability and report damage or faults. |
|  |  | 3.3 | Store and secure tools and equipment following workplace procedures. |

# Foundation Skills

Candidates require:

technology skills to:

use digital tools and devices to source job information and communicate effectively with others.

# Unit Mapping Information

Supersedes and is equivalent to CPCCBL3014A Install fire-rated masonry construction

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad

Assessment Requirements for CPCCBL3014 Install fire-rated masonry construction

# Modification History

| Release 2 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCBL3014A Install fire-rated masonry construction. Updated to meet the Standards for Training Packages 2012. |

# Performance Evidence

To demonstrate competency in this unit, a candidate must meet the performance criteria of this unit by constructing a 2.1 m long x 2.0 m high fire-resistant masonry wall.

In doing this, the candidate must:

install a fire-rated steel door jamb

fire-proof two service penetrations of various sizes

fit fire-rated insulation between brick wall and roof covering

comply with fire control and separation requirements of National Construction Code (NCC) and other standards and legislation.

# Knowledge Evidence

To be competent in this unit, a person must demonstrate knowledge of:

characteristics, applications and limitations of materials used in fire-rated masonry construction:

mortar ingredients

fire-resistant masonry blocks and bricks

fire seals and insulation

principles of heat and effects on materials

fire control and separation required by the NCC and other legislation:

fire resistant levels (FRL) of materials used in fire-rated masonry requirements

processes and techniques for:

installing fire-rated masonry

damp proofing and flashing

installing fire-rated steel door jambs

fitting continuous fire-rated seal between structure and roof cladding

construction terminology relating to fire-rated masonry construction

functional and operational features of plant, equipment, hand and power tools

workplace safety:

job safety and environmental analysis (JSEA)

work at heights

hazardous manual handling

exposure to silica

exposure to loud noise

acceptable brickwork tolerances stated in Australian Standards and building codes

features of working drawings and specifications

environmental requirements for workplace processes and waste disposal.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

This unit must be assessed in the workplace or a close simulation reflecting workplace conditions and standards, materials, equipment, activities, responsibilities, procedures, safety requirements and environmental considerations.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad