# Assessment Requirements for CPCCBL3016 Construct battered masonry walls and piers

Release: 3

Assessment Requirements for CPCCBL3016 Construct battered masonry walls and piers

# Modification History

| Release 3 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 2 | This version first released with CPC Construction, Plumbing and Services Training Package Release 8.0. Changes to Performance Evidence. |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCBL3016A Construct battered masonry walls and piers. Updated to meet the Standards for Training Packages 2012. |

# Performance Evidence

To demonstrate competency in this unit, a candidate must meet the performance criteria of this unit by constructing a minimum 1.0 m high x 1.5 m long x 230 mm battered masonry wall at 90 degrees to an existing or constructed plumb wall.

In doing this, the candidate must:

construct a 1.0 m long x 230 mm wide buttressed pier at 90 degrees and to the same height as the battered wall

lay brick on edge capping to top of wall

comply with battered masonry walls and piers standards and codes.

# Knowledge Evidence

To be competent in this unit, a person must demonstrate knowledge of:

characteristics, applications and limitations of materials for battered wall construction:

mortar ingredients and consistency

waterproofing products

designed concrete foundations to counteract overturning or sliding of battered walls

processes and techniques for constructing various battered walls and piers:

installing drainage pipes

backfilling materials and aggregates

installing weep holes

construction terminology relating to battered wall construction

brick bond patterns

functional and operational features of plant, equipment, hand and power tools

workplace safety requirements:

hierarchy of control

hazardous manual tasks

exposure to silica

exposure to loud noise

features of working drawings and specification

key requirements of the National Construction Code (NCC) relevant to battered masonry wall and pier construction

standards and building codes relating to acceptable tolerances in brickwork

environmental requirements for workplace processes and waste disposal.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

This unit must be assessed in the workplace or a close simulation reflecting workplace conditions and standards, materials, equipment, activities, responsibilities, procedures, safety requirements and environmental considerations.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad