# CPCCDE3025 Operate remote-controlled plant on demolition sites

Release: 3

CPCCDE3025 Operate remote-controlled plant on demolition sites

# Modification History

| Release 3 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 2 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.1. Rectified naming of Element 5. |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCDE3025A Operate remote-controlled plant on demolition sites. Updated to meet the Standards for Training Packages 2012. |

# Application

This unit of competency specifies the skills and knowledge required to operate remote-controlled plant to conduct high risk demolition operations, such as overhead cutting, sawing and jack hammering, and to operate remote-controlled plant work in restricted spaces, such as tunnels. Remote-controlled plant may be operated at ground level, below ground or on suspended floors.

Completion of the general construction induction training program specified by the model Code of Practice for Construction Work is required for any person who is to carry out construction work. Achievement of CPCWHS1001 Prepare to work safely in the construction industry meets this requirement.

Licensing, legislative, regulatory or certification requirements apply to demolition work in different states and territories. Candidates are advised to consult with the relevant regulatory authorities.

# Pre-requisite Unit

Nil.

# Unit Sector

Demolition.

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Plan and prepare for work. | 1.1 | Interpret and apply compliance requirements for remote-controlled plant demolition operations. |
|  |  | 1.2 | Complete tasks according to manufacturer specifications and regulatory, legislative and workplace policies and requirements. |
|  |  | 1.3 | Obtain work instructions from relevant information, confirm with team members and other relevant personnel, and apply to planning and preparation. |
|  |  | 1.4 | Follow instructions and advice from structural engineer regarding load bearing capacities and demolition techniques on suspended floors. |
|  |  | 1.5 | Identify and apply work health and safety (WHS) requirements to task planning according to safety plans. |
|  |  | 1.6 | Identify environmental requirements for the project according to environmental plans. |
|  |  | 1.7 | Identify, select and fit personal protective equipment (PPE). |
|  |  | 1.8 | Check maintenance tools and equipment for serviceability and rectify and report faults. |
|  |  | 1.9 | Conduct remote-controlled plant pre-start, start-up, park and shutdown operational checks. |
|  |  | 1.10 | Check remote-controlled plant controls and functions, including brakes and manoeuvrability, for serviceability, and rectify and report faults. |
| 2 | Select, check and fit attachments. | 2.1 | Select remote-controlled plant attachments to carry out tasks consistent with job requirements. |
|  |  | 2.2 | Check attachments, arms and connections for excessive wear, and replace and report as required. |
|  |  | 2.3 | Fit attachments securely according to safe work method and fit safety pins or clips. |
|  |  | 2.4 | Check hydraulic components and rectify or replace faulty components and report actions. |
|  |  | 2.5 | Test operation of hydraulic system before applying load. |
|  |  | 2.6 | Test attachment operation by raising and lowering attachment and lock attachment in raised position for travel. |
|  |  | 2.7 | Remove, clean and store attachments. |
| 3 | Prepare demolition site. | 3.1 | Determine requirements of site demolition plan and discuss these with team members. |
|  |  | 3.2 | Conduct site inspection of property or dilapidation survey to determine condition of worksite and surrounds before work begins and amend and seek approval of safe work method statement (SWMS) as required. |
|  |  | 3.3 | Obtain confirmation from supervisor and regulatory authorities that all existing services have been disconnected. |
|  |  | 3.4 | Identify hazardous material for separate handling. |
|  |  | 3.5 | Position mechanical equipment and plant in operating locations and arrange appropriate exclusion zone and traffic control. |
| 4 | Operate remote-controlled plant loader for demolition process. | 4.1 | Identify and apply remote-controlled plant operating techniques to achieve optimum output while achieving specified tolerances. |
|  |  | 4.2 | Identify site hazards associated with remote-controlled plant demolition operations, including position of operator and personnel in relation to plant, equipment and environment and follow safe operating techniques to minimise risks. |
|  |  | 4.3 | Move remote-controlled plant safely between work locations and observe relevant codes and traffic management requirements. |
|  |  | 4.4 | Implement control measures to manage hazards arising from demolition process according to the demolition plan, safe work methods and regulatory requirements. |
|  |  | 4.5 | Assess demolition tasks immediately prior to starting and review SWMS to incorporate management of previously unpredicted hazards. |
|  |  | 4.6 | Operate remote-controlled plant at a safe distance to demolish building elements using suitable attachments according to approved demolition plan and current safe work method. |
|  |  | 4.7 | Maintain communication with team members during the demolition process. |
|  |  | 4.8 | Monitor worksite conditions and progress in consultation with team members and stop work or adjust techniques as required. |
| 5 | Carry out machine operator inspection and maintenance tasks. | 5.1 | Carry out machine operator inspection and maintenance tasks. |
|  |  | 5.2 | Carry out machine operator inspection and maintenance tasks. |
|  |  | 5.3 | Carry out machine operator inspection and maintenance tasks. |
| 6 | Clean up work area. | 6.1 | Clear work area and sort materials and waste and position for removal, as required, according to site demolition project environmental management plan. |
|  |  | 6.2 | Clean, check, maintain and store maintenance tools and equipment. |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

Supersedes and is equivalent to CPCCDE3025A Operate remote-controlled plant on demolition sites.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad