# CPCCJS3006 Construct, assemble and install composite external stairs

Release: 2

CPCCJS3006 Construct, assemble and install composite external stairs

# Modification History

| Release 2 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCJS3006A Construct fabricated stairs. Title change. Updated to meet the Standards for Training Packages 2012. |

# Application

This unit of competency specifies the skills and knowledge required to construct and install stairs with one or more flights incorporating composite components, meeting all relevant requirements of the National Construction Code (NCC), Australian Standards, work health and safety (WHS), and Commonwealth and state or territory legislation.

It includes installing treads, stringers, handrails and balustrades fabricated from composite (non-timber) materials.

Completion of the general construction induction training program specified by the model Code of Practice for Construction Work is required for any person who is to carry out construction work. Achievement of CPCWHS1001 Prepare to work safely in the construction industry meets this requirement.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

CPCCWHS2001 Apply WHS requirements, policies and procedures in the construction industry.

# Unit Sector

Carpentry and Joinery

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Plan and prepare. | 1.1 | Read and interpret work instructions and plan sequence of work. |
|  |  | 1.2 | Plan all work to comply with laws and regulations, the National Construction Code (NCC), Australian Standards, work health and safety (WHS) and environmental requirements, manufacturers’ specifications, workplace requirements, drawings and specifications. |
|  |  | 1.3 | Select tools and equipment, check for serviceability and report any faults. |
|  |  | 1.4 | Select and use personal protective equipment (PPE) for each part of the task. |
|  |  | 1.5 | Inspect work site, locate services, assess hazards and apply risk controls, including required signage and barricades. |
|  |  | 1.6 | Select materials required for task, calculate quantities, handle safely and prepare and position ready for use. |
| 2 | Set out stair components. | 2.1 | Determine exit and ground finish levels from drawings and site location. |
|  |  | 2.2 | Calculate rise, going and pitch of stair and check for compliance with requirements. |
|  |  | 2.3 | Make full-size set out of stairs to determine rise, going and pitch of stairs to actual location of landings, stringers, treads and newel posts. |
|  |  | 2.4 | Determine location of stair and newel posts from drawings and pitch of stairs and full-size set out. |
|  |  | 2.5 | Mark location of footings to layout of designed stairs to meet drawings and specifications and full-size set out. |
|  |  | 2.6 | Select material for stringers and check against set out. |
|  |  | 2.7 | Select components for newel posts set-out to design of stairs, storey rods and drawings and full-size set out. |
|  |  | 2.8 | Assemble newel posts and check to provide tight fit for stringers and bearers. |
|  |  | 2.9 | Check material for treads against set out and square to length. |
| 3 | Assemble and install stairs. | 3.1 | Prepare footings with post support. |
|  |  | 3.2 | Attach tread supports to stringers to set-out. |
|  |  | 3.3 | Erect newel posts and temporarily brace to plumb. |
|  |  | 3.4 | Attach stringers to building using fixings and fasteners to maintain stair width. |
|  |  | 3.5 | Fix treads and decking to location and bracing and lateral ties to newel posts. |
| 4 | Fit and fix hand railing and balustrade. | 4.1 | Check material for hand railing and balustrade to length. |
|  |  | 4.2 | Fit and fix hand railing into place above nosing line of a flight and above a landing deck. |
|  |  | 4.3 | Fit and fix balustrade into place. |
| 5 | Finish stairs. | 5.1 | Remove sharp edges and finish to specification. |
|  |  | 5.2 | Adhere non-slip surface to treads. |
| 6 | Clean up. | 6.1 | Clean up, meeting all legislative and workplace requirements for safety, waste disposal and materials handling. |
|  |  | 6.2 | Check, maintain and store tools and equipment and report any faults. |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

Supersedes and is equivalent to CPCCJS3006A Construct fabricated stairs

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad