# CPCCLHS3001 Licence to operate a personnel and materials hoist

Release: 2

CPCCLHS3001 Licence to operate a personnel and materials hoist

# Modification History

| Release 2 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCLHS3001A Licence to operate a personnel and materials hoist. Updated to meet the Standards for Training Packages 2012. |

# Application

This unit specifies the skills and knowledge required to safely operate a personnel and materials hoist, that is, a cantilever or tower hoist or several winches configured to operate as a hoist intended to carry goods, materials or people.

Completion of the general construction induction training program, specified in the Safe Work Australia model Code of Practice: Construction Work, is required by anyone carrying out construction work. Achievement of CPCWHS1001 Prepare to work safely in the construction industry meets this requirement.

Competence in this unit does not in itself result in a licence. A licence is obtained after competence is assessed under applicable Commonwealth, state or territory work health and safety (WHS) regulations.

# Pre-requisite Unit

Nil.

# Unit Sector

Licensing

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Plan task. | 1.1 | Review task instructions, consult with relevant persons to seek clarification as required, and obtain relevant workplace information. |
|  |  | 1.2 | Obtain and interpret information, including safe work method statements (SWMSs), required to ensure that activities are performed in compliance with workplace-specific and safe work requirements. |
|  |  | 1.3 | Obtain and interpret information required to ensure that equipment inspection, use, maintenance and storage complies with manufacturer requirements. |
|  |  | 1.4 | Identify workplace and task-specific hazards and determine required risk controls and safety measures and equipment. |
|  |  | 1.5 | Calculate load weight, check that it is within the working load limit (WLL) of the hoist and distribute it in accordance with manufacturer requirements. |
|  |  | 1.6 | Establish required communication methods with relevant persons. |
|  |  | 1.7 | Review emergency procedures in consultation with relevant persons. |
| 2 | Select and prepare equipment. | 2.1 | Put risk controls and safety measures and equipment in place in consultation with relevant persons. |
|  |  | 2.2 | Access hoist safely. |
|  |  | 2.3 | Visually check hoist for damage and defects. |
|  |  | 2.4 | Carry out pre-start checks, including checking the hoist logbook and controls. |
|  |  | 2.5 | Carry out operational checks, including testing hoist safety devices and checking for fire extinguisher and other emergency equipment. |
|  |  | 2.6 | Ensure hoist is operating correctly and is free of obstructions. |
|  |  | 2.7 | Check communication equipment, lighting and alarm systems to make sure they are working and fit for use. |
|  |  | 2.8 | Conduct test run of hoist, check for abnormal noises and shut down, tag and report identified abnormalities as required. |
|  |  | 2.9 | Isolate, report, tag out and record defective equipment. |
| 3 | Conduct hoist operations. | 3.1 | Maintain communication with relevant persons during hoist operations and emergency situations. |
|  |  | 3.2 | Operate loaded hoist in accordance with manufacturer requirements. |
|  |  | 3.3 | Use and interpret communication signals. |
|  |  | 3.4 | Conduct emergency lowering in accordance with manufacturer requirements. |
| 4 | Shut down and secure hoist. | 4.1 | Shut down hoist in accordance with manufacturer requirements. |
|  |  | 4.2 | Carry out shut-down checks. |
|  |  | 4.3 | Isolate and secure power against unauthorised access. |
|  |  | 4.4 | Report, tag out, isolate and record damages and defects. |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

Supersedes and is equivalent to CPCCLHS3001A Licence to operate a personnel and materials hoist.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad

Assessment Requirements for CPCCLHS3001 Licence to operate a personnel and materials hoist

# Modification History

| Release 2 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCLHS3001A Licence to operate a personnel and materials hoist. Updated to meet the Standards for Training Packages 2012. |

# Performance Evidence

To demonstrate competency in this unit, a candidate must meet the elements and performance criteria by operating a personnel and materials hoist, and conducting three lifts carrying loads of different weights. The hoist must be stopped at three different levels, including at least one floor.

The candidate must:

check relevant workplace information, including safe work method statements (SWMSs), and equipment service and maintenance records and checklists

perform all activities in compliance with safe work practices and workplace-specific procedures and policies

check, use, maintain and store equipment in compliance with manufacturer requirements

identify hazards and use appropriate risk controls and safety measures and equipment

calculate load weights and confirm that they are within the capacity of the hoist

check and use safety devices

move load

implement shut-down procedures.

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

safe work requirements for operating a personnel and materials hoist crane under Australian Standards, and Commonwealth and state or territory work health and safety (WHS) legislation and codes of practice

workplace information, including legislative requirements covered by:

SWMSs

safety data sheets (SDSs)

permits and certifications

information about equipment:

service and maintenance checklists and records

manufacturer specifications and manuals

workplace procedures, including emergency plans and incident reporting

hazard identification and mitigation strategies, including the hierarchy of control:

elimination

substitution

isolation

engineering controls

administrative controls

use of personal protective equipment (PPE)

hazards commonly encountered in hoist operations:

ground instability

obstructions

impact of other tasks being performed in the workplace

other plant

hoist overload

hazardous chemicals and dangerous goods

electrical hazards

insufficient lighting

hazardous manual tasks

damaged or poor-quality equipment

pedestrian traffic

wind and other adverse weather conditions

falling objects

falls from heights

risk controls and equipment:

firefighting equipment

safety tags on electrical switches and isolators

traffic barricades and controls

pedestrian barricades and controls

hoist safety devices, including:

hoist safety gates and guards

hoist safety interlocks

adequate illumination

testing and tagging requirements and electrical hazards associated with power supply for hoists

processes to prevent interference to and from radio equipment being used near a personnel and materials hoist

PPE:

hard hat

safety boots

gloves

high-visibility clothing

breathing, hearing, sight, skin and sun protection

relevant persons:

other hoist operators

riggers

maintenance personnel

supervisors

communication equipment:

two-way radios

intercoms

bells

lights

pre-start operational checks to ensure:

safe access and egress, including ground conditions

security and safe distance between tower ties

earth leakage protection of power supply

power leads are secured above ground level and not attached to scaffolds or building structure

tower guides are clean and free of rust and damage

signage is clearly displayed and legible

brakes and drive mechanism are in good working order

overhead protection

intercom and signalling systems are in good working order

barriers, fencing and gates are in place

lubrication has been applied where required

hoist rope, where used, is safe

sheaves and anchorage points are accessible

obstructions have been removed

loose gear and equipment have been removed

processes for determining the weight of the load from labels, markings and load paperwork

processes for identifying and interpreting the working load limit (WLL) of hoists

routine problems encountered in the operation of a hoist, and inspection techniques and adjustments required for correction

processes for responding to unplanned and unsafe situations, including equipment failure or loss of control and adverse weather conditions

emergency and evacuation procedures, including fire procedures

shut-down procedures:

position platform at designated parking level

isolate power from control panel

isolate and secure mains power supply

remove key from control panel

secure landing gates to prevent unauthorised access

secure fencing and barriers around base to prevent unauthorised access

hoist characteristics, capabilities and operation to suit loads of different shapes, sizes and weights.

# Assessment Conditions

Assessors must meet the requirements for assessors outlined in the Standards for Registered Training Organisations.

Only assessors who are accredited in the licence class by the appropriate WHS regulator for the jurisdiction where the licence is obtained are permitted to conduct the final high-risk work licence assessment. The final licence assessment will only be undertaken with candidates who have completed training and been formally assessed against all elements in this unit.

Assessment must be conducted in the workplace or in a simulated workplace environment using realistic workplace conditions, materials, activities, responsibilities, procedures, safety requirements and environmental considerations, including:

the use of full-scale, industry-standard equipment, not simulators

performance of tasks within the timelines expected in a workplace.

Candidates must have access to:

all personnel and equipment required to perform the tasks specified in the Performance Evidence

a personnel and materials hoist capable of carrying loads of different weights and able to be stopped at three different levels, including at least one floor

workplace information and records, including:

equipment and maintenance checklists

record system for service and maintenance history

incident reports

workplace procedures, including SWMSs, SDSs and emergency plans

equipment manuals and manufacturer specifications

relevant plant supplier information.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad