# Assessment Requirements for CPCCLTC4001 Licence to operate a tower crane

Release: 3

Assessment Requirements for CPCCLTC4001 Licence to operate a tower crane

# Modification History

| Release 3 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 2 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.1. Minor edits to formatting. |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCLTC4001A Licence to operate a tower crane. Updated to meet the Standards for Training Packages 2012. |

# Performance Evidence

To demonstrate competency in this unit, a candidate must meet the elements and performance criteria by safely operating from the cabin, a tower crane with a minimum tower height of 20 m and a maximum rated capacity of at least 3000 kg. The candidate must move four loads of different shapes, sizes and weights in conjunction with a licensed dogger.

For all loads, the candidate must:

check relevant workplace information, including safe work method statements (SWMSs), and equipment service and maintenance records and checklists

perform all activities in compliance with safe work practices and workplace-specific procedures and policies

check, use, maintain and store equipment in compliance with manufacturer requirements

identify hazards and implement appropriate risk controls and safety measures and equipment

confirm that load weight is within the rated capacity of the crane

locate and interpret signs and labels

check and use safety devices

lift and move loads using the main hook through an obstacle course including a 90-degree minimum slew, with the load in full view of the candidate and using all crane operational controls:

luffing/trolleying levers

hoisting and lowering levers

slewing levers

foot pedals

engine speed control

use two-way radio and interpret hand and whistle signals to communicate with licensed dogger

implement shutdown procedures.

Movements must include:

luffing or trolleying

slewing

hoisting and lowering.

Loads must include:

a load of more than 50% of the configured capacity of the crane

stillage containing at least ten scaffolding standards or loose steel pipes of 200 kg or more that requires a licensed dogger to sling

an uneven load that requires a licensed dogger to sling

a round load with a minimum diameter of 300 mm and minimum length of 3 m that requires a licensed dogger to sling.

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

safe work requirements for operating a tower crane under Australian Standards, and Commonwealth and state or territory work health and safety (WHS) legislation and codes of practice

workplace information, including legislative requirements covered by:

SWMSs

permits and certifications

information about equipment:

service and maintenance checklists and records

manufacturer specifications and manuals

workplace procedures, including emergency plans and incident reporting

hazard identification and mitigation strategies, including the hierarchy of control:

elimination

substitution

isolation

engineering controls

administrative controls

use of personal protective equipment (PPE)

hazards commonly encountered in tower crane operations:

overhead and underground hazards, including powerlines

other cranes and booms

other plant

obstructions

combustible substances, including fuel

electrical hazards

insufficient lighting

traffic

wind and other adverse weather conditions

pedestrian traffic

work required at heights

risk controls and safety measures and equipment:

safety tags on electrical switches and isolators

insulated powerlines

safety observer

exclusion zone

disconnected power

traffic barricades and controls

pedestrian controls

movement of obstructions

evacuation procedures

adequate illumination

firefighting equipment, including a fire extinguisher of adequate capacity near the operator’s position

appropriate earthing for the conditions, such as static electricity build up due to electromagnetic radiation (EMR) or wind

fall prevention and fall arrest equipment

PPE:

hard hat

safety boots

gloves

high-visibility clothing

breathing, hearing, sight, skin and sun protection

hand and whistle communication signals:

stop

hoist up and down

luff boom up and down

trolley in and out

slew left and right

signs and labels:

crane data plates and labels

load charts

crane decals

operational control lever labels

tower crane safety devices:

audible and visual devices

aviation lights

illumination

motion limits

overload limits

anti-collision devices

crane controls:

luffing levers

hoisting and lowering levers

slewing levers

trolleying levers

slew and boom brake and locks

foot pedals

emergency stop control

engine speed control

crane movements:

luffing

slewing

hoisting and lowering

trolleying

multiple crane communication systems

processes for responding to unplanned and unsafe situations, including equipment failure or loss of control

emergency and evacuation procedures, including fire procedures

shutdown procedures as applicable:

raise boom or jib to shutdown position

park trolley in shutdown position

retract hoist rope and hook block

idle engine to stabilise temperature

turn off engine

place in weather vane mode

remove key from ignition

isolate electrical supply

isolate fuel

lock and secure cabin

lock and secure platform access hatch

tower crane characteristics, capabilities and operation to suit loads of different shapes, sizes and weights

use and interpretation of crane manufacturer specifications, including load charts to enable the crane to be configured within its rated capacity.

# Assessment Conditions

Assessors must meet the requirements for assessors outlined in the Standards for Registered Training Organisations.

Only assessors who are accredited in the licence class by the appropriate WHS regulator for the jurisdiction where the licence is obtained are permitted to conduct the final high-risk work licence assessment. The final licence assessment will only be undertaken with candidates who have completed training and been formally assessed against all elements in this unit.

Assessment must be conducted in the workplace or in a simulated workplace environment using realistic workplace conditions, materials, activities, responsibilities, procedures, safety requirements and environmental considerations, including:

the use of full-scale, industry-standard equipment, not simulators

performance of tasks within the timelines expected in a workplace.

Candidates must have access to:

a cabin-operated tower crane with a minimum tower height of 20 m and a maximum rated capacity of at least 3000 kg, which is in a safe and compliant condition

lifting and other equipment required for operating a tower crane

risk controls and safety measures and equipment, including a fire extinguisher of adequate capacity near the operator’s position

communication equipment, including whistle and two-way radios

loads as specified in the Performance Evidence

licensed dogger to sling and direct loads

workplace information and records, including:

record system for service and maintenance history

incident reports

workplace procedures, including emergency plans

equipment manuals and manufacturer specifications.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad