# CPCCLTC4002 Licence to operate a self-erecting tower crane

Release: 3

CPCCLTC4002 Licence to operate a self-erecting tower crane

# Modification History

| Release 3 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 2 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.1. Missing text in Element and Performance Criteria Point 2 corrected. |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCLTC4002A Licence to operate a self-erecting tower crane. Updated to meet the Standards for Training Packages 2012. |

# Application

This unit specifies the skills and knowledge required to safely operate a self-erecting tower crane.

Self-erecting cranes are common on small to medium-sized construction sites. They usually have a horizontal boom that folds out during erection and are usually operated by remote control rather than from a cabin. For licencing purposes, self-erecting tower cranes are a separate category to other types of tower crane.

Completion of the general construction induction training program, specified in the Safe Work Australia model Code of Practice: Construction Work, is required by anyone carrying out construction work. Achievement of CPCWHS1001 Prepare to work safely in the construction industry meets this requirement.

Competence in this unit does not in itself result in a licence. A licence is obtained after competence is assessed under applicable Commonwealth, state or territory work health and safety (WHS) regulations.

# Pre-requisite Unit

Nil.

# Unit Sector

Licencing

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Plan task. | 1.1 | Review task instructions, consult with relevant persons to seek clarification as required, and obtain relevant workplace information. |
|  |  | 1.2 | Obtain and interpret information, including safe work method statements (SWMSs), required to ensure that activities are performed in compliance with workplace-specific and safe work requirements. |
|  |  | 1.3 | Obtain and interpret information required to ensure that equipment inspection, use, maintenance and storage complies with manufacturer requirements. |
|  |  | 1.4 | Identify workplace and task-specific hazards, including, where relevant, hazards associated with working at heights, and determine required risk controls and safety measures and equipment. |
|  |  | 1.5 | Calculate load weight in consultation with relevant persons. |
|  |  | 1.6 | Consult with relevant persons to confirm that load weight is within the rated capacity of the crane and the working load limit (WLL) of the lifting gear. |
|  |  | 1.7 | Determine and inspect crane and load movement paths. |
|  |  | 1.8 | Confirm implementation of and compliance with traffic management plan. |
|  |  | 1.9 | Establish required communication methods with relevant persons. |
|  |  | 1.10 | Establish and maintain communication with relevant persons to ensure lift plan and risk controls are communicated clearly, including any impact on other workplace activities. |
| 2 | Prepare for task. | 2.1 | Check signs and labels to ensure visibility and legibility. |
|  |  | 2.2 | Assess wind and other adverse weather conditions and determine whether their impact on the crane and loads is within the manufacturer requirements for safe operation. |
|  |  | 2.3 | Safely access the area from which the crane will be operated. |
|  |  | 2.4 | Test crane safety devices in accordance with workplace requirements. |
|  |  | 2.5 | Carry out pre-start crane checks and shut down, tag out, report and record identified damage or defects, in accordance with safe work practices, workplace-specific procedures and policies, and manufacturer requirements. |
|  |  | 2.6 | Check that crane logbook has been completed and signed, that it is correct for crane type and current compliance, and that any rectifications have been signed off. |
|  |  | 2.7 | Start crane, carry out operational checks, check for abnormal noises, smoke or fumes and shut down, tag out, report and record identified abnormalities, in accordance with safe work practices, workplace-specific procedures and policies, and manufacturer requirements. |
| 3 | Transfer load. | 3.1 | Position boom or jib and hook block over load as directed by licensed dogger. |
|  |  | 3.2 | Carry out test lift to allow for checks and adjustments to slinging to be made by licensed dogger. |
|  |  | 3.3 | Under the direction of a licensed dogger, transfer loads using appropriate crane movements. |
|  |  | 3.4 | Use radio and interpret hand and whistle signals to communicate with licensed dogger. |
|  |  | 3.5 | Lower and land load, allowing lifting gear to be disconnected. |
| 4 | Shut down and secure crane. | 4.1 | Apply hoist, trolley and travel brakes. |
|  |  | 4.2 | Shut down crane in accordance with manufacturer requirements. |
|  |  | 4.3 | Carry out routine shut-down checks, and secure crane and equipment against unauthorised access. |
|  |  | 4.4 | Safely exit the area from which the crane has been operated. |
|  |  | 4.5 | Report and record any damage or defects. |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

Supersedes and is equivalent to CPCCLTC4002A Licence to operate a self-erecting tower crane.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad

Assessment Requirements for CPCCLTC4002 Licence to operate a self-erecting tower crane

# Modification History

| Release 3 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 2 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.1. Missing text in Element and Performance Criteria Point 2 corrected. |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCLTC4002A Licence to operate a self-erecting tower crane. Updated to meet the Standards for Training Packages 2012. |

# Performance Evidence

To demonstrate competency in this unit, a candidate must meet the elements and performance criteria by safely operating a self-erecting tower crane with a maximum rated capacity of at least 2000 kg to move at least four loads of different shapes, sizes and weights in conjunction with a licensed dogger slinging and guiding loads.

For all loads, the candidate must:

check relevant workplace information, including safe work method statements (SWMSs) and equipment service and maintenance records and checklists

perform all activities in compliance with safe work practices and workplace-specific procedures and policies

check, use, maintain and store equipment in compliance with manufacturer requirements

identify hazards and implement appropriate risk controls and safety measures and equipment

confirm that load weight is within the rated capacity of the crane

locate and interpret signs and labels

check and use safety devices

lift and move loads using the main hook through an obstacle course, including a 90-degree minimum slew, using all crane operational controls:

trolleying levers

hoisting and lowering levers

slewing levers

use two-way radio and interpret hand and whistle signals to communicate with licensed dogger

implement shutdown procedures.

Movements must include:

luffing, as applicable, or trolleying

slewing

hoisting and lowering.

Loads must include:

a load of more than 50% of the configured capacity of the crane

stillage containing at least ten scaffolding standards or loose steel pipes of 200 kg or more that requires a licensed dogger to sling

an uneven load that requires slinging

a round load with a minimum diameter of 300 mm and minimum length of 3 m that requires slinging.

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

safe work requirements for operating a self-erecting tower crane under Australian Standards, and Commonwealth and state or territory work health and safety (WHS) legislation and codes of practice

workplace information, including legislative requirements covered by:

SWMSs

permits and certifications

information about equipment:

service and maintenance checklists and records

manufacturer specifications and manuals

workplace procedures, including emergency plans and incident reporting

hazard identification and mitigation strategies, including the hierarchy of control:

elimination

substitution

isolation

engineering controls

administrative controls

use of personal protective equipment (PPE)

hazards commonly encountered in self-erecting tower crane operations:

overhead and underground hazards, including powerlines

other cranes and booms

other plant

obstructions

combustible substances, including fuel

electrical hazards

insufficient lighting

traffic

wind and other adverse weather conditions

pedestrian traffic

work required at heights

risk controls and safety measures and equipment:

safety tags on electrical switches and isolators

insulated powerlines

safety observer

exclusion zone

disconnected power

traffic barricades and controls

pedestrian controls

movement of obstructions

evacuation procedures

adequate illumination

firefighting equipment

appropriate earthing for the conditions such as static electricity build up due to electromagnetic radiation (EMR) or wind

fall prevention and fall arrest equipment

PPE:

hard hat

safety boots

gloves

high-visibility clothing

breathing, hearing, sight, skin and sun protection

hand and whistle communication signals:

stop

hoist up and down

trolley in and out

slew left and right

signs and labels:

crane data plates and labels

load charts

crane decals

operational control lever labels

self-erecting tower crane safety devices:

audible and visual devices

aviation lights

illumination

motion limits

overload limits

radios

anti-collision devices

crane controls:

hoisting and lowering levers

slewing levers

slew brake and locks

trolleying levers

emergency stop control

crane movements:

slewing

hoisting and lowering

trolleying

multiple crane communication systems

processes for responding to unplanned and unsafe situations, including equipment failure or loss of control

emergency and evacuation procedures, including fire procedures

shutdown procedures:

raise boom or jib to shut-down position

park trolley in shut-down position

retract hoist rope and hook block

place in weather vane mode

turn off and secure remote to prevent unauthorised access

isolate electrical supply

lock and secure against access

self-erecting tower crane characteristics, capabilities and operation to suit loads of different shapes, sizes and weights

use and interpretation of crane manufacturer specifications, including load charts to enable the crane to be configured within its rated capacity.

# Assessment Conditions

Assessors must meet the requirements for assessors outlined in the Standards for Registered Training Organisations.

Only assessors who are accredited in the licence class by the appropriate WHS regulator for the jurisdiction where the licence is obtained are permitted to conduct the final high-risk work licence assessment. The final licence assessment will only be undertaken with candidates who have completed training and been formally assessed against all elements in this unit.

Assessment must be conducted in the workplace or in a simulated workplace environment using realistic workplace conditions, materials, activities, responsibilities, procedures, safety requirements and environmental considerations, including:

the use of full-scale, industry-standard equipment, not simulators

performance of tasks within the timelines expected in a workplace.

Candidates must have access to:

a self-erecting tower crane with a maximum rated capacity of at least 2000 kg, which is in a safe and compliant condition

lifting and other equipment required for self-erecting tower crane operations

risk controls and safety measures and equipment

communication equipment, including whistle and two-way radios

loads as specified in the Performance Evidence

a licensed dogger to sling and direct loads

workplace information and records, including:

equipment and maintenance checklists

record system for service and maintenance history

incident reports

workplace procedures, including emergency plans

equipment manuals and manufacturer specifications.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad