# CPCCOM3006 Carry out levelling operations

Release: 2

CPCCOM3006 Carry out levelling operations

# Modification History

| Release 2 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCCM3006 Carry out levelling operations. Updated to reflect current industry terminology, tools and equipment. |

# Application

This unit of competency specifies the skills and knowledge required to carry out levelling operations to establish correct and accurate set-out of buildings and components, meeting all relevant requirements of the National Construction Code (NCC), Australian Standards, work health and safety (WHS), and Commonwealth and state or territory legislation.

It includes:

using the rise and fall and height of collimation methods

setting up, testing and using levelling devices

undertaking closed and open traverses using a range of levelling equipment

calculating distances using stadia lines.

Completion of the general construction induction training program specified by the model Code of Practice for Construction Work is required for any person who is to carry out construction work. Achievement of CPCWHS1001 Prepare to work safely in the construction industry meets this requirement.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

Nil

# Unit Sector

Common

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Plan and prepare. | 1.1 | Read and interpret work instructions and plan sequence of work. |
|  |  | 1.2 | Plan all work to comply with laws and regulations, Australian Standards, work health and safety (WHS) and environmental requirements, manufacturers’ specifications, workplace requirements, drawings and specifications. |
|  |  | 1.3 | Select tools and equipment, including personal protective equipment (PPE), check for serviceability and report any faults. |
|  |  | 1.4 | Fit PPE correctly. |
|  |  | 1.5 | Inspect work site, assess hazards and services, and apply risk controls, including required signage and barricades. |
|  |  | 1.6 | Select materials required for task, calculate quantities, handle safely and prepare and position ready for use. |
|  |  | 1.7 | Confirm team roles and verbal and non-verbal communication signals. |
| 2 | Carry out levelling procedures using rise and fall method. | 2.1 | Set up and test instrument for operational effectiveness and accuracy. |
|  |  | 2.2 | Set up or locate datum point. |
|  |  | 2.3 | Take and record readings from the datum and at nominated or selected stations following project specifications. |
|  |  | 2.4 | Identify backsights, intermediate sights and foresights, and book levels. |
|  |  | 2.5 | Transfer instrument to another location, repeat process, and record change station and record readings. |
|  |  | 2.6 | Calculate reduced levels for all stations using rise and fall method, and check accuracy of recordings using the three-check method. |
| 3 | Carry out levelling procedures using height of collimation method. | 3.1 | Set up and test instrument for operational effectiveness and accuracy, and check tolerances. |
|  |  | 3.2 | Set up or locate datum point. |
|  |  | 3.3 | Take and record readings to datum and establish the height of collimation. |
|  |  | 3.4 | Take readings at nominated or selected stations following project specifications, and record in field book. |
|  |  | 3.5 | Transfer instrument to another location, establish new height of collimation, and record change station in field book. |
|  |  | 3.6 | Calculate reduced levels using height of instrument method. |
|  |  | 3.7 | Calculate reduced levels for all stations and record heights and levels in field book. |
|  |  | 3.8 | Check accuracy of readings using height of collimation method of calculation and three check method. |
| 4 | Calculate distances using stadia lines. | 4.1 | Calculate distances from instrument to stations using staff, stadia lines, and identified factor of levelling instrument |
|  |  | 4.2 | Record readings and distances. |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

Supersedes and is equivalent to CPCCCM3006 Carry out levelling operations.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad

Assessment Requirements for CPCCOM3006 Carry out levelling operations

# Modification History

| Release 2 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCCM3006 Carry out levelling operations. Updated to reflect current industry terminology, tools and equipment. |

# Performance Evidence

To demonstrate competency, a candidate must meet the performance criteria of this unit by carrying out levelling operations over a minimum of two change points to:

identify site hazards

establish approximate distances and transfer of reduced levels

measure and record:

ground levels at respective critical set-out points

heights or levels for building components

heights or levels of ceilings and floors

slab or pad levels for placement of steel columns or masonry piers

determine shooting levels for:

concrete slabs

excavation or footings

transfer levels or heights for construction projects

calculate rise and fall and height from collimation.

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

compliance requirements of the Australian Standards relevant to carrying out levelling operations

workplace quality policies and standards relevant to carrying out levelling operations

safety requirements for carrying out levelling operations

basic construction and levelling processes

construction plans, symbols and construction terminology relevant to carrying out levelling operations

processes for interpreting engineering drawings and sketches for carrying out levelling operations

processes for setting out construction tasks for levelling operations

project quality requirements for carrying out levelling operations

site isolation and traffic control responsibilities and authorities

levelling techniques commonly used in construction work

levelling activities:

establishing approximate distances and transfer of reduced levels

locating site hazards and services

measuring and recording ground levels at respective critical set-out points

measuring and recording heights or levels for installing building components

measuring and recording heights or levels of ceilings and floors

measuring and recording slab or pad levels for placement of steel columns or masonry piers

taking levels for concrete slabs

taking levels for excavation or footings

calculating rise and fall and height from collimation

types of tools and equipment used for carrying out levelling operations:

spirit levels

straight edge

automatic/optical levelling devices

laser levels

water level

processes for checking operation and tolerances:

two peg test for automatic level

reverse readings for spirit level

types of level readings:

negative readings

datum

backsight

foresight

intermediate sight

plumb

maintenance of levelling devices:

authorised operator servicing and minor replacements

cleaning

monitoring, recording and reporting faults.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

This unit must be assessed in the workplace or a close simulation reflecting workplace conditions and standards, materials, equipment, activities, responsibilities, procedures, safety requirements and environmental considerations.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad