# Assessment Requirements for CPCCON3043 Cure concrete

Release: 2

Assessment Requirements for CPCCON3043 Cure concrete

# Modification History

| Release 2 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCCO3043 Cure concrete |

# Performance Evidence

To demonstrate competency, a candidate must meet the elements and performance criteria of this unit by curing concrete in either a non-precast work environment or a precast work environment:

for the non-precast work environment, curing concrete at five different sites, each measuring at least 100 square metres

for the precast work environment, curing a minimum of three panels on five different occasions.

A total of two curing techniques must be used:

curing compound

another technique selected from the following:

covering

flooding or ponding

fogging

steaming.

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

basic science of concrete involving cement chemical reaction (hydration) dependent on water content, temperature and time:

effect on plastic state performance

benefits and non-benefits of admixture types and doses

plastic shrinkage cracking risk

effect on hardened state performance:

early age and later age strength development

drying shrinkage and cracking

compatibilities of curing compounds with different floor finishes including vinyl, timber and natural stone products

compliance requirements of legislation, regulations, codes and Australian Standards relevant to concrete curing tasks:

environmental protection and waste disposal

work health and safety (WHS)

concrete curing techniques:

covering

curing compound

flooding or ponding

fogging

steaming

effects of temperature, wind and low humidity on the properties of concrete:

detrimental effect of water addition to concrete properties

precautions that should be taken to minimise any potential adverse effects when curing concrete

health risks associated with silica dust exposure

methods for calculating curing duration

specifications used to interpret material quantities and concrete curing requirements

types and applications of materials and compounds used when curing concrete

types, characteristics, uses and limitations of tools and equipment used when curing concrete

ways to minimise moisture loss during curing using different curing methods

workplace requirements for curing concrete:

cleaning up the work area

maintaining and storing materials, tools and equipment

quality

reporting problems

safety requirements including personal protective equipment (PPE).

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

Assessment must be conducted in the workplace or a simulated workplace using realistic conditions, materials, activities, responsibilities, procedures, safety requirements and environmental considerations.

Candidates must have access to documentation, specifications, materials, tools and equipment required to achieve the performance evidence.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad