# Assessment Requirements for CPCCPB3001 Fix standard plasterboard wall sheets

Release: 2

Assessment Requirements for CPCCPB3001 Fix standard plasterboard wall sheets

# Modification History

| Release 2 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and equivalent to CPCCPB3001A Fix standard plasterboard wall sheets. Updated to meet the Standards for Training Packages 2012. |

# Performance Evidence

To demonstrate competency, a candidate must satisfy all of the elements, performance criteria and foundation skills of this unit by fixing a minimum 15 square metres of plasterboard wall sheets, including:

butt joints between studs

backblocking joints

provisions for a door and a window opening

joints positioned over windows and doors away from the corner of the opening

a penetration for electrical and plumbing services.

All work must be performed to the standard required in the workplace and must comply with Commonwealth and state/territory laws and regulations, the National Construction Code, Australian Standards, work health and safety (WHS) and environmental requirements, manufacturers’ specifications, and workplace requirements.

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

compliance requirements of the National Construction Code and Australian Standards for fixing plasterboard sheets, including AS/NZS2589 Gypsum linings - application and finishing

levels of finish, including notifications required for application for level 5

requirements for spacing of stud adhesive and temporary blocks

quality policies and standards for fixing plasterboard sheets

safety requirements, including personal protective equipment (PPE), job safety analyses and safety data sheets

processes for safe handling and lifting of standard plasterboard sheets

environmental requirements for fixing standard plasterboard sheets

manufacturers’ product specifications for fixing wall sheets

types and uses of adhesives and fasteners for fixing standard plasterboard sheets

materials storage and environmentally friendly waste management

plans, drawings and specifications for fixing standard plasterboard sheets

plasterboard fixing techniques for standard plasterboard sheets

plasterboard materials

processes for the calculation of material requirements for fixing standard plasterboard sheets

types of tools and equipment for fixing standard plasterboard sheets.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

This unit must be assessed:

in the workplace or a close simulation of the workplace environment, provided that simulated or project-based assessment techniques replicate construction workplace conditions, materials, activities, responsibilities and procedures

using realistic tasks or simulated tasks, relevant specifications and work instructions, standard work practices, safety requirements and environmental constraints.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad