# CPCCPD3022 Apply paint by brush and roller

Release: 3

CPCCPD3022 Apply paint by brush and roller

# Modification History

| Release 3 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 2 | This version first released with CPC Construction, Plumbing and Services Training Package Release 6.4. Correction to Training Package title in the Modification History from CPC Property Services to CPC Construction, Plumbing and Services Training Package. |
| Release 1 | This version first released with CPC Property Services Training Package Release 5.0. Supersedes and is equivalent to CPCCPD3022A Apply paint by brush and roller. Updated to meet the Standards for Training Packages 2012. |

# Application

This unit of competency specifies the skills and knowledge required to apply brushed or rolled paint coatings to different materials to form a protective and decorative painted finish.

The unit includes preparation of the work area, mixing of materials, application of paint, finishing of the surface and completion of clean-up activities.

A person who has achieved this unit of competency would be expected to take responsibility for organising and completing these tasks with a high degree of self direction.

Completion of the general construction induction training program specified by the model Code of Practice for Construction Work is required for any person who is to carry out construction work.  Achievement of CPCWHS1001 Prepare to work safely in the construction industry meets this requirement.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

| CPCCWHS2001 | Apply WHS requirements, policies and procedures in the construction industry |
| --- | --- |

# Unit Sector

Painting and Decorating

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Plan and prepare. | 1.1 | Review work instructions to apply paint by brush and roller. |
|  |  | 1.2 | Plan all work to meet relevant requirements of work health and safety (WHS), Commonwealth and state or territory legislation, environmental plans and obligations, manufacturers’ specifications, and workplace requirements. |
|  |  | 1.3 | Conduct work with regard to responsibilities in heritage-listed buildings. |
|  |  | 1.4 | Select and fit personal protective equipment (PPE) as required for each task. |
|  |  | 1.5 | Inspect work site, assess hazards and apply risk controls, including required signage and barricades. |
|  |  | 1.6 | Identify materials appropriate to the work application, calculate quantities and prepare, handle safely and locate ready for use. |
|  |  | 1.7 | Select tools and equipment, check for serviceability and report any faults. |
| 2 | Prepare work area and materials. | 2.1 | Check that materials and substrate surfaces are prepared in accordance with manufacturers’ recommendations and relevant Australian Standards. |
|  |  | 2.2 | Protect surrounding surfaces with drop sheets, masking or removal of objects. |
|  |  | 2.3 | Check that where doors, windows and associated furniture are removed they are safely stacked, stored and protected. |
|  |  | 2.4 | Check work site to ensure provision of adequate ventilation and implementation of precautions to prevent fire and explosion. |
|  |  | 2.5 | Implement measures to ensure application area remains free of dust and foreign matter. |
|  |  | 2.6 | Mix each of oil based, water based and two pack paint, to designed proportion, consistency, specified ratio and drying time. |
|  |  | 2.7 | Stir paint and colours thoroughly using separate stirring sticks or other devices. |
| 3 | Apply paint with brush or roller. | 3.1 | Select brush, roller or brush/roller combination for surface profile, size of area, type of paint and finish. |
|  |  | 3.2 | Apply paint to achieve required level of opacity, finish, texture and sheen. |
| 4 | Clean up. | 4.1 | Clean up, meeting all legislative and workplace requirements for safety, waste disposal, materials handling and protection of the environment. |
|  |  | 4.2 | Clear work area, reuse or recycle materials and place waste and unwanted materials into job waste bins or rubbish stockpile. |
|  |  | 4.3 | Check and maintain tools and equipment and clean with appropriate solvents. |
|  |  | 4.4 | Dispose of paint waste and water and solvents used in cleaning painting equipment in an environmentally sustainable manner. |
|  |  | 4.5 | Store tools, equipment and sealed unused materials to avoid spontaneous ignition. |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

Supersedes and equivalent to CPCCPD3022A Apply paint by brush and roller.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad