# Assessment Requirements for CPCCPD3027 Remove and apply wallpaper

Release: 4

Assessment Requirements for CPCCPD3027 Remove and apply wallpaper

# Modification History

| Release 4 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 3 | This version first released with CPC Construction, Plumbing and Services Training Package Release 6.4. Correction to Training Package title in the Modification History from CPC Property Services to CPC Construction, Plumbing and Services Training Package Release 1. |
| Release 2 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.1. Minor edits to formatting. |
| Release 1 | This version first released with CPC Property Services Training Package Release 5.0. Supersedes and is equivalent to CPCCPD3027A Apply wallpaper. Minor title change. Updated to meet the Standards for Training Packages 2012. |

# Performance Evidence

To demonstrate competency, a candidate must meet the performance criteria of this unit by:

removing a minimum of six square metres of wallpaper

preparing and applying a minimum of six square metres of lining paper

preparing and applying a minimum of six square metres of vinyl wallpaper

preparing and applying a minimum of six square metres of unpasted wallpaper.

In removing this wallpaper, the following procedures, products and techniques must be included:

dry stripping

soaking

steam stripping

In applying this wallpaper, a minimum of the following must be included:

walls and internal and external corners

a reveal

a window

an arch

walls containing power points and switches

a ceiling to a minimum of three metres by three strips wide

one set pattern

one drop pattern

one random pattern.

All work must be performed to the standard required in the workplace and must meet the requirements of work health and safety (WHS), Commonwealth and state or territory legislation, manufacturers’ specifications, and environmental plans and obligations.

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

safety requirements relevant to applying wallpaper

environmental requirements relevant to applying wallpaper

painting and decorating terminology

use and application of woven and non-woven wallpaper

plans, drawings and specifications relevant to applying wallpaper

plans for hanging wallpaper

common wallpapering defects

compatibility of preparatory materials and wallpaper

types of wallpaper:

borders

lining paper

unpasted and ready pasted wallpapers (simplex and duplex)

vinyl wallpaper

washable wallpaper

paste on wall

types of wallpaper adhesives:

cellulose

latex

acetate

starch

other special adhesive blends

preparation for wallpaper applications for walls:

set plaster

flat surfaces

curved surfaces

surface preparation for:

fibre cement products

fibrous plaster

medium density fibre board

plasterboard

lined surface

rendered surface

types, characteristics, uses and limitations of tools and equipment relevant to applying wallpaper

procedures used to apply straight pattern match, drop pattern match and random match wallpapers

use of laser levels relevant to applying wallpaper

procedures, products and techniques associated with removal of wallpaper, including:

dry stripping

soaking

steam stripping

processes for the calculation of material requirements relevant to applying wallpaper

quality requirements relevant to applying wallpaper

surface preparation techniques for the application of wallpaper

types and properties of wall coverings and their suitability to various substrates

types, uses and limitations of commonly used wallpaper adhesives

wallpaper hanging processes and techniques.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

This unit must be assessed:

in the workplace or a close simulation of the workplace environment, provided that simulated or project-based assessment techniques replicate construction workplace conditions, materials, activities, responsibilities and procedures.

using realistic tasks or simulated tasks, relevant specifications and work instructions, standard work practices, safety requirements and environmental constraints.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad