# CPCCPD3029 Remove graffiti and apply anti-graffiti coatings

Release: 3

CPCCPD3029 Remove graffiti and apply anti-graffiti coatings

# Modification History

| Release 3 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 2 | This version first released with CPC Construction, Plumbing and Services Training Package Release 6.4. Correction to Training Package title in the Modification History from CPC Property Services to CPC Construction, Plumbing and Services Training Package. |
| Release 1 | This version first released with CPC Property Services Training Package Release 5.0. Supersedes and is equivalent to CPCCPD3029A Remove graffiti and apply protective coatings. Updated to meet the Standards for Training Packages 2012. |

# Application

This unit of competency specifies the skills and knowledge required to remove graffiti and apply preventative materials to different surfaces to form graffiti-resistant surface systems.

A person who has achieved this unit of competency would be expected to take responsibility for organising and completing these tasks with a high degree of self-direction.

Completion of the general construction induction training program specified by the model Code of Practice for Construction Work is required for any person who is to carry out construction work.  Achievement of CPCWHS1001 Prepare to work safely in the construction industry meets this requirement.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

| CPCCWHS2001 | Apply WHS requirements, policies and procedures in the construction industry |
| --- | --- |

# Unit Sector

Painting and Decorating

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Plan and prepare. | 1.1 | Review work instructions to remove graffiti and apply protective coatings. |
|  |  | 1.2 | Plan all work to meet relevant requirements of work health and safety (WHS), Commonwealth and state or territory legislation, environmental plans and obligations, manufacturers’ specifications, and workplace requirements. |
|  |  | 1.3 | Select and fit personal protection equipment PPE as required for each task. |
|  |  | 1.4 | Inspect work site, inspect and identify existing coating, assess hazards and apply risk controls, including required signage and barricades. |
|  |  | 1.5 | Identify materials appropriate to the work application, calculate quantities and prepare, handle safely and locate ready for use. |
|  |  | 1.6 | Select tools and equipment, check for serviceability and rectify and report any faults. |
| 2 | Prepare site and materials. | 2.1 | Set-up site area for graffiti removal and preventative application processes. |
|  |  | 2.2 | Protect surrounding surfaces with drop sheets, masking or removal of objects. |
|  |  | 2.3 | Check work site to ensure provision of adequate ventilation and implementation of precautions to prevent fire and explosion. |
|  |  | 2.4 | Install storm water protection systems. |
| 3 | Remove graffiti. | 3.1 | Apply selected removal processes to remove graffiti. |
|  |  | 3.2 | Remove graffiti from affected area following manufacturers’ specifications. |
| 4 | Apply anti-graffiti coating. | 4.1 | Clean and prepare substrate surfaces for application of sacrificial and non-sacrificial anti-graffiti coatings. |
|  |  | 4.2 | Prepare and apply selected anti-graffiti coatings following manufacturers’ specifications. |
| 5 | Clean up. | 5.1 | Clean up, meeting all legislative and workplace requirements for safety, waste disposal, materials handling and protection of the environment. |
|  |  | 5.2 | Clear work area, reuse or recycle materials and place waste and unwanted materials into job waste bins or rubbish stockpile. |
|  |  | 5.3 | Check and maintain tools and equipment and clean with appropriate solvents. |
|  |  | 5.4 | Dispose of paint waste and water and solvents used in cleaning tools and equipment in an environmentally sustainable manner. |
|  |  | 5.5 | Store tools, equipment and sealed unused materials to avoid spontaneous ignition. |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

Supersedes and equivalent to CPCCPD3029A Remove graffiti and apply protective coatings.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad