# CPCCRI3014 Perform advanced structural steel erection

Release: 2

CPCCRI3014 Perform advanced structural steel erection

# Modification History

| Release 2 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCRI3014A Perform advanced structural steel erection. Updated to meet the Standards for Training Packages 2012. |

# Application

This unit specifies the skills and knowledge required to erect or install permanent steel structures, and to coordinate the slinging, stability, lifting, moving and placement of loads in conjunction with the crane operator. It includes selection of equipment, and load distribution and calculation.

It applies to workers who erect and install permanent steel structures, and who would be expected to take responsibility for organising and completing these tasks with a high degree of self direction.

Completion of the general construction induction training program, specified in the Safe Work Australia model Code of Practice: Construction Work, is required by anyone carrying out construction work. Achievement of CPCWHS1001 Prepare to work safely in the construction industry meets this requirement.

Licensing, legislative, regulatory or certification requirements apply to this unit of competency in some states and territories. For further information, check with the relevant regulatory authority.

# Pre-requisite Unit

CPCCWHS2001 Apply WHS requirements, policies and procedures in the construction industry

# Unit Sector

Construction

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Plan and prepare to perform advanced structural steel erection. | 1.1 | Interpret work order and associated documentation and clarify requirements with relevant persons. |
|  |  | 1.2 | Plan all work to comply with relevant legislation, regulations, standards, codes, work health and safety, environmental and workplace requirements. |
|  |  | 1.3 | Select plant, tools and equipment required for work tasks, check for serviceability and rectify or report identified faults. |
|  |  | 1.4 | Calculate materials quantities. |
|  |  | 1.5 | Acquire materials for task and locate them safely ready for use. |
| 2 | Select and check equipment. | 2.1 | Erect required safety signage and barricades and fit personal protective equipment (PPE). |
|  |  | 2.2 | Select resources, materials, tools and equipment in accordance with load charts and inspect for compliance with work order. |
|  |  | 2.3 | Inspect lifting equipment in accordance with manufacturer and workplace requirements. |
|  |  | 2.4 | Label, reject and dispose of any lifting and load-shifting equipment identified as inconsistent with manufacturer requirements, to prevent its use in any circumstance. |
|  |  | 2.5 | Select elevated work platforms and other means of mechanical access systems as specified. |
|  |  | 2.6 | Install fall arrest equipment. |
|  |  | 2.7 | Select personnel cartage systems. |
| 3 | Connect equipment. | 3.1 | Calculate and confirm load mass and load distribution. |
|  |  | 3.2 | Sling loads to protect load and prevent damage to slings. |
|  |  | 3.3 | Secure whole or part loads to prevent uncontrolled movement. |
|  |  | 3.4 | Attach slings, or parts of slings, to load and position them to ensure safe movement. |
|  |  | 3.5 | Attach slings, or parts of slings, to hook while hoist wire is vertical. |
|  |  | 3.6 | Attach tag lines to load as specified. |
|  |  | 3.7 | Perform test lifts to ensure safe and secure movement of load as specified. |
| 4 | Move and position loads. | 4.1 | Determine load destination and prepare landing area to accept load. |
|  |  | 4.2 | Assemble lifting or pulling device and erect as specified. |
|  |  | 4.3 | Move load safely to required destination and secure in position in accordance with work order and workplace requirements. |
|  |  | 4.4 | Use standard communication signals to coordinate safe movement of load. |
| 5 | Remove equipment. | 5.1 | Dismantle lifting/moving equipment and packing, return to resting position and inspect for wear. |
|  |  | 5.2 | Complete logbook and site records in accordance with workplace requirements. |
| 6 | Clean up. | 6.1 | Clear work area and dispose of, reuse or recycle materials in accordance with regulatory and workplace requirements. |
|  |  | 6.2 | Clean, check, maintain and store plant, tools and equipment in accordance with manufacturer and workplace requirements. |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

Supersedes and is equivalent to CPCCRI3014A Perform advanced structural steel erection.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad