# CPCCRI3015 Perform advanced tilt-up slab erection

Release: 3

CPCCRI3015 Perform advanced tilt-up slab erection

# Modification History

| Release 3 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 2 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.1. Minor edits to formatting. |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCRI3015A Perform advanced tilt-up slab erection. Updated to meet the Standards for Training Packages 2012. |

# Application

This unit specifies the skills and knowledge required to conduct advanced tilt-up slab erection, and to coordinate the slinging, stability, lifting, moving, placement and storage of slabs in conjunction with the crane operator. It includes selection of equipment, and load distribution and calculation.

It applies to workers who perform advanced tilt-slab erection, and who would be expected to take responsibility for organising and completing these tasks with a high degree of self direction.

Completion of the general construction induction training program, specified in the Safe Work Australia model Code of Practice: Construction Work, is required by anyone carrying out construction work. Achievement of CPCWHS1001 Prepare to work safely in the construction industry meets this requirement.

Licensing, legislative, regulatory or certification requirements apply to this unit of competency in some states and territories. For further information, check with the relevant regulatory authority.

# Pre-requisite Unit

CPCCWHS2001 Apply WHS requirements, policies and procedures in the construction industry

# Unit Sector

Construction

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Plan and prepare to perform advanced tilt-up slab erection. | 1.1 | Interpret work instructions and associated documentation and clarify requirements with relevant persons. |
|  |  | 1.2 | Plan all work to comply with relevant legislation, regulations, standards, codes, work health and safety, environmental and workplace requirements. |
|  |  | 1.3 | Select plant, tools and equipment required for work tasks, check for serviceability and rectify or report identified faults. |
|  |  | 1.4 | Calculate materials quantities. |
|  |  | 1.5 | Acquire materials for task and locate them safely ready for use. |
| 2 | Select and check equipment. | 2.1 | Erect required safety signage and barricades and fit personal protective equipment (PPE). |
|  |  | 2.2 | Select resources, materials and equipment in accordance with load charts and inspect for compliance with work instructions. |
|  |  | 2.3 | Inspect lifting equipment in accordance with manufacturer and workplace requirements. |
|  |  | 2.4 | Label, reject and dispose of any lifting and load-shifting equipment identified as inconsistent with manufacturer requirements, to prevent its use in any circumstance. |
|  |  | 2.5 | Select elevated work platforms and other means of mechanical access systems as specified. |
|  |  | 2.6 | Install fall arrest equipment. |
|  |  | 2.7 | Select personnel cartage systems. |
| 3 | Erect tilt-up slabs. | 3.1 | Review lift plan and determine job sequencing schedule and documentation, and communicate this information to relevant persons to ensure coordination. |
|  |  | 3.2 | Calculate and confirm load mass and load distribution. |
|  |  | 3.3 | Determine erection sequence for tilt-up slab components. |
|  |  | 3.4 | Assemble and erect lifting or pulling devices for movement of loads. |
|  |  | 3.5 | Sling load. |
|  |  | 3.6 | Shift load in accordance with job safety and environmental analysis (JSEA) and safe work method statement (SWMS). |
|  |  | 3.7 | Maintain stability of load during lifting, tilting and transportation. |
|  |  | 3.8 | Direct load to landing position in accordance with engineering specifications. |
| 4 | Position and anchor tilt-up slabs. | 4.1 | Position tilt slab components in accordance with engineering specifications. |
|  |  | 4.2 | Achieve shape of structure by checking dimensions and applying temporary bracing. |
|  |  | 4.3 | Anchor tilt slab components in accordance with manufacturer requirements and engineering specifications. |
| 5 | Remove rigging equipment. | 5.1 | Dismantle lifting/shifting equipment and packing, return to resting position and inspect for wear. |
|  |  | 5.2 | Complete logbook and site records in accordance with workplace requirements. |
| 6 | Clean up. | 6.1 | Clear work area and dispose of, reuse or recycle materials in accordance with regulatory and workplace requirements. |
|  |  | 6.2 | Clean, check, maintain and store tools and equipment in accordance with manufacturer and workplace requirements. |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

Supersedes and is equivalent to CPCCRI3015A Perform advanced tilt-up slab erection.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad

Assessment Requirements for CPCCRI3015 Perform advanced tilt-up slab erection

# Modification History

| Release 3 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 2 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.1. Minor edits to formatting. |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCRI3015A Perform advanced tilt-up slab erection. Updated to meet the Standards for Training Packages 2012. |

# Performance Evidence

To demonstrate competency in this unit, a candidate must meet the elements and performance criteria by performing advanced tilt-up slab erection to erect:

one multi-point precast tilt-up slab of at least 10 tonnes

one cast in situ tilt-up slab of at least 10 tonnes.

Each task must be performed using correct rigging, slinging and de-rigging techniques, including:

determining lifting points, temporary supports and permanent fixing points from panel erection drawings by following the lift plan

positioning and bracing the slab

safe unloading from the truck

rotating panel from one plane to another

incorporating at least two bracing techniques.

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

workplace and regulatory requirements for performing advanced tilt-up slab erection under applicable Commonwealth, state or territory work health and safety (WHS) legislation and codes of practice:

job safety and environmental analyses (JSEAs)

safe work method statements (SWMSs)

safety manuals and instructions for plant, tools and equipment

signage and barricades

personal protective equipment (PPE)

working at heights requirements

environmental and worksite safety plans

reporting problems

scope of work for performing advanced tilt-up slab erection:

calculations:

load charts

fleet angles

diverter sheaves

lead loads

head loads

load angle factors

multiple fall

load share

precast compliance charts

safe working loads

load slinging methods:

straight sling

adjustable sling

reeved sling

inclined sling

advanced lifting techniques:

rotation (turning panel from one plane to another)

standard lift from a truck

lift from flat on ground

tilt-up slabs:

prefabricated and cast in situ tilt-up slabs

slabs with cast-in lifting and/or bracing inserts

bracing of tilt-up slabs:

lateral bracing

end bracing

main bracing

knee bracing

deadman bracing

types, characteristics and limitations of tools, equipment and materials required for performing advanced tilt-up slab erection:

chain slings

flexible steel wire rope (FSWR) slings

air winches

chain blocks

chain winches

come-alongs

drifts

eye bolts

hammers

hand operated creeper winches

jacks

podgers

pulley blocks

shackles

shifting spanners

sledge hammers

spanners

spirit levels and automatic levels

tape measures

trolleys

turn buckles

wedges

wrenches

requirements for use of tools, equipment and materials for performing advanced tilt-up slab erection:

safe operating procedures

operational, maintenance and basic diagnostic procedures

workplace and equipment safety requirements

signalling methods and communications

processes for completing logbooks and site records

requirements for cleaning up work area, maintaining and storing plant, tools, equipment and materials, and for environmentally friendly waste management.

# Assessment Conditions

Assessors must meet the requirements for assessors outlined in the Standards for Registered Training Organisations.

Assessment must be conducted in the workplace or a simulated workplace using realistic conditions, materials, activities, responsibilities, procedures, safety requirements and environmental considerations.

Candidates must have access to:

industry-standard equipment used to perform the tasks specified in the Performance Evidence

appropriate workplace information and records, including policies, procedures and legislative requirements.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad