# CPCCSG3013 Paint letters and decorative effects for signs

Release: 1

CPCCSG3013 Paint letters and decorative effects for signs

# Modification History

Release 1.

This version first released with CPC Construction, Plumbing and Services Training Package Version 2.

Revised unit of competency. Replaces superseded equivalent CPCCSI2006A Signwrite to simple forms and CPCCSI3003A Signwrite to decorative forms.

# Application

This unit of competency specifies the outcomes required to manually paint signs on surfaces, comprised of various letter types and graphics. It covers setting out the sign to scale from a smaller design template using different layout methods; selecting and using tools and equipment to paint letters and graphics; and using brush strokes, techniques and decorative effects to achieve the desired result.

The unit supports sign manufacturers who produce customised signs that are painted directly on to surfaces for a range of purposes, including advertising boards and retail signs.

No licensing, legislative, regulatory or certification requirements apply to this unit of competency at the time of endorsement.

# Pre-requisite Unit

Nil

# Competency Field

Signs and graphics

# Unit Sector

Construction

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. Where bold italicised text is used, further information is detailed in the range of conditions. |  |
| --- | --- | --- | --- |
| 1 | Plan and prepare work. | 1.1 | Job requirements are identified from drawings, specifications, instructions and client briefs, confirmed, and applied to planning and preparation. |
|  |  | 1.2 | Work site is assessed and scaffolding requirements are identified and installed according to statutory and licensing requirements. |
|  |  | 1.3 | Health and safety requirements are identified and applied to planning. |
|  |  | 1.4 | Sequence of tasks is determined and confirmed with relevant personnel. |
|  |  | 1.5 | Tools and equipment are selected to carry out tasks according to job requirements, checked for serviceability, and faults are rectified or reported before starting work. |
|  |  | 1.6 | Colour selection is determined, consistent with job requirements. |

| 2 | Lay out sign. | 2.1 | Size and dimensions of sign to be painted are calculated according to sign design and job requirements. |
| --- | --- | --- | --- |
|  |  | 2.2 | Layout method is selected according to size and scale of the work and is implemented. |

| 3 | Produce lettering and graphics. | 3.1 | Surface is prepared according to job specifications. |
| --- | --- | --- | --- |
|  |  | 3.2 | Letters are painted on to surface using tools, equipment and sign-writing techniques according to job requirements. |
|  |  | 3.3 | Letter spacing is consistently applied according to job requirements. |
|  |  | 3.4 | Errors in paint application are identified and rectified through the sign-writing process. |
|  |  | 3.5 | Decorative effects are applied to letters according to job requirements. |
|  |  | 3.6 | Graphics are painted on to surface using tools, equipment and brush techniques according to job requirements. |

| 4 | Clean up. | 4.1 | Finished sign and surrounding surface environment are cleaned and waste materials removed according to statutory and regulatory authority requirements. |
| --- | --- | --- | --- |
|  |  | 4.2 | Tools and equipment, including personal protective equipment (PPE), are cleaned, maintained and stored according to workplace requirements. |

# Foundation Skills

| This section describes the language, literacy, numeracy and employment skills essential to performance in this unit but not explicit in the performance criteria. |  |
| --- | --- |
| Skill | Performance feature |
| Learning skills to: | transfer key principles and techniques of painting decorative effects when working with different sign designs. |
| Numeracy skills to: | measure surface and perform calculations to accurately identify required surface area for design calculate scale and dimensions of final sign from drawings and apply to layout processes for sign-writing. |
| Oral communication skills to: | enable clear and direct communication, using questioning to identify and confirm requirements, share information, listen and understand. |
| Reading skills to: | interpret a range of familiar workplace documentation, including hand drawings, manufacturer specifications and instructions. |
| Writing skills to: | report faults in tools and equipment to manufacturers and other relevant personnel. |

# Range of Conditions

| This section specifies work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. Bold italicised wording, if used in the performance criteria, is detailed below. |  |
| --- | --- |
| Health and safety requirements must include: | selection, correct fitting, and use of PPE prescribed under legislation, regulations and workplace policies and practices emergency procedures, including: evacuation first aid location and use of firefighting equipment hazard identification and risk control procedures safe operating procedures, including the conduct of operational risk assessment and treatments associated with: traffic control work site visitors and the public working at heights working in confined spaces working in proximity to others use of machines use of tools and equipment workplace environmental requirements. |
| Tools and equipment include: | cutting knives mahl sticks oils appropriate to the application paint pots sign-writing pencils, pens and brushes stirring sticks tape measures and rulers tapes. |
| Decorative effects must include: | three dimensional (3-D) colour effects shading. |

# Unit Mapping Information

### CPCCSI2006A Signwrite to simple forms

CPCCSI3003A Signwrite to decorative forms

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad