# CPCCSG3015 Produce airbrushed signs

Release: 1

CPCCSG3015 Produce airbrushed signs

# Modification History

Release 1.

This version first released with CPC Construction, Plumbing and Services Training Package Version 2.

Revised unit of competency. Replaces superseded equivalent CPCCSI3015A Produce airbrushed signage.

# Application

This unit of competency specifies the outcomes required to select and apply air brushes, inks, paints and application techniques to produce various forms of airbrushed signs.

The unit supports sign manufacturers who use traditional painting and airbrushing techniques to achieve a visual style of signs.

No licensing, legislative, regulatory or certification requirements apply to this unit of competency at the time of endorsement.

# Pre-requisite Unit

Nil

# Competency Field

Signs and graphics

# Unit Sector

Construction

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. Where bold italicised text is used, further information is detailed in the range of conditions. |  |
| --- | --- | --- | --- |
| 1 | Plan and prepare work. | 1.1 | Job requirements are identified from drawings, specifications, instructions and client briefs, and confirmed. |
|  |  | 1.2 | Sequence of tasks is determined and confirmed with relevant personnel. |
|  |  | 1.3 | Colour is selected according to job requirements. |
|  |  | 1.4 | Tools and equipment are selected to carry out tasks according to job requirements, checked for serviceability, and faults are rectified or reported before starting work. |
|  |  | 1.5 | Surfaces are identified and prepared according to planned airbrush application. |
|  |  | 1.6 | Health and safety requirements are identified and planned according to schedule of tasks. |

| 2 | Apply masks. | 2.1 | Adhesive masks are prepared and used according to job requirements. |
| --- | --- | --- | --- |
|  |  | 2.2 | Hand-held templates are prepared and applied according to job requirements. |

| 3 | Apply inks or paints using airbrush. | 3.1 | Airbrush inks or paints are applied to surface using application techniques consistent with job requirements. |
| --- | --- | --- | --- |
|  |  | 3.2 | Light and shade effects are applied using techniques suitable to the selected medium. |
|  |  | 3.3 | Defects in application are identified and corrective action is taken. |
|  |  | 3.4 | Airbrush work is finalised according to job specifications. |

| 4 | Clean up and store equipment. | 4.1 | Airbrush and associated components and accessories are dismantled, checked, cleaned and maintained according to manufacturer recommendations and environmental requirements. |
| --- | --- | --- | --- |
|  |  | 4.2 | Airbrush inks, paints and solvents are stored safely according to manufacturer specifications and workplace requirements. |
|  |  | 4.3 | Work area is cleaned and waste materials are removed according to statutory and regulatory authority requirements. |
|  |  | 4.4 | Other tools and equipment, including personal protective equipment (PPE) are cleaned, maintained and stored according to workplace requirements. |

# Foundation Skills

| This section describes the language, literacy, numeracy and employment skills essential to performance in this unit but not explicit in the performance criteria. |  |
| --- | --- |
| Skill | Performance feature |
| Learning skills to: | transfer key principles and techniques of airbrushing to new sign designs. |
| Numeracy skills to: | calculate and apply scale to enlargements of sign designs. |
| Oral communication skills to: | communicate clearly with client and others using verbal and non-verbal communication. |
| Reading skills to: | interpret a range of familiar workplace documentation, including drawings, specifications and instructions from clients and others. |
| Writing skills to: | produce legible lettering using ink or paint and brushes. |

# Range of Conditions

| This section specifies work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. Bold italicised wording, if used in the performance criteria, is detailed below. |  |
| --- | --- |
| Health and safety requirements must include: | selection, correct fitting, and use of PPE prescribed under legislation, regulations and workplace policies and practices emergency procedures, including: evacuation first aid location and use of fire-fighting equipment hazard identification and risk control procedures safe operating procedures, including the conduct of operational risk assessment and treatments associated with: lighting restricted access barriers use of safety data sheets (SDS) use of tools and equipment work site visitors and the public working in proximity to others workplace environmental requirements. |

# Unit Mapping Information

CPCCSI3015A Produce airbrushed signage

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad

Assessment Requirements for CPCCSG3015 Produce airbrushed signs

# Modification History

Release 1.

This version first released with CPC Construction, Plumbing and Services Training Package Version 2.

Revised unit of competency. Replaces superseded equivalent CPCCSI3015A Produce airbrushed signage.

# Performance Evidence

A person demonstrating competency in this unit must satisfy all of the elements, performance criteria and foundation skills of this unit, and must produce two airbrushed signs as follows:

one head and shoulders portrait of a person, measuring at least 600 x 900 mm, using at least six different colours, and demonstrating the use of texture, gradient and line work

one graphic produced on a motor vehicle bonnet, including the use of at least six different colours, and demonstrating the use of adhesive masks to accurately produce edge definition.

In producing the above work, the person must:

interpret specifications for preparing and applying various airbrush techniques

select, set up and check airbrush equipment and accessories prior to use

identify defects associated with airbrush application and take corrective action

maintain, clean, dismantle and reassemble airbrush, components and accessories

prepare and apply a variety of masks to use in conjunction with airbrush techniques

apply airbrush techniques using a variety of freehand and masked techniques

comply with relevant safety and environmental legislation, regulations and codes of practice applicable to workplace operations.

# Knowledge Evidence

A person demonstrating competency in this unit must demonstrate knowledge of:

airbrushing terminology and use of abbreviations

Australian standards applicable to airbrushing sign work, including AS 1319 Safety signs for the occupational environment

characteristics and applications of different airbrushes used to airbrush signs, including:

single action

double action

internal mix

external mix

characteristics and applications of different paint types used to airbrush signs, including:

oil-based

water-based

principles and techniques of producing airbrushed signs, including application of:

freehand techniques

geometric shapes

graphics

lettering

purpose and application of safety data sheets (SDS) when producing airbrushed signs

statutory and regulatory authority requirements relevant to airbrushing sign work, including those relating to removal of waste products

techniques for producing airbrushed signs through the choice of relevant materials

types of airbrushes, components and accessories used in airbrushed sign work

work health and safety (WHS) requirements and safe work methods applicable to airbrushing.

# Assessment Conditions

Suitable assessment of performance requires:

equipment:

air hose

air brushes, components and accessories

buckets

compressed air and leads

masks

measuring tapes and rules

paint pots

stencil knives

straight edges

materials:

inks

masking materials

motor vehicle bonnet

paints

specifications:

AS 1319 Safety signs for the occupational environment

SDS

timeframe:

according to job requirements.

#### Assessor requirements

As a minimum, assessors must satisfy the assessor requirements in the Standards for Registered Training Organisations (RTOs) current at the time of assessment.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad