# CPCCSH3005 Apply and trim decorative additions to shopfittings and components

Release: 1

CPCCSH3005 Apply and trim decorative additions to shopfittings and components

# Modification History

Release 1.

This version first released with CPC Construction, Plumbing and Services Training Package Version 2.

Revised unit of competency. Replaces superseded equivalent CPCCSH3005A Apply and trim decorative finishes.

# Application

This unit of competency specifies the outcomes required to apply and trim decorative panelling, frames, borders and mirrors to shopfittings and components according to specified designs. It covers setting out the position of decorative additions, fixing them into place, and trimming them to ensure a quality finish.

The unit supports shopfitters who may work off site on units being fabricated for installation, or on site on installed units and internal walls, working concurrently with other shopfitting work. Work must be scheduled to accommodate both client requirements, where the shop is operational, and the requirements of other shopfitters and associated contractors.

No licensing, legislative, regulatory or certification requirements apply to this unit of competency at the time of endorsement.

# Pre-requisite Unit

Nil

# Competency Field

Shopfitting

# Unit Sector

Construction

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. Where bold italicised text is used, further information is detailed in the range of conditions. |  |
| --- | --- | --- | --- |
| 1 | Plan and prepare. | 1.1 | Job requirements are obtained, confirmed and applied to planning. |
|  |  | 1.2 | Work site is inspected, conditions and hazards are identified and reported as required, and health and safety requirements are confirmed with relevant personnel and applied to planning. |
|  |  | 1.3 | Type and quantity of decorative additions to be applied are confirmed, obtained, prepared, safely handled and located ready for use. |
|  |  | 1.4 | Substrate is confirmed and method for fixing decorative additions is identified. |
|  |  | 1.5 | Tools and equipment are selected to carry out tasks according to job requirements, checked for serviceability, and faults are rectified or reported before starting work. |
|  |  | 1.6 | Position of decorative additions is set out according to specifications and confirmed with client. |

| 2 | Set out and cut decorative additions. | 2.1 | Lengths of decorative additions are set out and marked for locations according to job specifications. |
| --- | --- | --- | --- |
|  |  | 2.2 | Plan is drawn to develop a true mitre for a compound angle or for a mitre of material of different widths, and angles are calculated by dissection. |
|  |  | 2.3 | Decorative additions including curved work are cut to required size according to manufacturer specifications. |

| 3 | Carry out fixing processes and finishing techniques. | 3.1 | Preparation requirements for fixing of additions are carried out according to job requirements. |
| --- | --- | --- | --- |
|  |  | 3.2 | Decorative additions are fitted and fixed according to job requirements. |
|  |  | 3.3 | Applied additions are trimmed and finished according to job requirements. |

| 4 | Clean up. | 4.1 | Work area is cleared and waste material disposed of according to work site requirements. |
| --- | --- | --- | --- |
|  |  | 4.2 | Tools and equipment are cleaned, maintained and stored according to workplace requirements. |

# Foundation Skills

| This section describes the language, literacy, numeracy and employment skills essential to performance in this unit but not explicit in the performance criteria. |  |
| --- | --- |
| Skill | Performance feature |
| Learning skills to: | apply previous experience of different substrates in shops and types of decorative additions when selecting and using suitable tools, equipment and fixing methods. |
| Numeracy skills to: | calculate material requirements and determine efficient cutting strategies to minimise waste apply geometry to develop a true raking mould shape or mitre. |
| Oral communication skills to: | use questioning to identify and confirm job requirements interact with workplace personnel and contribute to safe and efficient application of decorative additions. |
| Reading skills to: | read and interpret a range of workplace documentation, including job safety analyses (JSA) or safe work method statements (SWMS), drawings and specifications for the placement of decorative additions. |
| Writing skills to: | complete workplace documentation, including equipment fault reports. |
| Initiative and enterprise skills to: | respond to workplace challenges, including short timeframes and material delivery delays, and put ideas into action to overcome issues and meet project schedules. |

# Range of Conditions

| This section specifies work environments and conditions that may affect performance. Essential operating conditions that may be present (depending on the work situation, needs of the candidate, accessibility of the item, and local industry and regional contexts) are included. Bold italicised wording, if used in the performance criteria, is detailed below. |  |
| --- | --- |
| Job requirements must include: | job specifications for applying, trimming and finishing decorative additions, including: details of the additions to be applied and application methods to be used, including drawings and specifications quality requirements for the completed work timeframe for completion information regarding other personnel undertaking concurrent shopfitting activities at the work site and arrangements for interaction during completion of work work site procedures specified by retail project manager or client JSA or SWMS for tasks schedule for completing work, including client requirements regarding times when work can be undertaken workplace procedures incorporating requirements of relevant legislation, regulations, codes and Australian standards workplace and work site environmental and sustainability requirements, including: clean-up management dust and noise management waste management. |
| Health and safety requirements must include: | selection, correct fitting, and use of personal protective equipment (PPE) prescribed under legislation, regulations and workplace policies and practices emergency procedures, including: location and use of firefighting equipment first aid procedures evacuation procedures hazard identification and risk control procedures safe operating procedures, including: safety data sheets (SDS) manufacturer specifications signage and barricades requirements for: lighting work site visitors and the public working at heights working in proximity to others working platforms and scaffolding selection and use of tools and equipment identifying, safe handling, securing and labelling of hazardous materials. |
| Decorative additions must include: | decorative panelling frame or border using extruded sections or mouldings mirrors. |
| Fixing materials must include: | adhesives, including double sided adhesive tape brackets dowel joints nails plastic lock concealed fixings screws. |
| Tools and equipment must include: | air compressor and hoses bevels chisels clamps hammers hand saws measuring tapes and rules nail guns power drills power leads saw stools screwdrivers spirit levels squares. |

# Unit Mapping Information

CPCCSH3005A Apply and trim decorative finishes

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad