# Assessment Requirements for CPCCSH3010 Prepare shop floors for new coverings

Release: 1

Assessment Requirements for CPCCSH3010 Prepare shop floors for new coverings

# Modification History

Release 1.

This version first released with CPC Construction, Plumbing and Services Training Package Version 2.

New unit of competency.

# Performance Evidence

A person demonstrating competency in this unit must satisfy all of the elements, performance criteria and foundation skills of this unit, and must remove four of the floor coverings listed in the range of conditions and:

clear and clean the exposed surface

test and apply primers and cementitious compounds to patch and smooth the surface ready for the application of new shop floor coverings.

A minimum of 10 square metres of floor covering is to be cleared, cleaned and primed for each of the above four floor coverings selected.

In doing the above work, the person must:

interpret and confirm job requirements

apply safe handling requirements for equipment, products and materials, including use of personal protective equipment

follow work instructions, operating procedures and inspection processes to:

minimise the risk of injury to self or others

prevent damage to goods, equipment and products

maintain required production output and product quality

complete measurements, calculate area, and estimate material requirements

interpret scales and symbols on simple floor plans

correctly use equipment, processes and procedures to remove floor coverings

follow safety procedures relevant to preparing floors for new coverings

work with others and in a team by recognising dependencies and using cooperative approaches to optimise work flow and productivity.

# Knowledge Evidence

A person demonstrating competency in this unit must demonstrate knowledge of:

state or territory work health and safety (WHS) legislation, regulations, standards and codes of practice relevant to removing floor coverings and floor smoothing techniques, including:

AS/NZS 2455.1 Textile floor coverings – Installation practice – General

AS 1884 Floor coverings – Resilient sheet and tiles – Installation practices

organisational and site standards, requirements, policies and procedures for removing existing floor coverings and using floor smoothing techniques

environmental protection requirements relevant to preparing floors for new coverings

established communication channels and protocols to follow during above work

relevant problem identification and resolution to apply during above work

types of tools and equipment and procedures for their safe use, operation and maintenance

characteristics of materials used for floors, and products used for preparing floor surfaces

set-up and operation of equipment used when preparing floors for new coverings

procedures for recording, reporting and maintaining workplace records and information

characteristics of, and techniques for fixing, floor coverings

workplace safety system requirements relating to removing floor coverings and applying floor smoothing compounds.

# Assessment Conditions

Suitable assessment of performance requires:

equipment and tools:

as listed in the range of conditions

physical conditions:

retail space with floor area to be prepared for new floor covering

relationships with team members and supervisor:

work may be completed as part of a team, under supervision

relationship with client:

consultation with the client is required

specifications:

AS/NZS 2455.1 Textile floor coverings – Installation practice – General

AS 1884 Floor coverings – Resilient sheet and tiles – Installation practices

as listed in job requirements in the range of conditions

manufacturer specifications

job safety analyses (JSA)

safe work method statements (SWMS)

safety data sheets (SDS)

timeframe:

according to job requirements.

### Assessor requirements

As a minimum, assessors must satisfy the assessor requirements in the Standards for Registered Training Organisations (RTOs) current at the time of assessment.

# Links

Companion Volume implementation guides are found in VETNet - https://vetnet.education.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad