# CPCCST2003 Finish stone

Release: 2

CPCCST2003 Finish stone

# Modification History

| Release 2 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCST2003A Finish stone. Updated to meet the Standards for Training Packages 2012. |

# Application

This unit specifies the skills and knowledge required to cut and finish both hard and soft stone. It includes determining stone hardness prior to cutting and finishing stone.

The unit is suitable for people with basic skills and knowledge who undertake routine stonemasonry work tasks under the direction of more experienced workers.

Completion of the general construction induction training program specified by the model Code of Practice for Construction Work is required for any person who is to carry out construction work. Achievement of CPCWHS1001 Prepare to work safely in the construction industry meets this requirement.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

CPCCWHS2001 Apply WHS requirements, policies and procedures in the construction industry

# Unit Sector

Stonemasonry

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Plan and prepare. | 1.1 | Read, interpret and apply finish stone requirements from current plans, specifications, standards and codes and confirm information with relevant person. |
|  |  | 1.2 | Follow requirements in workplace safety and environmental documentation and workplace procedures. |
|  |  | 1.3 | Clarify job priorities and sequence job tasks in consultation with other workers. |
|  |  | 1.4 | Identify potential hazards and implement control measures. |
|  |  | 1.5 | Select and check condition of plant, equipment and tools and report damage or faults to relevant person. |
|  |  | 1.6 | Check conformity of materials against the job requirements and place stone ready for the work. |
|  |  | 1.7 | Identify stone hardness using Mohs scale of mineral hardness. |
| 2 | Cut stone. | 2.1 | Select, fit correctly and use personal protective equipment (PPE) appropriate for job. |
|  |  | 2.2 | Mark stone and position onto cutting table to obtain maximum efficiency in cutting. |
|  |  | 2.3 | Align stone to cutting blade and secure in position to ensure stability. |
|  |  | 2.4 | Adjust machinery to maximum depth of cut for size of stone. |
|  |  | 2.5 | Apply coolant as required for stone and job requirements. |
|  |  | 2.6 | Use primary and secondary cutting machines appropriate to their cutting capacity. |
| 3 | Polish stone. | 3.1 | Prepare stone with adhesives or fillers. |
|  |  | 3.2 | Set up and operate polishing heads. |
|  |  | 3.3 | Polish stone using various hand-operated and static polishing machines. |
|  |  | 3.4 | Apply adhesives, tinting for fillers and reinforcing materials to job requirements. |
|  |  | 3.5 | Clean stone face surfaces and apply finishes. |
| 4 | Clean up. | 4.1 | Clear the work area, and dispose of, reuse or recycle materials following workplace and environmental requirements. |
|  |  | 4.2 | Clean tools and equipment, check for serviceability and report damage or faults. |
|  |  | 4.3 | Store and secure tools and equipment following workplace procedures. |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

Supersedes and is equivalent to CPCCST2003A Finish stone

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad

Assessment Requirements for CPCCST2003 Finish stone

# Modification History

| Release 2 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCST2003A Finish stone. Updated to meet the Standards for Training Packages 2012. |

# Performance Evidence

To demonstrate competency in this unit, a candidate must meet the performance criteria of this unit by cutting and polishing one hard stone and one soft stone with a minimum size of 190 mm x 190 mm x 190 mm.

In doing this, the candidate must:

use two types of cutting equipment

use two types of polishing equipment

use two types of abrasives and polishing compounds.

# Knowledge Evidence

To be competent in this unit, a person must demonstrate knowledge of:

characteristics, applications and limitations of specified materials and products for cutting and finishing stone:

soft and hard stone

coolants used in the polishing process

range and grades of polishing compounds

adhesives and fillers

processes and techniques for:

material protection and storage

cutting stone

primary and secondary cutting machines

correct sequencing of various polishing heads

functional and operational features of equipment and hand and power tools:

angle grinders

buffs

circular saws

drills

edge polishers

frame saws

hole saws

Jenny Lind type polishers

line polishers

milling machines

rise and fall saws

rotating or transverse tables

stationary slab polishers

twin tables

wire saws

workplace processes and procedures for stonemasonry

workplace safety:

job safety and environment analysis (JSEA)

exposure to silica dust

exposure to loud noise

hazardous manual tasks

hazardous goods

vibration

chemical handling and storing

key features of plans and specifications

stonemasonry regulations, standards and codes

Mohs scale of mineral hardness

environmental requirements for workplace processes and waste disposal.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

This unit must be assessed in the workplace or a close simulation reflecting workplace conditions and standards, materials, equipment, activities, responsibilities, procedures, safety requirements and environmental considerations.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad