# CPCCST3014 Set and anchor stone facades

Release: 2

CPCCST3014 Set and anchor stone facades

# Modification History

| Release 2 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCST3014A Set and anchor stone facades. Updated to meet the Standards for Training Packages 2012. |

# Application

This unit specifies the skills and knowledge required to shift, set and anchor a variety of stone facades for construction projects.

This unit applies to stonemasons who work independently or in a team to set and anchor stone facades relating to stonemasonry work on a new or existing structure.

A person working at this level would be expected to take responsibility for organising and completing tasks assigned to them without needing close supervision.

Completion of the general construction induction training program specified by the model Code of Practice for Construction Work is required for any person who is to carry out construction work. Achievement of CPCWHS1001 Prepare to work safely in the construction industry meets this requirement.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

CPCCWHS2001 Apply WHS requirements, policies and procedures in the construction industry

# Unit Sector

Stonemasonry

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Plan the work. | 1.1 | Read, interpret and apply fixing stone facade requirements from current plans, specifications, standards and codes. |
|  |  | 1.2 | Follow relevant information from workplace procedures, workplace safety and environmental documentation. |
|  |  | 1.3 | Use measurements and apply formulas to calculate material quantities for the task. |
|  |  | 1.4 | Determine job priorities and sequence job tasks in consultation with others on site. |
|  |  | 1.5 | Identify potential hazards and determine and implement control measures. |
|  |  | 1.6 | Select and check condition of plant, equipment and tools and report damage or malfunction to relevant person. |
| 2 | Prepare for work. | 2.1 | Select, fit correctly and use personal protective equipment (PPE) appropriate for job. |
|  |  | 2.2 | Check conformity of materials against the specifications. |
|  |  | 2.3 | Set out stone facade dimensions, line and level from datum and establish base course. |
|  |  | 2.4 | Prepare area of construction to receive stone components. |
|  |  | 2.5 | Prepare surface and install base dowels to provide key for base course. |
|  |  | 2.6 | Check stone for faults; stone components are prepared, lifting locations are undamaged, and lifting gear is attached. |
| 3 | Shift, erect and fix stone. | 3.1 | Erect scaffolding as required in accordance with workplace safety requirements. |
|  |  | 3.2 | Select slings, clutches and lifting equipment for their correct function and inspect for wear and tear or damage. |
|  |  | 3.3 | Shift and transfer stone components to fixing location at structure. |
|  |  | 3.4 | Prepare base components by drilling holes for dowels and placing wedges or packers for adjustment. |
|  |  | 3.5 | Place and fix base course components; align and tie to stone components, and adjust and secure. |
|  |  | 3.6 | Install, join and fix stone facades and corners using appropriate fixing methods. |
|  |  | 3.7 | Flash, seal and caulk stone facade using recommended products. |
|  |  | 3.8 | Clean joins and stone face. |
| 4 | Clean up. | 4.1 | Clear the work area, and dispose of, reuse or recycle following workplace and environmental requirements. |
|  |  | 4.2 | Clean tools and equipment, check for serviceability and report damage or faults. |
|  |  | 4.3 | Store and secure tools and equipment following workplace procedures. |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

Supersedes and is equivalent to CPCCST3014A Set and anchor stone facades

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad

Assessment Requirements for CPCCST3014 Set and anchor stone facades

# Modification History

| Release 2 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCST3014A Set and anchor stone facades. Updated to meet the Standards for Training Packages 2012. |

# Performance Evidence

To demonstrate competency in this unit, a candidate must meet the performance criteria of this unit by setting and anchoring stone facades to a steel frame or concrete construction with minimum height of 1.5 m.

In doing this, the candidate must:

prepare the base tie down system and set and anchor stone facades to specifications

set stone facades around an opening

fix stone facades into an internal or external corner.

# Knowledge Evidence

To be competent in this unit, a person must demonstrate knowledge of:

characteristics, applications and limitations of specified materials and components used in anchoring stone facades

processes and techniques for fixing facade stone onto:

brick/concrete masonry walls

in situ, reinforced concrete

pre-cast reinforced concrete

structural steel framing

methods of fixing stone facades:

S hook

back clamp

corbel plate bracket

dog cramp

fish tailed cramp

pin bracket

slotted bracket

turned end cramp

construction terminology relating to anchor stone facades

masonry expansion and growth, control and articulation joints

types and safe use of lifting equipment

functional and operational features of plant, equipment and hand and power tools:

lifting gear and equipment

air compressor and hoses

power grinders

impact drills

battery operated tools

workplace processes and procedures

workplace safety:

exposure to silica

job safety and environmental analysis (JSEA)

hazardous manual tasks

working at heights

exposure to excessive noise

hazardous substances

features of working drawings and specifications

environmental requirements for workplace processes and waste disposal

digital technology to source information and communicate with others

fire control and separation requirements

key requirements of National Construction Code (NCC) relevant to stone facades.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

This unit must be assessed in the workplace or a close simulation reflecting workplace conditions and standards, materials, equipment, activities, responsibilities, procedures, safety requirements and environmental considerations.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad