# Assessment Requirements for CPCCST3016 Build solid stonemasonry walls

Release: 2

Assessment Requirements for CPCCST3016 Build solid stonemasonry walls

# Modification History

| Release 2 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCST3016A Build solid stonemasonry walls. Updated to meet the Standards for Training Packages 2012. |

# Performance Evidence

To demonstrate competency in this unit, a candidate must meet the performance criteria of this unit by building a minimum 2.1 m long and 2.0 m high solid masonry wall with a minimum of one external or internal corner.

In doing this, the candidate must:

set out gauge and bond, window and door opening, flashing, weepholes and ventilation

lay stones level, straight and plumb at corners

install lintels for opening maintaining gauge and bond.

# Knowledge Evidence

To be competent in this unit, a person must demonstrate knowledge of:

characteristics, applications and limitations of materials for building solid masonry walls

types of natural and manufactured stone used in wall construction:

limestone

sandstone

artificial stone

autoclaved aerated concrete (AAC)

processes and techniques for building stonemasonry walls:

subfloor, wall and roof construction

damp proofing, flashing and ventilation

tying components

lintels and load bearing components

gable and eaves construction

construction terminology relating to laying stone

mortar ingredients and consistency

stone bond patterns

types and safe use of lifting equipment

functional and operational features of plant, equipment and hand and power tools:

cement mixer

elevated work platforms (EWP)

power drills

air compressor and hoses

battery operated tools

workplace processes and procedures

workplace safety:

exposure to silica

job safety and environmental analysis (JSEA)

hazardous manual tasks

working at heights

exposure to loud noise

working drawings and specifications

material protection and storage

environmental requirements for workplace processes and waste disposal

sustainability principles

construction standards, codes and regulations

key requirements of the National Construction Code (NCC) relevant to solid masonry walls

digital technology to source information and communicate with others.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

This unit must be assessed in the workplace or a close simulation reflecting workplace conditions and standards, materials, equipment, activities, responsibilities, procedures, safety requirements and environmental considerations.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad