# CPCCST3019 Lay stonemasonry stairs

Release: 2

CPCCST3019 Lay stonemasonry stairs

# Modification History

| Release 2 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCST3019A Lay stonemasonry stairs. Updated to meet the Standards for Training Packages 2012. |

# Application

This unit specifies the skills and knowledge required to lay internal or external masonry stairs. It includes calculating total rise and going of flight and riser and tread sizes.

This unit applies to stonemasons who work independently or in a team to lay stonemasonry stairs on a new or existing structure.

A person working at this level would be expected to take responsibility for organising and completing tasks assigned to them without needing close supervision.

Completion of the general construction induction training program specified by the model Code of Practice for Construction Work is required for any person who is to carry out construction work. Achievement of CPCWHS1001 Prepare to work safely in the construction industry meets this requirement.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

CPCCWHS2001 Apply WHS requirements, policies and procedures in the construction industry

# Unit Sector

Stonemasonry

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Plan and prepare. | 1.1 | Read, interpret and apply stonemasonry stair construction requirements from current plans, specifications and codes. |
|  |  | 1.2 | Follow relevant information from workplace procedures, workplace safety and environmental documentation. |
|  |  | 1.3 | Use measurements and formulas to calculate material quantities. |
|  |  | 1.4 | Determine job priorities and sequence job tasks in consultation with others on site. |
|  |  | 1.5 | Identify potential hazards and determine and implement control measures. |
|  |  | 1.6 | Select and check condition of plant, equipment and tools and report damage or malfunction to relevant person. |
| 2 | Set out stairs. | 2.1 | Select, fit correctly and use personal protective equipment (PPE) appropriate for job. |
|  |  | 2.2 | Set out location of stairs on a prepared footing to comply with plans and specifications. |
|  |  | 2.3 | Calculate and set out the total rise and going of the flight of stairs. |
|  |  | 2.4 | Calculate and set out minimum tread width and maximum riser height. |
|  |  | 2.5 | Select and check colour, shape and conformity of stone against the specifications and place materials in preparation for the work. |
| 3 | Lay stonemasonry stairs. | 3.1 | Mix mortar to colour, texture and consistency for job requirements and place on mortar board ready for use. |
|  |  | 3.2 | Determine starting point and lay base stones level, square and aligned to set out marks. |
|  |  | 3.3 | Lay risers and treads maintaining level consistent joints. |
|  |  | 3.4 | Form steps ensuring thy are square, level, plumb and true. |
|  |  | 3.5 | Finish joints to specified profile. |
|  |  | 3.6 | Remove excess mortar and brush down face of stairs prior to mortar drying. |
| 4 | Clean up. | 4.1 | Clear the work area, and dispose of, reuse or recycle materials following workplace and environmental requirements. |
|  |  | 4.2 | Clean tools and equipment, check for serviceability and report damage or faults. |
|  |  | 4.3 | Store and secure tools and equipment following workplace procedures. |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

Supersedes and is equivalent to CPCCST3019A Lay stonemasonry stairs

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad

Assessment Requirements for CPCCST3019 Lay stonemasonry stairs

# Modification History

| Release 2 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCCST3019A Lay stonemasonry stairs. Updated to meet the Standards for Training Packages 2012. |

# Performance Evidence

To demonstrate competency in this unit, a candidate must meet the performance criteria of this unit by constructing a single flight of stonemasonry stairs.

In doing this, the candidate must:

build in three treads with a fall to run off water

build in one wing wall maintaining gauge and bond

lay stonemasonry stairs within tolerances stated in relevant standards and codes.

# Knowledge Evidence

To be competent in this unit, a person must demonstrate knowledge of:

construction terminology relating to stonemasonry stairs:

geometry of stairs

open flight

straight flight

quarter and half space landings

solid treads

veneer face

standards and codes for masonry stair construction

key requirements of the National Construction Code (NCC) relevant to stonemasonry stairs

mortar ingredients and consistency

rising damp

types and characteristics of stone:

granite

marble

slate

locally available stone

functional and operational features of plant, equipment and tools

battery operated tools

workplace processes and procedures

workplace safety:

job safety and environmental analysis (JSEA)

exposure to silica

hazardous manual tasks

exposure to excessive noise

features of working drawings and specifications

environmental requirements for workplace processes and waste disposal.

using digital tools and devices to source information and communicate with others.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

This unit must be assessed in the workplace or a close simulation reflecting workplace conditions and standards, materials, equipment, activities, responsibilities, procedures, safety requirements and environmental considerations.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad