# Assessment Requirements for CPCCST4007 Construct a fire brick wall and arch using refractory materials

Release: 2

Assessment Requirements for CPCCST4007 Construct a fire brick wall and arch using refractory materials

# Modification History

| Release 2 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.1. Elements and Performance Criteria 1.1 rewritten for clarity. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and equivalent to CPCCBC4042A Construct a fire brick wall and arch using refractory materials. Updated to meet the Standards for Training Packages 2012. |

# Performance Evidence

To demonstrate competency, a candidate must meet the performance criteria of this unit by constructing a fire brick wall and arch. This work may include a 230mm fire brick wall to incorporate an opening (600mm x 600mm) with a segmental arch above, a minimum of 1.5m long and 1.2m high to the top of the arch.

In doing this, the candidate must:

demonstrate understanding of the purpose and application of refractory bricklaying

comply with Work Health and Safety (WHS) regulations applicable to workplace operations

use safe and effective procedures to handle hazardous materials

read and interpret two-dimensional refractory plans and specifications

select and use processes, tools and equipment appropriate to the job requirements

prepare materials and samples in accordance with job requirements

identify typical faults and problems with materials and equipment that occur and take necessary action to rectify

communicate effectively with others to ensure safe work site operations.

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

Building Code of Australia (BCA) and other Australian standards relevant to the nature of work and materials being used

characteristics and applications of different materials used in refractory brick work:

required bond:

header bond

stretch bond

adjustable props:

metal props

packers

timber toms

timber wedges

wedge shape bricks include the following common sizes:

51mm

63mm

69mm

methods and processes relating to the use of refractory bricks and insulation materials for the construction of fire resistant structures:

measurements and placements:

accurate positioning and maintenance of the centreline of the key brick is through the vertical centre line of the arch ensuring:

all bricks are cut and laid to maintain even joints

even joint thickness around the extrados

organisational procedures for the acquisition and storage of materials

workplace and equipment safety requirements, including relevant statutory regulations, codes and standards.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

Assessment of performance must be undertaken in the workplace or in a simulated workplace environment. Tasks are to be performed to the level of proficiency and within the time limits expected in a workplace.

Assessors are responsible for ensuring that the person demonstrating competency has access to:

materials appropriate to refractory brickwork

plant, tools and equipment relevant to refractory brickwork.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad