# Assessment Requirements for CPCCWC3002 Install and finish plasterboard and fibre cement sheeting to arch substrate

Release: 4

Assessment Requirements for CPCCWC3002 Install and finish plasterboard and fibre cement sheeting to arch substrate

# Modification History

| Release 4 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 3 | This version first released with CPC Construction, Plumbing and Services Training Package Release 6.2. Correction to prerequisite unit code from CPPWHS2001 Apply WHS requirements, policies and procedures in the construction industry to CPCCWHS2001 Apply WHS requirements, policies and procedures in the construction industry. |
| Release 2 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.1. Formatting correction to Elements and Performance Criteria. |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and equivalent to CPCCWC3002A Install and finish plasterboard and fibre cement sheeting to arches. Change to title. Updated to meet the Standards for Training Packages 2012. |

# Performance Evidence

To demonstrate competency, a candidate must satisfy all of the elements, performance criteria and foundation skills of this unit by:

installing and finishing to specification, one plasterboard arch with:

a minimum of two sheets

wall and corner junctions

application or arch beads

mechanical or adhesive fixing systems

setting out to specification one each of segmental, gothic, elliptical, Tudor and ogee arches.

All work must be performed to the standard required in the workplace and must comply with the National Construction Code (NCC), Commonwealth and state/territory laws and regulations, Australian Standards, work health and safety (WHS) and environmental requirements, manufacturers’ specifications, and workplace requirements.

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

compliance requirements of the National Construction Code and Australian Standards for installing and finishing plasterboard and fibre cement sheeting to arch substrates

quality policies and standards for installing and finishing plasterboard and fibre cement sheeting to arch substrates

safety requirements for installing and finishing plasterboard and fibre cement sheeting to arches, including personal protective equipment (PPE), job safety analyses, safe work method statements and safety data sheets

environmental requirements for installing and finishing plasterboard and fibre cement sheeting to arch substrates

plans, drawings and specifications for installing and finishing plasterboard and fibre cement sheeting to arch substrates

processes for the calculation of material requirements for installing and finishing plasterboard and fibre cement sheeting to arch substrates

techniques for fixing materials to steel and timber frames for installing and finishing plasterboard and fibre cement sheeting to arch substrates

types, uses and limitations of materials used for installing and finishing plasterboard and fibre cement sheeting to to arch substrates

arch types, options, uses and limitations, including:

elliptical

gothic

may include

ogees

segmental

semi-circular

three-centred

types of tools and equipment for installing and finishing plasterboard and fibre cement sheeting to arch substrates.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

This unit must be assessed:

in the workplace or a close simulation of the workplace environment, provided that simulated or project-based assessment techniques replicate construction workplace conditions, materials, activities, responsibilities and procedures

using realistic tasks or simulated tasks, relevant specifications and work instructions, standard work practices, safety requirements and environmental constraints.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad