# Assessment Requirements for CPCCWF3009 Apply waterproofing for wall and floor tiling

Release: 2

Assessment Requirements for CPCCWF3009 Apply waterproofing for wall and floor tiling

# Modification History

| Release 2 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. New unit. |

# Performance Evidence

To demonstrate competency, a candidate must satisfy all of the elements, performance criteria and foundation skills of this unit by waterproofing an internal wet area and an external wet area ready for tiling, including:

one shower area 1800 millimetres high, 900 millimetres x 900 millimetres wide, incorporating:

a niche

tap penetrations

draining connection

water stop/hob

surrounds for an inserted bath with periphery barrier or water stop

a minimum of four square metres of  balcony/deck:

where the structural floor levels have a step down of no less than 40 millimetres from internal to external

the balcony/deck has fall to the drainage point

detailing is provided through threshold cross-section, indicating how waterproofing would terminate at balustrade posts.

All work must be performed to the standard required in the workplace and must comply with Commonwealth and state/territory laws and regulations, Australian Standards, work health and safety (WHS) and environmental requirements, manufacturers’ specifications and workplace requirements.

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

requirements of Australian Standards for setting out and applying waterproofing systems for wall and floor tiling to internal and external wet areas

types and uses of tools and equipment for applying waterproofing for wall and floor tiling

safety requirements Standards for setting out and applying waterproofing systems for wall and floor tiling to internal and external wet areas, including:

job safety analyses and/or safe work method statements in accordance with Safe Work Australia Code of Practice for Construction Work or state and/or territory equivalent

personal protective equipment (PPE)

techniques and methods for waterproofing for wall and floor tiling

assessment and appreciation of moisture content in substrate materials

manufacturers’ specifications for compatible substrates, waterproofing systems, tiles and tile adhesives

characteristics and applications of:

adhesives

liquid applied:

water-based

cementitious

moisture-cured

sheet membranes:

butynol

ethylene propylene diene monomer rubber (EPDM)

ethylene vinyl acetate (EVA)

expanded polystyrene (XPS)

primer:

water based

epoxy

bond breaker/fillet materials

construction systems and waterproofing considerations

penetrations and outlets, and termination and flashing principles

types and uses of materials storage and environmentally friendly waste management

types and uses of plans, drawings and specifications for waterproofing internal and external wet areas

requirements for different types of internal and external wet areas:

showers

laundries

toilets

bathrooms

balconies

principles and considerations of water exclusion and processes for inspection and completion activities

processes for the calculation of material requirements for applying waterproofing for wall and floor tiling

compatible substrate and materials for waterproofing wet areas.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

This unit must be assessed in the workplace or a close simulation using realistic workplace conditions, materials, activities, responsibilities, procedures, safety requirements and environmental considerations.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad