# CPCCWP3002 Apply waterproofing process to internal wet areas

Release: 3

CPCCWP3002 Apply waterproofing process to internal wet areas

# Modification History

| Release 3 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 2 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.1. Changes to Performance Evidence and Unit Sector. |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and equivalent to CPCCWP3002A Apply waterproofing process to internal wet areas. Updated to meet the Standards for Training Packages 2012. |

# Application

This unit specifies the skills and knowledge required to apply waterproofing to internal wet areas, meeting all relevant requirements of Australian Standards, work health and safety (WHS), and Commonwealth and state or territory legislation.

It includes identification of the appropriate waterproofing system to be used, its preparation and its application.

A person working at this level would be expected to take responsibility for organising and completing tasks assigned to them without close supervision.

Completion of the general construction induction training program specified by the model Code of Practice for Construction Work is required for any person who is to carry out construction work.  Achievement of CPCWHS1001 Prepare to work safely in the construction industry meets this requirement.

No licensing, legislative or certification requirements apply to this unit at the time of publication.

# Pre-requisite Unit

CPCCWHS2001 Apply WHS requirements, policies and procedures in the construction industry

# Unit Sector

Construction waterproofing

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Plan and prepare to apply waterproofing process to internal wet areas. | 1.1 | Read and interpret work instructions and plan sequence of work. |
|  |  | 1.2 | Select and use personal protective equipment (PPE) as required for each stage of the task. |
|  |  | 1.3 | Plan all work to comply with laws and regulations, Australian Standards, work health and safety (WHS) and environmental requirements, manufacturers’ specifications, workplace requirements, drawings and specifications. |
|  |  | 1.4 | Follow requirements of job safety analyses and safety data sheets. |
|  |  | 1.5 | Inspect work site, assess hazards and apply risk controls, including required signage and barricades. |
|  |  | 1.6 | Select tools and equipment, check for serviceability and report any faults. |
|  |  | 1.7 | Select materials required for task, calculate quantities, handle safely and prepare and position ready for use. |
| 2 | Select waterproofing system. | 2.1 | Locate internal structure to be waterproofed from drawings and specifications. |
|  |  | 2.2 | Check area for soundness and surface defects. |
|  |  | 2.3 | Select waterproofing systems suitable for and compatible with site conditions and substrate. |
| 3 | Prepare for waterproofing installation. | 3.1 | Set out internal wet area and check alignment, fixtures and finished levels for conformity with drawings and specifications, and report defects. |
|  |  | 3.2 | Test moisture content in substrate for compatibility with system and report defects. |
|  |  | 3.3 | Check adequacy of drainage and report defects. |
|  |  | 3.4 | Prepare flashings and termination seals and make ready for installation to manufacturers’ specifications. |
|  |  | 3.5 | Check that substrate has a uniform finish to manufacturers’ instructions. |
|  |  | 3.6 | Clean and prepare substrate ready for waterproofing. |
|  |  | 3.7 | Prime substrate ready for waterproofing. |
| 4 | Apply waterproofing. | 4.1 | Apply waterproofing system to prepared surface to manufacturers’ specifications. |
|  |  | 4.2 | Detail and terminate waterproofing membrane in accordance with system requirements. |
|  |  | 4.3 | Check completed waterproofing installation for conformity to manufacturers’ specifications. |
|  |  | 4.4 | Document movement joints for subsequent trades. |
|  |  | 4.5 | Perform final inspection and completion activities. |
|  |  | 4.6 | Protect waterproofing membrane. |
| 5 | Apply sealants to construction joints. | 5.1 | Select sealant for compatibility with application, substrate and finishes. |
|  |  | 5.2 | Install backing rods and/or release agents to prevent three-sided adhesion. |
|  |  | 5.3 | Apply sealant manufacturers’ specifications, tool to required finish and allow to cure. |
|  |  | 5.4 | Perform final inspection and completion activities. |
|  |  | 5.5 | Protect sealant, in accordance with manufacturers’ specifications. |
| 6 | Clean up. | 6.1 | Clear work area, dispose of materials, and reuse or recycle in compliance with legislation, regulations and codes of practice. |
|  |  | 6.2 | Clean, check, maintain and store tools and equipment in accordance with manufacturers’ recommendations. |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

Supersedes and equivalent to CPCCWP3002A Apply waterproofing process to internal wet areas.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad

Assessment Requirements for CPCCWP3002 Apply waterproofing process to internal wet areas

# Modification History

| Release 3 | Updated reference to general construction induction training program. |
| --- | --- |
| Release 2 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.1. Changes to Performance Evidence and Unit Sector. |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and equivalent to CPCCWP3002A Apply waterproofing process to internal wet areas. Updated to meet the Standards for Training Packages 2012. |

# Performance Evidence

To demonstrate competency, a candidate must satisfy all of the elements, performance criteria and foundation skills of this unit by waterproofing an internal wet area, including:

one shower area, a minimum 1800 millimetres high, 900 millimetres x 900 millimetres wide, incorporating:

a niche

tap penetrations

draining connection

water stop/hob

surrounds for an inserted bath with periphery barrier or water stop.

All work must be performed to the standard required in the workplace and must comply with laws and regulations, Australian Standards, work health and safety (WHS) and environmental requirements, manufacturers’ specifications and workplace requirements.

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

requirements of Australian Standards for waterproofing internal wet areas

safety requirements for waterproofing internal wet areas, including:

job safety analyses and/or safe work method statements in accordance with Safe Work Australia Code of Practice for Construction Work or state and/or territory equivalent

safety data sheets (SDS)

personal protective equipment (PPE)

manufacturers’ specifications for waterproofing systems and materials

assessment and appreciation of moisture content in substrate materials for waterproofing internal wet areas

waterproofing around fixtures:

vanities

toilets

shower screens

taps

penetrations

waterproofing materials, processes and systems for internal wet areas:

waterproofing systems/materials:

cementitious

water-based

moisture-cured

sealants

sprayed material membranes

sheet:

butyl

ethylene propylene diene monomer rubber (EPDM)

polyvinyl chloride (PVC)

decoupling

ancillary materials:

adhesives

sealants

drainage flanges

penetration flashings

water stops

internal waterproofing materials, processes and techniques

types and uses of materials storage and environmentally friendly waste management

types and uses of plans, drawings and specifications for waterproofing internal wet areas

types and uses of tools and equipment for waterproofing internal wet areas

principles and considerations of water exclusion

processes for the calculation of material requirements for waterproofing internal wet areas

quality requirements for waterproofing internal wet areas

termination and flashing principles.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

This unit must be assessed in the workplace or a close simulation using realistic workplace conditions, materials, activities, responsibilities, procedures, safety requirements and environmental considerations.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad