# CPCCWP5004 Determine origin of water and moisture ingress into buildings

Release: 1

CPCCWP5004 Determine origin of water and moisture ingress into buildings

# Modification History

| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 9.0. Newly created unit. |
| --- | --- |

# Application

This unit of competency specifies the skills and knowledge required to determine the origin of water and moisture ingress into buildings.

The unit applies to Class 1 and 10 Buildings, and Class 2 to 9 Buildings.

The unit covers gathering information, carrying out the investigation, carrying out the analysis and reporting on findings and conclusions.

The unit is suitable for builders, building supervisors building surveyors, building designers, architects, waterproofing design specialists, waterproofing supervisors, and consultants, engineers or other design specialists.

Licensing, legislative, regulatory or certification requirements may apply to this unit. Relevant work health and safety (WHS) state and territory regulatory authorities should be consulted to confirm jurisdictional requirements.

# Pre-requisite Unit

CPCWHS1001 Prepare to work safely in the construction industry

# Unit Sector

Building and Construction

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1. | Gather information. | 1.1 | Identify areas of water ingress in consultation with stakeholder(s). |
|  |  | 1.2 | Gather relevant drawings, historical reports, design or structural specifications and certification where available. |
|  |  | 1.3 | Identify signs and symptoms of the water or moisture ingress. |
|  |  | 1.4 | Confirm scope of project based on information and evidence. |
| 2. | Carry out investigation. | 2.1 | Identify potential water and moisture ingress points into buildings. |
|  |  | 2.2 | Determine if non-destructive or destructive investigation is required in consultation with stakeholders. |
|  |  | 2.3 | Select tools and technologies to carry out inspections and testing procedures in accordance with site conditions. |
|  |  | 2.4 | Carry out inspection and testing using valid testing methods in accordance with testing protocols and against controls and references. |
|  |  | 2.5 | Record findings of investigation and testing protocols. |
|  |  | 2.6 | Compare findings to information and evidence of water and moisture ingress areas and carry out further investigations where required. |
| 3. | Carry out analysis. | 3.1 | Collate evidence from investigations and testing in accordance with protocols. |
|  |  | 3.2 | Analyse causes of water and moisture ingress as determined by investigations and collected evidence. |
|  |  | 3.3 | Determine if external factors have influenced the findings. |
|  |  | 3.4 | Carry out re-testing to confirm influence of external factors where required. |
|  |  | 3.5 | Record results of analysis for stakeholders in accordance with evidence to inform report. |
| 4. | Report findings and conclusions. | 4.1 | Collate, sort and prioritise information according to the scope requirements within a report. |
|  |  | 4.2 | Support diagnostic report findings with relevant references to regulatory requirements, Australian Standards, design and manufacturers’ requirements in accordance with stakeholder brief. |
|  |  | 4.3 | Determine requirements for further support testing, follow-up investigations and monitoring of area in accordance with inspection and testing protocols. |
|  |  | 4.4 | Communicate relevant and supported conclusions according to stakeholder expectations, context and requirements in accordance with report brief. |
|  |  | 4.5 | Document the report in accordance with report brief. |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

No equivalent unit.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad

Assessment Requirements for CPCCWP5004 Determine origin of water and moisture ingress into buildings

# Modification History

| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 9.0. Newly created unit. |
| --- | --- |

# Performance Evidence

A person demonstrating competency in this unit must satisfy the requirements of the elements, performance criteria and foundation skills of this unit, in addition to the specific performance and knowledge evidence described below.

Candidates must conduct three (3) water and moisture ingress investigations and develop diagnostic reports for three (3) different areas, chosen from list provided:

one investigation and report from an internal wet area which includes a shower

one investigation and report of water ingress through:

an external wall, adjacent to a waterproofed balcony or deck; OR

water ingress below a concrete roof terrace

one investigation and report of water ingress below a waterproofed, lightweight, flat roof structure or balcony

one investigation and report of water ingress to habitable space below ground.

In doing so, the diagnosis and evidence reports must be written in such a way as to inform a remedial waterproofing design and scope of works and must include:

all relevant background information

investigation methodology

record of findings

identification of the cause(s) of the water and moisture ingress

review of evidence against site factors and influences

assessment of waterproofing design and application against compliance with National Construction Code (NCC) performance requirements, Australian Standards, manufacturers’ recommendations, building design and use of area.

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

application of scientific methodology

diagnostic procedures – non-destructive and destructive, including:

site investigation

inspecting protocols

analysing evidence

suitable technologies:

thermal

microwave

ground penetrating radar (GPR)

dye water testing

impedance and capacitance technologies

electronic field vector mapping

high and low voltage holiday testing

setting control parameters and testing methods, including:

selection of appropriate test method

calibration of testing equipment

effects on testing equipment by site conditions and testing procedures

contributing site factors and influence, including:

water ingress sources

surface water pathways

sub-surface water pathways

properties of water and moisture movement

gradient and falls

hydrostatic pressure

effects of finishes on surface tension

capillarity

size and effectiveness of outfalls

penetrations and intrusion points

condensation

ventilation

building elements and integration, including:

principles of water shedding

relationship between structural components

building movement

wind loadings

construction materials and their properties

NCC performance requirements relevant to areas tested, including:

waterproofing

damp proofing

condensation management

NCC evidence of suitability in relation to waterproofing system performance, including:

independent test reports

CodeMark Australia

other independent testing organisations

Australian Standards and design requirements relevant to areas inspected

waterproofing system manufacturers’ requirements, including:

data sheets and specifications

installation method statements

performance of system according to service conditions

suitability of materials as fit for purpose

compatibility of components within the system

stakeholders:

architect

engineer

builder

building owner

consumer

design team

construction team

certifier

building surveyor

regulators

report content and communication of findings, including:

types of diagnostic reports and structures

tables and diagrams

photographic evidence

interpreting test results

references to Australian Standards and designs

verbal communication strategies

different audiences

presenting evidence.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

Assessment of performance must be undertaken in the workplace or simulated workplace environment. A simulated workplace is one that fully replicates the resources, environment and any time and productivity pressures that exist in the actual workplace, and which meets industry standards for safety and environmental practices.

Candidates must have access to:

relevant tasks

Australian Standards, relevant building legislation, industry codes, NCC and requirements of workplace policies and procedures as required by Commonwealth, state and territory regulators

relevant environmental requirements.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad