# CPCPCM3021 Flash penetrations through roofs and walls

Release: 1

CPCPCM3021 Flash penetrations through roofs and walls

# Modification History

| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCPCM3021A Flash penetrations through roofs and walls. Updated to meet the Standards for Training Packages 2012. |
| --- | --- |

# Application

This unit specifies the skills and knowledge required to set out, cut and flash roof and wall penetrations.

In some jurisdictions, this unit of competency may form part of accreditation, licensing, legislative, regulatory or certification requirements.

# Pre-requisite Unit

CPCPCM2043 Carry out WHS requirements

# Unit Sector

Plumbing

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Identify installation requirements. | 1.1 | Access, read and determine flashing penetration requirements from relevant job plans and specifications, codes, Australian standards, manufacturer’s specifications and jurisdictional requirements. |
|  |  | 1.2 | Identify and apply workplace policies and procedures, work health and safety (WHS) and environmental requirements. |
| 2 | Prepare for work. | 2.1 | Create a materials list and collect materials. |
|  |  | 2.2 | Select appropriate tools and equipment, including personal protective equipment (PPE). |
| 3 | Flash penetrations. | 3.1 | Identify location of wall and roof penetrations, allowing for existing and future services. |
|  |  | 3.2 | Set out and cut penetrations. |
|  |  | 3.3 | Install supports as required and fit the penetrating product. |
|  |  | 3.4 | Fit and fix purpose-made or proprietary flashing as per relevant job plans and specifications, codes, Australian standards manufacturer’s specifications and jurisdictional requirements. |
|  |  | 3.5 | Apply fire resistant requirements to penetrations to meet standards and codes where required. |
| 4 | Clean up. | 4.1 | Clear the work area, and dispose of, reuse or recycle materials in accordance with state and territory legislation and workplace policies and procedures. |
|  |  | 4.2 | Clean tools and equipment, check for serviceability and report any damage, store and secure. |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

Supersedes and is equivalent to CPCPCM3021A Flash penetrations through roofs and walls.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad