# Assessment Requirements for CPCPCM3023 Fabricate and install non-ferrous pressure piping

Release: 3

Assessment Requirements for CPCPCM3023 Fabricate and install non-ferrous pressure piping

# Modification History

| Release 3 | This version first released with CPC Construction, Plumbing and Services Training Package Release 6.5. Amended Performance Criteria 3.2. |
| --- | --- |
| Release 2 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.1. Changes to Element and Performance Criteria 2.2 and 3.5. |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCPCM3023A Fabricate and install non-ferrous pressure piping. Updated to meet the Standards for Training Packages 2012. |

# Performance Evidence

To demonstrate competency, a candidate must meet the performance criteria for this unit by:

fabricating, installing and testing a DN20 copper line from a supply point to three fixed outlets, supported and clipped according to relevant job plans and specification, codes, Australian Standards, manufacturer’s specifications and jurisdictional requirements with a minimum:

one 15 mm mechanical bend

one 20 mm mechanical bend

one 15 mm spring bend

one 20 mm spring bend

one 15 mm capillary elbow

one 20 mm capillary elbow

one expanded silver brazed joint

one silver brazed copper to brass joint

two branch formed silver brazed joints

one mechanical flared joint

branches connecting at least two other non-ferrous materials, ensuring branches and bends are square.

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

fabrication, installation and testing process for non-ferrous pressure pipe systems

relevant statutory authorities’ requirements and standards related to fabricating, installing and testing non-ferrous pressure pipe systems

effect of heat and other products on the properties of polymer pipe

types of non- ferrous materials, including joining techniques

hot works permit

how to access relevant information, including codes and standards

tools, materials and brazing equipment used to fabricate and install non-ferrous pressure piping

work health and safety (WHS) requirements to fabricate and install non-ferrous pressure piping.

# Assessment Conditions

Assessors must satisfy the requirements for assessors listed in the Standards for Registered Training Organisations.

This unit must be assessed in the workplace or a close simulation using realistic workplace conditions, materials, activities, responsibilities, procedures, safety requirements and environmental considerations.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad