# CPCPDR3024 Maintain and service domestic treatment plants and onsite sewerage facilities

Release: 1

CPCPDR3024 Maintain and service domestic treatment plants and onsite sewerage facilities

# Modification History

| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. New unit. |
| --- | --- |

# Application

This unit specifies the skills and knowledge required to maintain and service domestic treatment plants and onsite sewerage facilities.

The unit covers maintaining system components, conducting and analysing effluent quality tests and maintaining systems on new or existing domestic or commercial structures.

In some jurisdictions, this unit of competency may form part of accreditation, licensing, legislative, regulatory or certification requirements.

# Pre-requisite Unit

CPCPDR4013 Design and size domestic treatment plant disposal systems

# Unit Sector

Plumbing

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Prepare for work. | 1.1 | Identify and arrange work health and safety (WHS) and environmental requirements necessary to undertake the work. |
|  |  | 1.2 | Complete a site risk assessment, including hazards associated with exposure to wastewater and wastewater treatment facilities. |
|  |  | 1.3 | Develop procedures to minimise risk associated with identified hazards. |
| 2 | Maintain components. | 2.1 | Check operation of pumps and aerators to identify maintenance requirements. |
|  |  | 2.2 | Perform maintenance of pump systems and components and aerators and replace as required. |
|  |  | 2.3 | Clean membranes, filters, UV light tubes and ozone-producing equipment where appropriate. |
|  |  | 2.4 | Measure sludge and scum levels in septic tanks and treatment plants. |
|  |  | 2.5 | Analyse and maintain land application areas and piping systems. |
|  |  | 2.6 | Determine appropriate procedures for loss of power and equipment failure. |
|  |  | 2.7 | Perform maintenance of relevant components and replace system devices where required. |
| 3 | Conduct and analyse effluent quality tests. | 3.1 | Calibrate and use commercial testing equipment to conduct water quality tests and record results in accordance with legislative requirements. |
|  |  | 3.2 | Collect, prepare and send water samples to an accredited laboratory for effluent quality tests where appropriate. |
|  |  | 3.3 | Identify rectification work required from effluent quality test results. |
|  |  | 3.4 | Perform rectification work as required. |
| 4 | Maintain systems. | 4.1 | Analyse and determine the correct treatment processes for a range of small decentralised treatment facilities. |
|  |  | 4.2 | Maintain a range of wastewater treatment facilities in compliance with legislation. |
|  |  | 4.3 | Maintain black and/or grey water systems to comply with end user, water and regulatory quality parameters. |
| 5 | Complete documentation and report outcomes. | 5.1 | Complete service reports and correspondence outlining maintenance issues. |
|  |  | 5.2 | Provide advice to end user. |
|  |  | 5.3 | Produce an operation and maintenance schedule for end user. |
|  |  | 5.4 | Document and report outcomes to local authorities and property owners. |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

No equivalent unit.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad

Assessment Requirements for CPCPDR3024 Maintain and service domestic treatment plants and onsite sewerage facilities

# Modification History

| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. New unit. |
| --- | --- |

# Performance Evidence

To demonstrate competency, a candidate must meet the performance criteria for this unit by:

applying an operation and maintenance procedure for three of the following systems, ensuring a primary, secondary and tertiary system is selected:

primary treatment conventional septic tank

primary biological filter

secondary treatment aerated wastewater treatment plant

secondary treatment sand filter

tertiary ultraviolet (UV) disinfection and ozone oxidation

tertiary chlorine.

For each procedure the person must identify and report faults to appropriate person.

# Knowledge Evidence

To be competent in this unit, a person must demonstrate knowledge of:

disinfection technologies and land application systems

system processes to reduce nutrients

interpreting and applying test results for:

dissolved oxygen readings

clarity readings of wastewater

various air pressures on aerator

rectification of incorrect residual chlorine readings

calculation of pump duty

rectification of incorrect pH readings

microbiology of wastewater constituents

SI system of measures

work health and safety procedures

dissolved oxygen

pH parameters

air pressure from the aerator

residual chlorine parameter

pump operation

pump duty

infectious diseases

types of wastewater treatment processes

risks with working with wastewater

types of land application areas

maintenance procedures

legislation and regulations relating to wastewater

wastewater quality parameters

areas of consideration for health and safety:

protective clothing and equipment

use of tools and equipment

workplace environment and safety

handling of materials

hazardous materials

handling of materials

hazard control

electrical

lifting safety

first aid requirements for injuries

personal hygiene

areas of potential risk:

hygiene

biological

health

chemical

electrical

types of hazards:

chlorine

influent

effluent

electrical

UV

ozone

pH correction chemicals

correct operation of filters and pumps

types of land application:

absorption trenches and beds

evapotranspiration beds

mounds

shallow sub-surface irrigation systems

covered surface drip irrigation

low pressure effluent drip irrigation systems

above-ground irrigation

constructed wetlands:

reed beds

lagoons

devices used in effluent systems:

sludge recycling device

disinfecting device

dosing device

pumps

aerators

valves

tests for effluent quality:

biochemical oxygen demand (BOD)

sludge

suspended soils

clarity

pH

dissolved oxygen

bacterial content (pathogenic organisms)

free chlorine

total nitrogen

total phosphorous

rectification solutions:

recommended use of household products

adjustments to system

improvements to system

result information inclusions:

summary of own investigations

local authority reports

reports by other testing agencies or authorities

end users such as:

landowners

tenants

how to access relevant information, including codes and standards

tools, materials and equipment used to maintain and service domestic treatment plants and onsite sewerage facilities

work health and safety (WHS) requirements for maintaining and servicing domestic treatment plants and onsite sewerage facilities.

# Assessment Conditions

Assessors must satisfy the requirements for assessors listed in the Standards for Registered Training Organisations.

This unit must be assessed in the workplace or a close simulation using realistic workplace conditions, materials, activities, responsibilities, procedures, safety requirements and environmental considerations.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad