# Assessment Requirements for CPCPFS3038 Test and maintain fire hydrant and hose reel installations

Release: 3

Assessment Requirements for CPCPFS3038 Test and maintain fire hydrant and hose reel installations

# Modification History

| Release 3 | This version first released with CPC Construction, Plumbing and Services Training Package Release 6.2. PC2.1 Identify and calculate materials required to complete the task. PC2.2 Identify potential hazards and determine and implement control measures and personal protective equipment (PPE) requirements. PC2.3 Determine job priorities and sequence job tasks in consultation with others on site. The PCs have been repositioned to be consistent with the related CPCPFS sector units of competency. |
| --- | --- |
| Release 2 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.1. Knowledge Evidence formatted for clarity. |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCPFS3038A Test and maintain fire hydrant and hose reel installations. Updated to meet the Standards for Training Packages 2012. |

# Performance Evidence

To demonstrate competency, a candidate must meet the performance criteria for this unit by:

conducting routine testing and maintenance up to five yearly frequency, excluding fire pump performance and booster connection flow testing on two different fire hydrants and hose reel installations

diagnosing a critical defect and non-conformance and:

report as per AS 1851 Routine service of fire protection systems and equipment requirement

repair each defect and non-conformance

conduct testing to ensure rectification is effective

reinstate installations and complete a service report

replacing all serviceable components as per preventative maintenance requirements of AS 1851 Routine service of fire protection systems and equipment on:

three hydrant valves

two hose reels

replacing one faulty hose reel drum

setting a pressure reducing valve according to relevant specifications

performing system interfaces isolation of fire detection control and indicating equipment (FDCIE) and fire pumpsets

confirming correct location of hydrants, attack and feed hydrant and hose reels in accordance with AS 2419 Fire hydrant installations system design, installation and commissioning from a site or plan of 2000 square metres.

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

responsibilities relating to quality requirements for testing and maintaining fire hydrants and hose reel installations:

documentation and information systems and processes

policies and guidelines relating to work roles and responsibilities

workplace reporting

operational knowledge of hydrant and hose reel systems

purpose, function and maintenance requirements of the following components in hydrant and hose reel systems:

hydrant valves

hose reels

check valves

booster assemblies

gate valves

gauges

pipework

hydrant cabinets

booster cabinets

hose reel cabinets

hydrant and hose reel signage

isolation valves

pipe supports

anti-tamper switches

pump start initiating devices

block plans

processes, procedures and techniques of:

testing and maintaining fire hydrant and hose reel installations

system interfaces with hydrants and hose reels

water flow testing of hydrants and hose reels

how to access relevant information, including job plans and specifications, codes, Australian Standards, manufacturer’s specifications and jurisdictional requirements.to test and maintain fire hydrant and hose reel installations

tools, materials and equipment used to test and maintain fire hydrant and hose reel installations

work health and safety (WHS) requirements for performing tests and conducting fire hydrant and hose reel installations.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

This unit must be assessed in the workplace or a close simulation using realistic workplace conditions, materials, activities, responsibilities, procedures, safety requirements and environmental considerations.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad