# Assessment Requirements for CPCPFS3041 Inspect and test fire pumpsets

Release: 2

Assessment Requirements for CPCPFS3041 Inspect and test fire pumpsets

# Modification History

| Release 2 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.1. Changes to Elements and Performance Criteria 2.1, 2.3 and 3.8. Performance Evidence is formatted for clarity. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCPFS3041A Inspect and test fire pumpsets. Updated to meet the Standards for Training Packages 2012. |

# Performance Evidence

To demonstrate competency in this unit a candidate must meet the performance criteria for this unit by:

conducting four routine services on a fire pumpset, including:

three scheduled monthly, and

one scheduled six-monthly activities

and include:

compression ignition pumpsets used as a booster pumpset on a sprinkler/hydrant system

an electrical pumpset used as a:

booster pumpset on a sprinkler/hydrant system fed from a town mains supply or a static water supply

conducting all routine service in compliance with current Australian standards, regulations, codes and organisational requirements

completing work according to customer and organisational expectations within accepted timeframes.

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

application of fire pumpsets as defined in AS 2941 Fixed fire protection installations – Pumpset systems and AS 2118 Automatic fire sprinkler systems

terminology used in relation to pumpsets

basic principles of hydraulics:

basic head or pressure calculation for height per metre

metric and imperial pressure scales

basic principles of operation and purpose of components:

accumulators

batteries

circulation and system pressure relief valves

compression ignition pumpset drivers

electrical pumpset drivers

engine starting and control or monitor batteries

header tanks

impellers

isolating valves associated with pumpsets

jacking or jockey pumps

multi-stage pumps

pressure gauges

pump controllers and ancillary equipment for control and indications

pump drivers

pump glands and seals

pump performance curves

pump starting devices

pumpset couplings

safety guards

system pressure gauge schedules

water supply tanks:

atmospheric

suction with priming tanks

general operation of pumpsets:

cooling systems

design speed requirements

exhaust systems

fuel systems

full load operation

normal running operation

pre-start and post-start checks

pumpset controllers

starting and stopping methods

suction and discharge connections and pressure readings

pumpset types which may include those used in fire protection systems that comply with Australian standards such as:

AS 1851 Routine service of fire protection systems and equipment

AS 2118 Automatic fire sprinkler systems

AS 2419 Fire hydrant installations

AS 2941 Fixed fire protection installations - Pumpset systems

AS CA16-1962 Automatic sprinkler installations

NFPA 20 Standard for the installation of stationary fire pumps for fire protection (US)

system interfaces:

flow switches

pressure switches

tamper switches

valve positioning switches

control and indicating equipment (CIE)

devices that operate signals between pumpset and other services such as building monitoring services

other life safety systems:

warning systems

fire indicator panel (FIP)

security monitoring services

legislative and industry requirements:

dangerous goods regulations

licensing arrangements

environmental regulations

work health and safety (WHS) legislation, regulations and codes associated with fire pump rooms such as: working in isolation, lighting, fumes, trip hazards

relevant Commonwealth and state or territory building acts, regulations and codes, such as National Construction Code (NCC)

AS 1851 Routine service of fire protection systems and equipment

organisational requirements:

client-specific contractual requirements

documentation and information systems and processes

legal and organisational policies and guidelines, including personnel practices and guidelines outlining work roles, responsibilities and delegations

use of electronic job scheduling and communication devices

documentation to review and verify that installed systems comply with legislative and industry requirements such as:

building’s essential services or fire safety measures listing

relevant commonwealth and state or territory building acts, regulations and codes

relevant Australian standards listed on essential service listing

environmental regulations

methods of applying sustainability principles and concepts

testing procedures and activities that comply with requirements of AS 1851 Routine service of fire protection systems and equipment to verify the following pump functions:

pump starts at correct pressure

pump supplies correct kPa at no flow condition requirements

routine service frequency schedules:

reference to AS 1851 Routine service of fire protection systems and equipment schedules of work conducted at regular frequencies that relate to the work scope for monthly and six-monthly routine service schedules

how to access relevant information, including codes and standards

tools, materials and equipment used for inspecting and testing test fire pumpsets

WHS requirements for inspecting and testing test fire pumpsets.

# Assessment Conditions

Assessors must satisfy the requirements for assessors listed in the Standards for Registered Training Organisations.

This unit must be assessed in the workplace or a close simulation using realistic workplace conditions, materials, activities, responsibilities, procedures, safety requirements and environmental considerations.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad