# Assessment Requirements for CPCPFS3048 Install fixed fire pumpsets

Release: 2

Assessment Requirements for CPCPFS3048 Install fixed fire pumpsets

# Modification History

| Release 2 | This version first released with CPC Construction, Plumbing and Services Training Package Release 6.5. Amended Knowledge Evidence. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCPFS3048A Install fixed fire pumpsets. Updated to meet the Standards for Training Packages 2012. |

# Performance Evidence

To demonstrate competency, a candidate must meet the performance criteria for this unit by installing fixed fire pumpsets to AS 2419 Fire hydrant installations system design, installation and commissioning, including:

installing pump base and pump

installing pipework and associated valves and components

installing pressure relief valve and line

pressure testing piping system and pumpset

pressure maintenance pump (jockey pump)

pump start arrangement (pressure switch loop).

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

procedures and techniques for:

installation of fixed fire pumpset

pressure testing equipment

aligning pump and driver

structural systems, building materials and services that support or surround fixed fire pumpsets

characteristics, applications and limitations of approved materials and components of fixed fire pumpsets, above and below ground

types and operation of pumps used in fire protection:

end suction

multistage multi outlet centrifugal

split case

turbine

screw

jacking pumps

jockey pumps

types, requirements and operation of pump control panels

pump room requirements

ventilation requirements

atmospheric pressure measurements and principles in relation to fixed fire pumpset performance

performance measures for fixed fire pumpsets

power sources for fixed fire pumpsets, including operation and interaction of power sources

properties of water, including pressure and flow rates

vibration dampening

flexible tube vibration isolation

diesel driver exhaust sizing requirements

materials storage and environmentally responsible waste disposal relating to installing fixed fire pumpsets

how to access relevant information, including relevant job plans and specifications, codes, Australian Standards, manufacturer’s specifications and jurisdictional requirements to install fixed fire pumpsets

tools, materials and equipment used to install fixed fire pumpsets

work health and safety (WHS) requirements to install fixed fire pumpsets.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

This unit must be assessed in the workplace or a close simulation using realistic workplace conditions, materials, activities, responsibilities, procedures, safety requirements and environmental considerations.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad