# CPCPFS4027 Commission fire sprinkler systems

Release: 2

CPCPFS4027 Commission fire sprinkler systems

# Modification History

| Release 2 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.1. Knowledge Evidence formatted for clarity. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCPFS4027A Commission fire sprinkler systems. Updated to meet the Standards for Training Packages 2012. |

# Application

This unit specifies the skills and knowledge required to commission fire sprinkler systems.

It covers preparing for the work, identifying and confirming system specifications and requirements, physically testing and commissioning systems, and finalising work processes, including completing records and documentation.

The role may involve interaction with architects, builders, suppliers and relevant planning authorities and requires a sound understanding of applicable legislation including work health and safety (WHS).

This unit’s requirements may be carried out by experienced tradespeople such as hydraulic design consultants, fire services supervisors, plumbers or plumbing contractors who employ a team of plumbers.

Work may be undertaken on commercial or industrial buildings, which may be new work sites or existing structures.

In some jurisdictions, this unit of competency may form part of accreditation, licensing, legislative, regulatory or certification requirements.

# Pre-requisite Unit

Nil.

# Unit Sector

Plumbing

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Prepare for work | 1.1 | Consult with approving authority to determine commissioning inspections and tests required to validate fire sprinkler system performance meet design requirements. |
|  |  | 1.2 | Obtain and review documentation required for commissioning. |
|  |  | 1.3 | Consult with all associated persons to plan and sequence tasks. |
|  |  | 1.4 | Select and check the serviceability of appropriate tools, equipment and personal protective equipment (PPE). |
|  |  | 1.5 | Prepare work area to support efficient commissioning process. |
| 2 | Confirm that installation of system components correspond to design requirements. | 2.1 | Identify fire sprinkler system components and locations on drawings and specifications. |
|  |  | 2.2 | Inspect building to confirm that locations of system components correspond to design requirements and report any variances. |
|  |  | 2.3 | Confirm types of sprinkler heads are compatible and report if determined as incompatible. |
|  |  | 2.4 | Confirm spaces between sprinkler heads are correct and report any variances. |
|  |  | 2.5 | Inspect and confirm that pipework is functional and report any jointing and supporting defects. |
|  |  | 2.6 | Inspect valves for different commissioning tests to confirm correct type, labelling, and position. |
|  |  | 2.7 | Confirm pressure gauge schedule, block plan and interface diagram correspond to design requirements. |
|  |  | 2.8 | Check and verify water supply components, including any installed pumpsets against design requirements. |
|  |  | 2.9 | Identify changes to the building or existing conditions that could affect component performance. |
| 3 | Conduct pressure tests and restore system to normal pressure. | 3.1 | Conduct static air-pressure test to confirm integrity of the system, record results according to relevant standards and workplace requirements. |
|  |  | 3.2 | Conduct high pressure hydraulic test to confirm integrity of the system, record results according to relevant standards and workplace requirements. |
|  |  | 3.3 | Identify and report leaks. |
|  |  | 3.4 | Drain system and re-charge at normal pressure according to relevant standards and workplace requirements. |
| 4 | Conduct functional tests. | 4.1 | Conduct water supply functional proving test and record results according to relevant standards. |
|  |  | 4.2 | Conduct functional tests of system components and interfaces, record results according to relevant standards and design requirements. |
|  |  | 4.3 | Conduct tests according to environmental and sustainability requirements. |
| 5 | Finalise commissioning process. | 5.1 | Confirm system functionality and compliance with design specifications. |
|  |  | 5.2 | Confirm componentry is set to operational function and pipework is charged with water according to specifications. |
|  |  | 5.3 | Clean tools and equipment, check for damage and malfunction and store according to manufacturer recommendations and workplace procedures. |
|  |  | 5.4 | Access information to complete documentation according to workplace requirements and submit within specified timeframes. |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

Supersedes and is equivalent to CPCPFS4027A Commission fire sprinkler systems.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad

Assessment Requirements for CPCPFS4027 Commission fire sprinkler systems

# Modification History

| Release 2 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.1. Knowledge Evidence formatted for clarity. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCPFS4027A Commission fire sprinkler systems. Updated to meet the Standards for Training Packages 2012. |

# Performance Evidence

To determine competency in this unit, a person must demonstrate they can commission one wet system connected to fixed pumpsets, one wet system connected to town main supply and one pre-action fire sprinkler system, and in doing so:

correctly interpret plans and specifications to complete work to the specified standard within accepted timeframes

comply with appropriate legislation, organisational processes and regulatory requirements.

In doing this, the person must meet the performance criteria for this unit.

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

design and installation performance requirements of relevant installation standards including:

National Fire Protection Association (NFPA)

Factory Mutual performance-based codes of practice

AS 2118.1 Automatic fire sprinkler systems - General systems

industry and regulatory requirements for commissioning fire sprinkler systems including:

codes and standards

environmental and sustainability

licensing

quality assurance

work health and safety (WHS)

job safety analyses (JSA), safety data sheets (SDS) and safe work method statements (SWMS)

performance measures for fire sprinkler systems as referenced in design drawings and specifications

properties of water, including pressure, flow rates and atmospheric pressure

SI system of measurements

workplace and equipment safety requirements, including personal protective equipment (PPE) requirements relevant to the job

fire sprinkler systems including:

deluge

dry pipe

pre-action

storage sprinklers

wall wetting

water mist

wet pipe

approving authorities including:

client representative

council representative

fire brigade official

fire protection consultant engineer

insurance company representative

system designer

documentation including:

as-installed drawings

design calculations

logbooks

operator manuals

tools and equipment including:

digital tachometers

flow testing equipment

hand tools

laser levels

measuring devices

PPE

pressure gauges

fire sprinkler system components including:

air compressors

alarm valves

ball valves

brackets

discharge nozzles

flow switches

pipework

pressure gauges

pressure reducing valves

pressure relief valves

pressure switches

pumpsets

solenoids

sprinkler heads

water supply valves

functional tests to check effective operation of:

brigade booster connections

control and actuator mechanisms including:

pressure switches

flow switches

control valves, including:

pressure reducing

pressure relief

tank inflow

interfaces to other systems, including:

booster pumps

building services

control indicating equipment (CIE)

control of booster pumpsets

fire detection

heating, ventilation and air conditioning (HVAC)

occupant warning systems

valve monitoring controls

remote water proving points

water supply components, such as tank infill and suction lines.

# Assessment Conditions

Assessors must satisfy the requirements for assessors listed in the Standards for Registered Training Organisations.

This unit must be assessed in the workplace or a close simulation using realistic workplace conditions, materials, activities, responsibilities, procedures, safety requirements and environmental considerations.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad