# Assessment Requirements for CPCPRE2004 Caulk prefabricated concrete elements

Release: 1

Assessment Requirements for CPCPRE2004 Caulk prefabricated concrete elements

# Modification History

| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 9.0. Newly created unit. |
| --- | --- |

# Performance Evidence

To demonstrate competency, a candidate must meet the elements and performance criteria of this unit by:

caulking a 20 mm nominal gap between two prefabricated concrete elements, for at least 3 m.

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

compliance requirements of legislation, regulations, codes and Australian Standards relevant to joint caulking of concrete prefabricated elements, including:

environmental protection and waste disposal

work health and safety (WHS) regulations, including:

use of personal protective equipment (PPE) as specified on the safety data sheet (SDS)

risk when working at heights/on ladders

fall protection

drop zones

hazardous substances

safe manual handling techniques

respiratory crystalline silica

detrimental effect of water addition to concrete properties

precautions that should be taken to minimise any potential adverse effects when finishing concrete

caulk manufacturer technical data sheet

types, characteristics and purposes of construction drawings to confirm:

caulking thickness

fire rating requirements

specifications used to interpret concrete installation requirements

characteristics of caulking materials and their effect on, and appropriateness of use with, priming agents

types, characteristics and purposes of caulking surfaces and materials, including:

existing silicon or caulking material

existing backing rod

laitance

concrete prefabricated bonding agent residue

sealed concrete prefabricated surface

painted concrete prefabricated surface

moisture content of joint

silicone

polyurethane

modified polymer

polysulphide

acrylic

fire rating

caulking tool types and their purpose, including:

wire brush

soft brush

scraper

air compressors and hoses

portable blower

sander/grinder

caulking ‘slicks’ (sausages)

spatula

roller

putty knife

caulking gun

power leads

testing equipment (moisture meter)

workplace requirements for caulking prefabricated concrete elements:

cleaning up the work area

maintaining and storing plant, labelling, tools and equipment

reporting problems

quality

terminology used for caulking

manufacturer technical requirements

specifications used for joint caulking

types of concrete prefabricated elements

fire rating requirements relevant to concrete prefabricated element rectification

workplace procedures related to communication protocols

principles of sustainability relevant to material reuse

work planning and logical task sequencing

types of concrete surface variables that reduce optimal sealant adhesion

concrete prefabricated release agent residue

impact of laitance, dirt and dust on caulking

difference between closed cell and open cell backing rod

processes for calculating material measurements and quantities.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

Assessment must be conducted in the workplace or a simulated workplace using realistic conditions, materials, activities, responsibilities, procedures, safety requirements and environmental considerations.

Candidates must have access to erection design documentation, manufacturer technical data sheets, materials, plant, tools and equipment required to achieve the performance evidence.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad