# Assessment Requirements for CPCPRE3000 Lift and place prefabricated concrete elements

Release: 1

Assessment Requirements for CPCPRE3000 Lift and place prefabricated concrete elements

# Modification History

| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 9.0. Newly created unit. |
| --- | --- |

# Performance Evidence

To demonstrate competency, a candidate must meet the elements and performance criteria of this unit by:

determining the requirements for, and preparing for the lifting and placing of three prefabricated concrete elements in coordination with the crane driver and crane crew:

unloading, lifting, placing and temporarily securing one prefabricated element (that is at least 4 m x 3 m) by face-lifting the element

unloading, lifting and placing and temporarily securing one prefabricated element (that is at least 4 m x 3 m) by edge-lifting the element, and

unloading, lifting and conducting a mid-air rotation, placing and temporarily securing one prefabricated element (that is at least 7 m x 3 m).

# Knowledge Evidence

To be competent in this unit, a candidate must demonstrate knowledge of:

compliance requirements of legislation, regulations, codes of practice and Australian Standards relevant to the erection of prefabricated concrete elements, including:

AS 3850.1, 2 & 3 Prefabricated concrete elements re the types of rigging.

environmental protection and waste disposal

work health and safety (WHS) regulations, including:

Safe Work Australia ‘Guide to managing risk in construction: prefabricated concrete’ 2019

use of personal protective equipment (PPE)

risk when working at heights/on ladders

fall protection

drop zones

hazardous substances

safe manual handling techniques

principles of effective team leadership

erection design documentation

the strength of concrete at the time of lifting and handling from the manufacturers’ “birth certificate” contained in the erection documentation

faulty manufacturing and concrete defects which could compromise the strength of the prefabricated elements and pose a potential safety issue for lifting and handling

serviceability of temporary erection supports, including:

braces

props

working load limit (WLL)

erection documentation, including:

manufacturer specification

element drawings

rigging configurations and drawings

layout drawings

specifications required for the lifting and placing of prefabricated concrete elements

types of prefabricated concrete elements, including:

panels

columns

beams

planks

stairs

barriers

poles

pipes

box culverts

pits

wall elements

relevant types and procedures for the temporary support of prefabricated concrete elements, including:

erection sequence

orientation of prefabricated concrete components

bracing details, including type and angle

requirement for temporary brace and prop footings

placement and positioning of propping systems

levelling

crane operations, including:

crane standing

suspended slab operations

hoist reeving for single and multi-winch operations

requirements for single and multi-crane mid-air rotations

types, characteristics and limitations of tools, equipment and materials required for rigging, lifting and placing prefabricated concrete elements, including:

chain slings

flexible steel wire rope (FSWR) slings

concrete lifting clutches and compatible lifting inserts

post-installed lifting plates

hooks, shackles, chain links, swivels and lifting rings

sheaves and pulley blocks

lifting beams

equalising beams

spanners

spirit levels and automatic levels

tape measures

wrenches

heights access equipment, such as elevated work platforms (EWPs)

load slinging methods:

straight sling

adjustable sling

reeved sling

inclined sling

multi-point lift

balanced load and CoG

load equalisation and methods

advanced lifting techniques:

standard lift from a truck

load balancing when lifting from vehicles

flat lifting from supports or on ground

edge lifting and tilting by rotation about one edge

face lifting and tilting by rotation about one edge

single crane and multi-crane mid-air rotation

multi-crane lifts

workplace requirements when working with prefabricated concrete components:

cleaning up the work area

maintaining and storing plant, labelling, tools and equipment

material reuse

reporting problems

quality

workplace procedures related to communication protocols

principles of sustainability relevant to material reuse

work planning and logical task sequencing.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

Assessment must be conducted in the workplace or a simulated workplace using realistic conditions, materials, activities, responsibilities, procedures, safety requirements and environmental considerations.

Candidates must have access to erection design documentation, materials, manufacturer specifications, plant, tools and equipment required to achieve the performance evidence.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad