# Assessment Requirements for CPCPRF2022 Select and install roof sheeting and wall cladding

Release: 3

Assessment Requirements for CPCPRF2022 Select and install roof sheeting and wall cladding

# Modification History

| Release 3 | This version first released with CPC Construction, Plumbing and Services Training Package Release 6.5. Amended Performance Criteria 2.2, 4.2 and 5.2. |
| --- | --- |
| Release 2 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.1. Typographical error corrected in Element 5 and Performance Evidence reformatted for clarity. |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is not equivalent to CPCPRF2022A Select and install roof sheeting and wall cladding, and CPCPRF3021A Receive roofing materials. Updated to meet the Standards for Training Packages 2012. |

# Performance Evidence

To demonstrate competency, a candidate must meet the performance criteria of this unit by:

installing steel claddings to structures not less than 2 metres wide and 1.5 metres long including:

a flat deck roof system to a pitch ≥ 1° using concealed fixing method and installing safety wire mesh, blanket and foil

a pitched roof system ≥ 5° using an approved pierced fixing method and incorporating:

a valley, hip, ridge and barge

thermal insulation containing a blanket and foil for fire management

a non-structural cladded roof over rigid support using concealed fixing method including valley, hip, ridge, barge and roof discharge detail and moisture control pliable membrane

a sandwich panel roof or wall over supports using pierced fixing method with a minimum of two panels

a non-metallic roof sheet using pierced fixing and installing safety wire mesh

wall cladding incorporating side lapping with one external and internal corner and toe mould and including:

an opening of 300 x 600 mm

pliable building membrane.

# Knowledge Evidence

To be competent in this unit, a person must demonstrate knowledge of:

reading and interpreting plans, including electronic plans

current standards and codes for roof sheeting and insulation:

HB39 Installation Code for metal roof and wall cladding

National Construction Code (NCC)

AS 1562 Design and installation of sheet roof and wall cladding

AS 4200.2 Pliable building membranes and underlays - Installation

AS 3999 Bulk thermal insulation - Installation

calculation of material quantities

receival, storage, protection and checking quantities for delivery of roofing materials

roof components:

insulation to meet thermal acoustic, moisture and fire control

safety mesh

rigid supports

roof and safety battens

fixings and fasteners

metal and non-metallic roofing sheets

sandwich panel

thermal breaks and approved sealants

capillary action and thermal expansion

corrosion control

weathering

unloading and storage of roof materials following manufacturers’ instructions:

stacking dunnage

environmental protection

conformity of materials against the delivery docket

work health and safety (WHS) requirements:

working at heights

fragile materials

asbestos containing materials

selecting and installing roof sheeting and wall cladding

overhead hazards

installation in bushfire prone areas and requirements of The National Association of Steel-Framed Housing (NASH) Standard for construction in bushfire areas or AS 3959 Construction of buildings in bushfire-prone areas

how to access relevant information, including codes and standards

tools, materials and equipment used to select and install roof sheeting and wall cladding.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

This unit must be assessed in the workplace or a close simulation using realistic workplace conditions, materials, activities, responsibilities, procedures, safety requirements and environmental considerations.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad