# CPCPRF2022 Select and install roof sheeting and wall cladding

Release: 3

CPCPRF2022 Select and install roof sheeting and wall cladding

# Modification History

| Release 3 | This version first released with CPC Construction, Plumbing and Services Training Package Release 6.5. Amended Performance Criteria 2.2, 4.2 and 5.2. |
| --- | --- |
| Release 2 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.1. Typographical error corrected in Element 5 and Performance Evidence reformatted for clarity. |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is not equivalent to CPCPRF2022A Select and install roof sheeting and wall cladding, and CPCPRF3021A Receive roofing materials. Updated to meet the Standards for Training Packages 2012. |

# Application

This unit specifies the skills and knowledge to select and install insulation, roof battens and wall cladding involving metallic, non-metallic or other approved roof covering materials.

In some jurisdictions, this unit of competency may form part of accreditation, licensing, legislative, regulatory or certification requirements.

# Pre-requisite Unit

CPCPCM2043 Carry out WHS requirements

CPCPCM2055 Work safely on roofs

# Unit Sector

Plumbing and Fire Services

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Identify installation requirements. | 1.1 | Access, read and determine roof sheeting and wall cladding installation requirements from job specifications, relevant Australian Standards, codes, manufacturer’s instructions and jurisdictional requirements. |
|  |  | 1.2 | Identify and apply workplace policies and procedures, work health and safety (WHS) and environmental requirements. |
|  |  | 1.3 | Identify whether materials to be used are compatible with the surrounding environment and other materials within the installation. |
|  |  | 1.4 | Determine material types and calculate quantities from plans and specifications. |
|  |  | 1.5 | Identify roof insulation and wall cladding to meet thermal, acoustic, moisture and bushfire control requirements. |
|  |  | 1.6 | Identify safety mesh requirements to support the installation requirements. |
| 2 | Receive delivery of materials. | 2.1 | Provide safe access for delivery vehicles and on-site workers. |
|  |  | 2.2 | Ensure the point of delivery can support the load and assist in the unloading and transfer of materials. |
|  |  | 2.3 | Check conformity of materials against the delivery docket and specifications and report any shortfalls or damage. |
|  |  | 2.4 | Secure materials and protect from moisture and mechanical damage. |
| 3 | Prepare for work. | 3.1 | Identify and check condition of plant, equipment and tools required for the installation. |
|  |  | 3.2 | Select, fit and use appropriate personal protective equipment (PPE). |
|  |  | 3.3 | Check fall protection system is in place and secure. |
| 4 | Install roof sheeting. | 4.1 | Perform installation and fix selected insulation ensuring acoustic, thermal, moisture and bushfire control. |
|  |  | 4.2 | Measure, cut and fix roof battens for the installation at correct spacings to match plans, in accordance with jurisdictional legislation. |
|  |  | 4.3 | Measure, cut, weather and install roof sheets using appropriate fastening spacings and methods. |
|  |  | 4.4 | Remove swarf end of installation. |
| 5 | Install wall cladding. | 5.1 | Perform installation and fix selected insulation ensuring acoustic, thermal, moisture and fire control. |
|  |  | 5.2 | Measure, cut and fasten battens at correct spacings to match plans, in accordance with jurisdictional legislation. |
|  |  | 5.3 | Measure, cut and install wall cladding and flashings using appropriate fastening spacing and methods. |
|  |  | 5.4 | Install thermal breaks as required. |
| 6 | Clean up. | 6.1 | Remove swarf off cladding, flashings and gutter and clear any rivet stems and fasteners. |
|  |  | 6.2 | Clear the work area and dispose of, reuse or recycle materials in accordance with state or territory legislation and workplace requirements. |
|  |  | 6.3 | Clean tools and equipment, check for serviceability and report any damage, and store and secure. |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

Supersedes and is not equivalent to CPCPRF2022A Select and install roof sheeting and wall cladding, and CPCPRF3021A Receive roofing materials.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad

Assessment Requirements for CPCPRF2022 Select and install roof sheeting and wall cladding

# Modification History

| Release 3 | This version first released with CPC Construction, Plumbing and Services Training Package Release 6.5. Amended Performance Criteria 2.2, 4.2 and 5.2. |
| --- | --- |
| Release 2 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.1. Typographical error corrected in Element 5 and Performance Evidence reformatted for clarity. |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is not equivalent to CPCPRF2022A Select and install roof sheeting and wall cladding, and CPCPRF3021A Receive roofing materials. Updated to meet the Standards for Training Packages 2012. |

# Performance Evidence

To demonstrate competency, a candidate must meet the performance criteria of this unit by:

installing steel claddings to structures not less than 2 metres wide and 1.5 metres long including:

a flat deck roof system to a pitch ≥ 1° using concealed fixing method and installing safety wire mesh, blanket and foil

a pitched roof system ≥ 5° using an approved pierced fixing method and incorporating:

a valley, hip, ridge and barge

thermal insulation containing a blanket and foil for fire management

a non-structural cladded roof over rigid support using concealed fixing method including valley, hip, ridge, barge and roof discharge detail and moisture control pliable membrane

a sandwich panel roof or wall over supports using pierced fixing method with a minimum of two panels

a non-metallic roof sheet using pierced fixing and installing safety wire mesh

wall cladding incorporating side lapping with one external and internal corner and toe mould and including:

an opening of 300 x 600 mm

pliable building membrane.

# Knowledge Evidence

To be competent in this unit, a person must demonstrate knowledge of:

reading and interpreting plans, including electronic plans

current standards and codes for roof sheeting and insulation:

HB39 Installation Code for metal roof and wall cladding

National Construction Code (NCC)

AS 1562 Design and installation of sheet roof and wall cladding

AS 4200.2 Pliable building membranes and underlays - Installation

AS 3999 Bulk thermal insulation - Installation

calculation of material quantities

receival, storage, protection and checking quantities for delivery of roofing materials

roof components:

insulation to meet thermal acoustic, moisture and fire control

safety mesh

rigid supports

roof and safety battens

fixings and fasteners

metal and non-metallic roofing sheets

sandwich panel

thermal breaks and approved sealants

capillary action and thermal expansion

corrosion control

weathering

unloading and storage of roof materials following manufacturers’ instructions:

stacking dunnage

environmental protection

conformity of materials against the delivery docket

work health and safety (WHS) requirements:

working at heights

fragile materials

asbestos containing materials

selecting and installing roof sheeting and wall cladding

overhead hazards

installation in bushfire prone areas and requirements of The National Association of Steel-Framed Housing (NASH) Standard for construction in bushfire areas or AS 3959 Construction of buildings in bushfire-prone areas

how to access relevant information, including codes and standards

tools, materials and equipment used to select and install roof sheeting and wall cladding.

# Assessment Conditions

Assessors must meet the requirements for assessors contained in the Standards for Registered Training Organisations.

This unit must be assessed in the workplace or a close simulation using realistic workplace conditions, materials, activities, responsibilities, procedures, safety requirements and environmental considerations.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad