# CPCPRF3021 Receive roofing materials

Release: 2

CPCPRF3021 Receive roofing materials

# Modification History

| Release 2 | This version first released with CPC Construction, Plumbing and Services Training Package Release 6.2. Correction to prerequisite unit code from CPCPCM2043A Carry out WHS requirements to CPCPCM2043 Carry out WHS requirements. |
| --- | --- |
| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCPRF3021A Receive roofing materials. Updated to meet the Standards for Training Packages 2012. |

# Application

This unit specifies the skills and knowledge required to coordinate the delivery, receipt and handling of roofing materials on a site.

# Pre-requisite Unit

CPCPCM2043 Carry out WHS requirements.

# Unit Sector

Plumbing and Fire Services

# Elements and Performance Criteria

| Elements describe the essential outcomes. |  | Performance criteria describe what needs to be done to demonstrate achievement of the element. |  |
| --- | --- | --- | --- |
| 1 | Prepare for work. | 1.1 | Obtain and confirm plans and specifications by site visit. |
|  |  | 1.2 | Adhere to work health and safety (WHS) and environmental requirements and materials handling throughout the work. |
|  |  | 1.3 | Identify and adhere to quality assurance requirements according to workplace requirements. |
|  |  | 1.4 | Select and check tools and equipment, including personal protective equipment (PPE) and barricades and signs, for serviceability. |
|  |  | 1.5 | Prepare work area to support efficient receipt of roofing materials. |
| 2 | Plan delivery. | 2.1 | Interpret plans, specifications, material quantity details and other relevant information to determine amount and type of material to be delivered to site. |
|  |  | 2.2 | Inspect site to determine capacity, identify risks and hazards, and identify load-handling methods and techniques. |
|  |  | 2.3 | Obtain spot load limits on roof frame structure from structural engineer. |
|  |  | 2.4 | Sequence loads according to job requirements and work site capacity. |
|  |  | 2.5 | Place orders specifying items, quantities and sequence of delivery of each load. |
|  |  | 2.6 | Plan and sequence deliveries in conjunction with others involved in or affected by their arrival. |
| 3 | Receive delivery of materials. | 3.1 | Inform employees of delivery process. |
|  |  | 3.2 | Clear and provide access to site for crane and other support vehicles and equipment. |
|  |  | 3.3 | Prepare delivery sites and spot load sites and material securing equipment. |
|  |  | 3.4 | Position safety barricades and signs. |
|  |  | 3.5 | Move loads from delivery vehicle to spot load positions or other determined site location. |
|  |  | 3.6 | Cover and secure loads to prevent damage according to standards and manufacturer requirements, and report faults. |
|  |  | 3.7 | Apply sustainability principles and concepts when preparing for and undertaking work process. |
| 4 | Clean up. | 4.1 | Remove safety barricades and signs. |
|  |  | 4.2 | Clear work area and dispose of, re-use or recycle materials according to legislation, regulation, codes of practice and job specification. |
|  |  | 4.3 | Clean, check, maintain and store tools and equipment according to manufacturer recommendations and workplace procedures. |
|  |  | 4.4 | Complete documentation according to workplace requirements. |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Unit Mapping Information

Supersedes and is equivalent to CPCPRF3021A Receive roofing materials.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad