# Assessment Requirements for CPCPSN4011 Design and size sanitary plumbing systems

Release: 1

Assessment Requirements for CPCPSN4011 Design and size sanitary plumbing systems

# Modification History

| Release 1 | This version first released with CPC Construction, Plumbing and Services Training Package Release 5.0. Supersedes and is equivalent to CPCPSN4011B Design and size sanitary plumbing systems. Updated to meet the Standards for Training Packages 2012. |
| --- | --- |

# Performance Evidence

To demonstrate competency, a candidate must meet the performance criteria for this unit by:

designing, sizing and documenting the layout and schematic details for a commercial building with the design incorporating:

an approved sanitary plumbing system to a minimum of seven floor levels. The design must incorporate the following fixtures on each floor level:

a disabled bathroom including a water closet (WC) and basin

male toilet facilities including 3 WCs, 3 basins and 3 individual urinals or an equivalent sized urinal slab

female toilet facilities including 5 WCs and 3 basins

a shower

kitchen sink for tea-making purposes

one floor level must contain a commercial kitchen with food preparation area including at least:

one commercial dishwasher

one commercial pot wash facility

one commercial glasswasher

one bucket trap

one basin

one floor level must be below the approved point of connection (a basement) and involve an approved trade waste facility

designing, sizing and documenting the layout and schematic details for a residential building incorporating an approved sanitary plumbing system to a minimum of six floor levels, including a basement and the following fixtures on each floor level, excluding the basement:

3 baths

6 showers

6 WCs

6 basins

3 kitchen sinks

3 dishwashers

3 laundry tubs

3 clothes washing machines.

# Knowledge Evidence

To be competent in this unit, a person must demonstrate knowledge of:

relevant manufacturers’ instructions, Australian Standards and jurisdictional requirements

properties of soil and waste discharges, including temperature and corrosive discharges

approved materials and components, their characteristics and application relating to sanitary plumbing systems

acoustic treatment of sanitary pipework

principles of sanitary plumbing systems:

elevated pipework

fully vented modified

reduced velocity aerated stack systems

single stack

other approved systems

reduced and invert levels

drawing instruments and sketching techniques, including the use of conventional symbols

use of computers and computer-aided design (CAD) software for documentation

how to access relevant information, including codes and standards

tools, materials and equipment used to design, size and document the layout of sanitary drainage systems

work health and safety (WHS) requirements for designing, sizing and documenting the layout of sanitary drainage systems.

# Assessment Conditions

Assessors must satisfy the requirements for assessors listed in the Standards for Registered Training Organisations.

This unit must be assessed in the workplace or a close simulation using realistic workplace conditions, materials, activities, responsibilities, procedures, safety requirements and environmental considerations.

# Links

Companion volumes to this training package are available at the VETNet website - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=7e15fa6a-68b8-4097-b099-030a5569b1ad